3GPP TSG-SA WG6 Meeting #51-e	S6-222998

e-meeting, 10th – 19th October 2022	(revision of S6-222808)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0134 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NRM coordination for redundant PDU Session establishment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL2 |  |  |  |  |  | Date: |  |  | 2022-09-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SEALDD server or VAL server has the requirement of initiating redundant PDU Sessions for URLLC services. As specified in clause 4.15.6.10 of TS 23.502, “(DNN, S-NSSAI). This may be provided by the AF or determined by the NEF based on the AF Identifier when it is not provided by the AF and the AF provides only one instance of AF guidance for URSP determination”. SEALDD server or VAL server may have no slice specific information for the redundant PDU Sessions. NRM server is used for interacting with the control plane of 5GC for QoS negotiation and other cofigurations. Thus it is proposed to make NRM provide the service for SEALDD server or VAL server to trigger the redundant PDU Session establishment regarding to the DNN, S-NSSAI selection issues. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new service is provided by the NRM server for SEALDD server or VAL server to trigger the redundant PDU Session establishment. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SEALDD server or VAL server should be configured with DNN S-NSSAI information when using the URLLC services, which is not always available in real cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | 14.3.X(new) |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 14.3.X	AF influence URSP procedures for reliable transmission

#### 14.3.X.1	General

The procedures related to the AF influence URSP forreliable transmission are described in the following subclauses.

#### 14.3.X.2	AF influence URSP procedure for reliable transmission

Pre-conditions:

1.	The SEALDD server (or VAL server) has decided to use reliable transmission for a specific SEALDD client (or VAL client) and has got the UE address or UE ID.

Figure 14.3.X.2-1: AF influence URSP procedure

1.	The NRM server receives from the SEALDD server (or VAL server) about the request for reliable transmission service with the application descriptors for the two redundant transmission paths. The SEALDD client’s current UE address or UE ID is also provided to identify the affected UE.

2.	After receiving the request from SEALDD server (or VAL server), the NRM server allocates the available DNN and S-NSSAI information for the two application descriptors. NRM server triggers via N5/N33 with the AF guidance for URSP procedure as described in 3GPP TS 23.502 [11] clause 4.15.6.10. The AF request includes the application traffic descriptors and the allocated DNN, S-NSSAI, and also includes UE address or UE ID received in step 1.

3.	NRM server responds with the AF influence URSP configuration result.