3GPP TSG-SA WG6 Meeting #51-e meeting	S6-222656

10th – 19th October 2022, Online

Source:	vivo

Title:	Evaluation and conclusion of PEMC/PEGC/PINE registration

Spec:	3GPP TR 23.700-78

Agenda item:	9.1

Document for:	Approval

Contact:	Huazhang Lv (Huazhang.lv@vivo.com)

1. Introduction

This pCR proposes to put forward an evaluation of solution 12.

2. Reason for Change

This paper gives an evaluation PEMC/PEGC/PINE registration defined in solution 12.

3. Conclusions

N/A

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-78 v 0.5.0.

* * * First Change * * * *

### 9.2.2	Evaluation of key issue #1: PIN Management

Solution 12 addresses Key Issue 1 and describes the aspects of PINE/PEMC/PEGC registration procedure that ahead of PIN creation.

-	Solution #12 addresses the procedure that happens before PIN create.

-	For the PEMC, the PEMC sends Registration Request (GPSI) to the PIN server. After successful registration in PIN server, the PIN server allocates the PIN client ID to this PEMC and the PEMC receives the role of PEMC.

-	For the PINE and PEGC, the PINE/PEGC registers itself into PEMC with the device metadata (MAC address, vendor name, device description, PINE/PEGC Address). And the PEMC substitutes the PINE/PEGC to register on PIN server. After successful registration in PIN server, the PIN server allocates the PIN client ID to PINE/PEGC.

* * * Second Change * * * *

# 10	Conclusions

This clause provides conclusions of the study.

## 10.1	 Conclusion of PIN management of KI#1

The following principle is considered normative for PINE/PEMC/PEGC registration:

-	For the PEMC, the PEMC sends Registration Request (GPSI) to the PIN server. After successful registration in PIN server, the PIN server allocates the PIN client ID to this PEMC and the PEMC receives the role of PEMC.

-	For the PINE and PEGC, the PEMC substitutes the PINE/PEGC to register on PIN server with the device metadata from PINE/PEGC (MAC address, vendor name, device description, PINE/PEGC Address). After successful registration in PIN server, the PIN server allocates the PIN client ID to PINE/PEGC.

* * * End of Change * * * *