3GPP TSG-SA WG6 Meeting #52-bis-e	S6-230080-r4

e-meeting, 11th – 20th January 2023	Revision of S6-230080

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0358 | rev | 1- | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add Criteria to the Preconfigured Group Regroup Request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | FirstNet |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Enh4MCPTT |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are instances when having the ability to include criteria in the group regroup request are needed, particularly in emergency situations when there is a need to location personel in a specific geographical area These requirements being addressed are in TS22.28: - [R-6.6.4.2-001] The MCX Service shall provide a means for combining a multiplicity of MCX Users into a new, temporary group (i.e., to perform a "User Regroup operation") - [R-6.6.4.2-002] The MCX Service shall provide a means for combining a multiplicity of MCX Users into a new, temporary group based on a parameter or a combination of parameters (e.g., particular geographic area, Participant type, initiation of urgent type communication such as MCX Emergency Alert or MCX Emergency Group Communication). - [R-6.6.4.2-002a] The MCX Service shall provide a means for automatically combining a multiplicity of MCX Users into a temporary MCX Service Group based on certain criteria. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add two additional IE’s, service group criteria and service user criteria, to the Preconfigured regroup request for the MC service client-MC service server and the MC service server-MC service server messages. There is not a need to add this IE to the MC service server to the MC service client. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MCservers for preconfigured user/group regroup would not support having selection criteria in determining regroup participants. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.15.2.1 and 10.15.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Initial CR |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 10.15.2.1	Preconfigured regroup request (MC service client – MC service server)

Table 10.15.2.1-1 describes the information flow preconfigured regroup request from the MC service client to the MC service server.

Table 10.15.2.1-1 Preconfigured regroup request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the requester |
| MC service group ID | M | MC service group ID of the regroup group |
| MC service group ID | M | MC service group ID of the MC service group from which configuration is to be taken |
| MC service group ID list | O (see NOTE) | List of MC service groups to be regrouped into the group regroup group |
| MC service group criteria | O (see NOTE) | Carries the details of criteria which will be used by the MC service server for determining the groups. For example, it can be a location based criteria to determine the MC service group ID list, or it could be tags (e.g. “fire”, “medical”, “police”, etc.), or a combination of tags and location. |
| MC service user ID list | O (see NOTE) | List of MC service users to be regrouped into the user regroup group |
| MC service user criteria | O (see NOTE) | Carries the details of criteria which will be used by the MC service server for determining the participants. For example, it can be a location based criteria to determine the MC service user ID list in a particular area, or it could be tags (e.g. “fire”, “medical”, “police”, etc.), or a combination of tags and location. |
| NOTE: One and only one of these shall be present. |  |  |

Editor’s Note:	The specification of criteria sent to the MC server for user regroup and group regroup needs better definition, including possible definition of tags associated with a group or a user.

* * * Second Change * * * *

#### 10.15.2.2	Preconfigured regroup request (MC service server – MC service client)

Table 10.15.2.2-1 describes the information flow preconfigured regroup request from the MC service server to the MC service client.

Table 10.15.2.2-1 Preconfigured regroup request information elements

| Information Element | Status | Description |  |
| --- | --- | --- | --- |
| MC service ID | M | The MC service ID of the target MC service group member |  |
| MC service group ID | M | MC service group ID of the regroup group |  |
| MC service group ID | M | MC service group ID of the MC service group from which configuration is to be taken |  |
| MC service group ID list | O (see NOTE) | List of MC service groups to be regrouped into the group regroup group |  |
| MC service group criteria | O (see NOTE) | Carries the details of criteria which will be used by the MC service server for determining the groups. For example, it can be a location based criteria to determine the MC service group ID list, or it could be tags (e.g. “fire”, “medical”, “police”, etc.), or a combination of tags and location. |  |
| MC service user ID list | O (see NOTE) | List of MC service users to be regrouped into the user regroup group |  |
| MC service user criteria | O (see NOTE) |  | Carries the details of criteria which will be used by the MC service server for determining the participants. For example, it can be a location based criteria to determine the MC service user ID list in a particular area, or it could be tags (e.g. “fire”, “medical”, “police”, etc.), or a combination of tags and location. |
| NOTE: One and only one of these shall be present. |  |  |  |

Editor’s Note:	The specification of criteria sent to the MC server for user regroup and group regroup needs better definition, including possible definition of tags associated with a group or a user.

* * * Third Change * * * *

#### 10.15.2.3	Preconfigured regroup request (MC service server – MC service server)

Table 10.15.2.3-1 describes the information flow preconfigured regroup request from the MC service server to the MC service server.

Table 10.15.2.3-1 Preconfigured regroup request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the requester |
| MC service group ID | M | MC service group ID of the regroup group |
| MC service group ID | M | MC service group ID of the MC service group from which configuration is to be taken |
| MC service group ID list | O (see NOTE) | List of MC service groups to be regrouped into the group regroup group |
| MC service group criteria | O (see NOTE) | Carries the details of criteria which will be used by the MC service server for determining the groups.For example, it can be a location based criteria to determine the MC service group ID list in a particular area, or it could be tags (e.g. “fire”, “medical”, “police”, etc.), or a combination of tags and location. |
| MC service user ID list | O (see NOTE) | List of MC service users to be regrouped into the user regroup group |
| MC service user criteria | O (see NOTE) | Carries the details of which will be used by the MC service server for determining the participants. For example, it can be a location based criteria to determine the MC service user ID list in a particular area, or it could be tags (e.g. “fire”, “medical”, “police”, etc., or a combination of tags and location. |
| NOTE: One and only one of these shall be present. |  |  |

Editor’s Note:	The specification of criteria sent to the MC server for user regroup and group regroup needs better definition, including possible definition of tags associated with a group or a user.

* * * End of Changes * * * *