3GPP TSG-SA WG6 Meeting #55	S6-232099

Berlin, Germany 22nd – 26th May 2023	(revision of S6-231708)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 357 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Application Group Profile EN resolution proposal |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-06-16 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This contribution proposes to address one of the outstanding ENs in agreed CR 0308. Specifically the EN stating “It is FFS whether the Application Group profile should be in AC profile or not”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The association of the Application Group profile to the Application Profile is made explicit by proposing that AC Profile and Application Group profile are nested under Information Element AC Profiles. This should not break backwards compatibility, where it is noted AC Profile is already nested under an AC Profiles IE. For example, for the ECSServProvReq type (clause 8.1.5.2.2, TS 24.558) the data type for acProfs is array(ACProfile). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The solution specified for Common EAS discovery will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.X & 8.3.3.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is dependant on SA6#54-e agreed CR 0308. Key Issue#17 (TR 23.700-98) |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | In revision 1, it is proposed to not limit the service provisioning request to a single AC Profile + Application Group profile, which was the restriction in the initial proposal (in cases that the Application Group profile was included in the service provisioning request). |  |  |  |  |  |  |  |  |

* * * First Change * * * *

Proposed changes on top of those provided in accepted CR 0308.

### 8.2.X	Application Group profile

An Application Group profile includes information about the Application Group used to determine common EAS.

Table 8.2.X-1: Application Group profile

| Information element | Status | Description |
| --- | --- | --- |
| Application Group ID | M | Identity of a group of UEs using the same application. |
| EAS ID | M | Identifier of EAS |
| Expected Group Geographical Service Area. | O | The expected location(s) (e.g. route) of the group of UEs during the AC's operation schedule. This geographic information can express a geographic point, polygon, route, signalling map, or waypoint set. |

Editor's Note: It is FFS which additional IEs are required in the Application Group Profile.

Editor’s Note: It is FFS how the EES can determine common EAS requirements if different values than the ones provided by the “AC Service KPIs” of the corresponding AC Profile are required for optimal group performance

* * * Second Change * * * *

Proposed changes on top of those provided in accepted CR 0308.

##### 8.3.3.3.2	Service provisioning request

Table 8.3.3.3.2-1 describes the information elements for service provisioning request from the EEC to the ECS.

Table 8.3.3.3.2-1: Service provisioning request

| Information element | Status | Description |
| --- | --- | --- |
| EECID | M | Unique identifier of the EEC. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| AC Profiles | O | List of information about services the EEC wants to connect to. |
| > AC Profile | M | Application Profile as described in Table 8.2.2-1. |
| > Application Group profile | O | Application Group profile associated with the AC Profile, as defined in Table 8.2.X-1. |
| EEC Service Continuity Support | O | Indicates if the EEC supports service continuity or not. The IE also indicates which ACR scenarios are supported by the EEC. When requesting service provisiojing for T-EES discovery, if the EEC requires that T-EES must support "EEC excuted ACR via T-EES" scenario, then EEC includes only "EEC excuted ACR via T-EES" in this IE |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token) |
| Connectivity information | O | List of connectivity information for the UE, e.g. PLMN ID, SSID. |
| UE location | O | The location information of the UE. The UE location is described in clause 7.3.2. |
| ECSP identifiers | O | The list of desired EES provider IDs, by the EEC. |

Editor's Note: it is FFS how EEC context pull request will work if S-EES does not support "EEC executed ACR via T-EES" scenario.

Editor's Note: [SA3] Whether the EECID and the UE ID included in request of EDGE-1 & 4 interactions is part of the security credential is SA3's responsibility.

* * * End of Changes * * * *