3GPP TSG-SA WG6 Meeting #56	S6-232359

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0322 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Redundant server to server messaging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | at&t |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | R-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When introducing the Ad Hoc Group Communication (AHGC) to MCData service we added extra MCData server to MCData server messaging for ad hoc group data session request and response which are never used in any of the AHGC procedures. These server to server messagings are also not used in the regular MCData group data sessions and are redundant. We had discussed this issue in the last meeting (#55). There were misunderstandings that this MCData server to MCData server messaging is needed in a group data session request/response so that the group data session request/response messages to a targeted MCData client will be routed thru its hosting MCData server and the “participating functionality” can be applied. In fact, in our MCX architecture model, all MC service clients will connect to their assigned (hosting) MC service server to access private or group communications. All communications to and from a MC service client will always go through its assigned (hosting) MC service server (coded in its URI); there is no way for a MC service client to access MCX services that bypassing its assigned (hosting) MC service server. A MCData client is identified by the MCData ID which is defined (clause 8.1.2 23.280) as a unique URI that is “uniquely identifies an MC service user in an MC system. The MC service ID indicates the MC system where the MC service ID is defined.” In a real world deployment today, it will come with the format of user-id@mcdataserverX.mncAA.mccBB.3gppnetwork.org where the mncAA.mccBB identifies the operator, the mcvideoserver1 identifies the MCData server of that operator and the user-id identifies the unique user (client) at a specific MCData server at that operator. When a MCData server sends a message to a targeted MCData client, with the format of the MCData ID, the message will always be routed to the assigned (hosting) MCData server (providing participating functionality) before delivering to the target MCData client. No need to send a message directy to that hosting MCData server to make sure the message will be routed there. Based on the above clarifications the MCData server to MCData server messaging for AHGC data session request and response need to be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Void the Ad hoc group call request and response information tables. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will create confusion and misguide the readers and stage 3 development. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.17.2.3, 7.17.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 7.17.2.3	Void

* * * next Change * * * *

#### 7.17.2.6	Void