3GPP TSG-SA WG6 Meeting #57	S6-232910

Xiamen, China 9th – 13th October 2023	(revision of S6-23xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0434 | rev | - | Current version: | 19.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MCGWUE initial configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCGWUE |  |  |  |  |  | Date: |  |  | 2023-10-09 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently, the list of permitted MC gateway UE parameter is added to MCPTT, MCVideo and MCData user profile configuration data in 3GPP TS 23.379, 3GPP TS 23.281, and 3GPP TS 23.282, respectively. However, this parameter is required to be initially configured to enable the MC gateway client hosted by a non-3GPP device to perform connection authorisation procedure towards an MC gateway UE. Please refer to pre-condition number 2 in clause 11.5.1.2.3: - The MC gateway client hosted at the non-3GPP device has been provided with an appropriate GW MC service ID. At this point, the MC service client has not received its user profile to be able to obtain this information from. Additionally, this information is utilized by the MC gateway client and not the MC service client. Please refer to clause 10.1.1.1. Therefore, clause A.9 is added to include this parameter. Similarly, the precondition from clause 11.5.1.3.3: - The MC gateway client, which is hosted by the MC gateway UE, has been configured with the necessary parameters needed for connectivity with the MC gateway UE. Furthermore, clause A.7 is reformulated to reflect the type of service provided by each GW MC service ID. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - adding clause A.9 describing the initial MC gateway client configuration data. - updating clause A.7 to have the GW MC service ID reflects the type of service provided. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The initial configuration data of the MC gateway client is essential to perform the initial connection towards MC gateway UE, namely, the connection authorisation. If the initial configuration data is not correctly described, stage 3 work will be delayed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.7, A.9 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# A.7	Initial MC gateway UE configuration data

The initial MC gateway UE configuration data is essential to the MC gateway UE to successfully connect MC clients to the MC system. The initial MC gateway UE configuration data can be the same or different across MC gateway UEs.

Data in table A.7-1 is provided to the MC gateway UE during the bootstrap process and can be configured on the MC gateway UE offline using the CSC-11 reference point or via other means.

Table A.7-1: Initial MC gateway UE configuration data (on-network)

| Reference | Parameter description |
| --- | --- |
| Subclause 5.15 of 3GPP TS 22.280 [3] | List of permitted GW MCPTT IDs for MCPTT services |
|  | > GW MCPTT ID |
| Subclause 5.15 of 3GPP TS 22.280 [3] | List of permitted GW MCVideo IDs for MCVideo services |
|  | > GW MCVideo ID |
| Subclause 5.15 of 3GPP TS 22.280 [3] | List of permitted GW MCData IDs for MCData services |
|  | > GW MCData ID |

NOTE:	Configured MC service IDs indicate the support of an MC service.

# * * * * Second change * * * *

# A.9	Initial MC gateway client configuration data

The initial MC service gateway client configuration data is essential to the MC gateway client to be utilized during the connection authorisation procedure.

Table A.9-1: Initial MC gateway client configuration data (on-network)

| Reference | Parameter description |
| --- | --- |
| Subclause 5.15 of 3GPP TS 22.280 [3] | List of permitted GW MCPTT ID to enable connecting to MC gateway UE for MCPTT services |
|  | > GW MCPTT ID |
| Subclause 5.15 of 3GPP TS 22.280 [3] | List of permitted GW MCVideo ID to enable connecting to MC gateway UE for MCVideo services |
|  | > GW MCVideo ID |
| Subclause 5.15 of 3GPP TS 22.280 [3] | List of permitted GW MCData ID to enable connecting to MC gateway UE for MCData services |
|  | > GW MCData ID |

# * * * * End of changes * * * *