3GPP TSG-SA WG6 Meeting #57	S6-232963

Xiamen, China, 9th – 13th October 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0376 | rev |  | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on floor idle for multi-talker case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon, UIC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMONASTERY2 |  |  |  |  |  | Date: |  |  | 2023-09-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In SA6#56, the CR 0362 was agreed to add the floor idle procedure as the floor idle is referred in multiple clauses in TS 23.379 and the information floor for floor idle is well defined in 10.9.1.2.8. 10.9.1.2.8 Floor idle Table 10.9.1.2.8-1 describes the information flow floor idle, from the floor control server to the floor participant, which is used to indicate that a session is in idle status, i.e. the floor is not granted to any party. This information flows is sent in unicast or broadcast. However, this floor idle which indicates that the floor is not granted to any party, is also applicable for multi-talker control. This needs to be captured in the TS 23.379. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a NOTE in clause 10.9.1.3.5 that the floor idle is also applicable for multi-talker control case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The floor participant in multi-talker will not never receive the floor idle. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.9.1.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /************************ First change ******************************/

##### 10.9.1.3.5	Floor idle during an MCPTT session

During silence (when no talk burst is ongoing), the floor control server may send floor idle message to all floor participants from time to time.

NOTE : Floor idle is also applicable to multi-talker control.

Figure 10.9.1.3.5-1 shows the high level procedure that the floor idle is conducted for the MCPTT session already established between the floor participant and the floor control server. Only three MCPTT UEs are shown in the figure for the simplicity.

Pre-condition:

1.	The floor control is established between the floor participants and floor control server.

2.	The floor participant A is granted the floor.

Figure 10.9.1.3.5-1: Floor idle during an MCPTT session

1.	User A stops talking and wants to stop sending voice media over the session. The floor participants A sends the floor release message to the floor control server.

2.	The floor control server sends the floor idle message to the participants due to e.g., all the floor requests are processed and no new floor request is received.

3.	Floor participants may notify the MCPTT user about the floor is idle.

NOTE:	Floor idle may be sent multiple times during the MCPTT session.