3GPP TSG-SA WG6 Meeting #57	S6-233246

Xiamen, China, 09th – 13th October, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0244 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Subscription update service operation for the SS_VALServiceAreaConfiguration API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL2 |  |  |  |  |  | Date: |  |  | 2023-09-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the last SA6 meeting, the Subscribe-notify service operations for the SEAL-X reference point were added to the SS_VALServiceAreaConfiguration API. However, the consumer (SEAL server) cannot update the existing subscription due to the absence of the related service operation. Also, the alignment of the VAL Service Area configuration service description is needed with the latest updates in Release-18. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces an update subscription service operation and essential alignment of the VAL Service Area configuration service description with the latest updates. Editorial and minor corrections. Additionally, some typos and referencing problems are handled. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Impossibility to update the existing subscriptions in the SS_VALServiceAreaConfiguration API. Inconsistencies in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2.42, 9.3.2.43, 9.3.2.X (new), 9.3.2.Y (new), 9.3.13.1, 9.3.13.6, 9.3.13.8, 9.3.13.9 (new), 9.4.1, 9.4.8.9 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| () |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

* * * First Change * * * *

#### 9.3.2.42	VAL service area subscription request

Table 9.3.2.42-1 describes the information flow from a SEAL server to a location management server to subscribe VAL service area identifier update events or update subscription for the VAL service area identifier update events at the LM server.

Table 9.3.2.42-1: VAL service area subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Indicates the SEAL server identity. (NOTE) |
| List of the subscribed events | M | Indicates the list of the requested events. |
| > Event type | M | Indicates the subscribed event (e.g., update/deletion of the VAL service area ID) |
| > VAL service area IDs | O | Indicates the list of the VAL service area identifiers. This information element shall be provided when the requested event type is update or delete VAL service area ID. |
| Notification Target URI | M | Target URI where the VAL server wishes to receive the notifications about VAL service area update events. |
| Subscription duration | O | It indicates the subscription duration. |
| NOTE: This information element shall not be updated via the update VAL service area subscription request. |  |  |

* * * Next change * * * *

#### 9.3.2.43	VAL service area subscription response

Table 9.3.2.43-1 describes the information flow from a location management server to a SEAL server for VAL service area subscription response or update VAL service area subscription response.

Table 9.3.2.43-1: VAL service area subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Subscription status | M | It indicates the subscription result. |

* * * Next change * * * *

#### 9.3.2.X	VAL service area unsubscribe request

Table 9.3.2.X-1 describes the information flow from a SEAL server to a location management server for VAL service area unsubscribe request.

Table 9.3.2.X-1: VAL service area unsubscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Indicates the SEAL server identity. |

* * * * Next Change * * * *

#### 9.3.2.Y	VAL service area unsubscribe response

Table 9.3.2.Y-1 describes the information flow from the location management server to the SEAL server for VAL service area unsubscribe response.

Table 9.3.2.Y-1: Location information unsubscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Request status | M | It indicates the unsubscribe request result. |

* * * Next change * * * *

#### 9.3.13.1	General

The VAL service area refers to the geographical area(s) (e.g., coordinates, civic addresses, network areas) served by a VAL server. The VAL service area is identified by the VAL service area identifier (ID). The VAL server may have more than one VAL service area defined. These VAL service areas are configured by the VAL server to the location management server and are assigned with a unique VAL service area identifier. When the VAL service area configurations are in place the VAL server may use the VAL service area ID as part of the location information in the SEAL APIs requiring location information. These configurations can be updated by the VAL server in-order to modify the geographical area(s) associated with the VAL service area and when is updated, the SEAL internally manage the updates without requiring the VAL server to re-invoke the APIs where the updated VAL service area are being used.

* * * Next change * * * *

#### 9.3.13.6	VAL service area identifier subscribe procedure

Figure 9.3.13.6-1 illustrates the high level procedure of the subscribe VAL service area update events.

Figure 9.3.13.6-1: Subscribe VAL service area procedure

1.	The SEAL server sends the VAL service area subscription request to the location management server to subscribe for the VAL service area identifier(s) update events. In the request message, the SEAL server includes the information as specified in table 9.3.2.42-1.

2.	The location management server shall check if the SEAL server is authorized to susbcribe for the VAL service area identifier(s) update event(s).

3.	if the SEAL server is authorized to subscribe for the VAL service area identifier(s) update events, the location management server creates the subscription based on the provided parameters in the request.

4.	The location management server sends the VAL service area subscription response with the information as specified in table 9.3.2.43-1.

* * * Next change * * * *

#### 9.3.13.8	VAL service area identifier unsubscribe procedure

Figure 9.3.13.8-1 illustrates the high level procedure for a SEAL server to unsubscribe the notifications about the VAL service area identifier update events at the location management server.

Pre-conditions:

-	the SEAL server has an active VAL service area subscription at the location management server.

Figure 9.3.13.8-1: Unsubscribe VAL service area procedure

1.	When the SEAL server decides to terminate a VAL service area subscription, it sends a VAL service area unsubscribe request to the location management server. In the request message, the SEAL server includes the information as specified in table 9.3.2.X-1.

2.	If the SEAL server is authorized to terminate the subscription, the location management server terminates the VAL service area subscription.

3.	The location management server sends a VAL service area unsubscribe response to the SEAL server, the location management server includes the information as specified in table 9.3.2.Y-1.

* * * Next change * * * *

#### 9.3.13.9	VAL service area identifier subscription update procedure

Figure 9.3.13.9-1 illustrates the high level procedure of the update subscription for the VAL service area update events.

Figure 9.3.13.9-1: VAL service area identifier subscription update procedure

1.	The SEAL server sends the VAL service area subscription update request to the location management server to update the existing subscription for the VAL service area identifier(s) update events. In the request message, the SEAL server includes the information as specified in table 9.3.2.42-1.

2.	The location management server shall check if the SEAL server is authorized to update the subscription for the VAL service area identifier(s) update event(s).

3.	if the SEAL server is authorized, the location management server checks whether the target susbcription exists and then updates the subscription for the VAL service area identifier(s) update events.

4.	The location management server sends the VAL service area subscription update response with the information as specified in table 9.3.2.43-1.

* * * Next change * * * *

### 9.4.1	General

Table 9.4.1-1 illustrates the SEAL APIs for location management.

Table 9.4.1-1: List of SEAL APIs for location management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_LocationReporting | Create_Trigger_Location_Reporting Update_Trigger_Location_Reporting Cancel_Trigger_Location_Reporting | VAL server | Request /Response |
| SS_LocationInfoEvent | Subscribe_Location_Info | VAL server | Subscribe/Notify |
|  | Unsubscribe_Location_Info | VAL server |  |
|  | Notifiy_Location_Info | VAL server |  |
| SS_LocationInfoRetrieval | Obtain_Location_Info | VAL server | Request /Response |
| SS_LocationAreaInfoRetrieval | Obtain_UEs_Info | VAL server | Request/Response |
| SS_LocationMonitoring | Subscribe_Location_Monitoring | VAL server | Subscribe/Notify |
|  | Unsubscribe_Location_Monitoring | VAL server |  |
|  | Notify_Location_Monitoring_Events | VAL server |  |
|  | Subscribe_Location_Area_Monitoring | VAL server | Subscribe/Notify |
|  | Notify_Location_Area_Monitoring_Events |  |  |
|  | Update_Location_Area_Monitoring_Subscribe |  |  |
|  | Unsubscribe_Location_Area_Monitoring |  |  |
| SS_VALServiceAreaConfiguration | Configure_VAL_Service_Area | VAL server | Request/Response |
|  | Obtain_VAL_Service_Area | VAL server |  |
|  | Update_VAL_Service_Area | VAL server |  |
|  | Delete_VAL_Service_Area | VAL server |  |
|  | Subscribe_VAL_Service_Area_Change_Event | SEAL server | Subscribe/Notify |
|  | Notify_VAL_Service_Area_Change_Event | SEAL server |  |
|  | Unsubscribe_VAL_Service_Area_Change_Event | SEAL server |  |
|  | Update_Subscription_VAL_Service_Area_Change_Event | SEAL server |  |

* * * Next change * * * *

#### 9.4.8.9	Update_Subscription_VAL_Service_Area_Change_Event

API operation name: Update_Subscription_VAL_Service_Area_Change_Event

Description: Subscription update for the VAL service area identifiers change events.

Known Consumers: SEAL server.

Inputs: See subclause 9.3.2.42.

Outputs: See subclause 9.3.2.43.

See subclause 9.3.13.9 for the details of usage of this API operation.

* * * End of changes * * * *