3GPP TSG-SA WG6 Meeting #58	S6-233581

Chicago, IL, USA, 13th – 17th November 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0401 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Change list of participants when criteria was used to setup the ad hoc group call (MCPTT) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2023-11-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Ad hoc can be formed by a list of participants or by criteria provided by an authorized user. The list of participants created by the criteria may not be complete or some necessary users may not be in the group. An authorized user should be able to override the list of participants if needed by providing a distinct list of participants also when criteria was used to setup the ad hoc group. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specifies that the critieria is not longer in use when an authorised user overrides the criteria by modifying the list of participants using a list of MCPTT IDs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing functionality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.19.3.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

##### 10.19.3.1.4	Modification of ad hoc group call participants by the initiator

Figure 10.19.3.1.4-1 below illustrates the modification of ad hoc group call participants procedure by the initiator of the ad hoc group call.

Pre-conditions:

1.	The MCPTT client 1 is the initiator of the ad hoc group call.

2.	The MCPTT users on MCPTT client 1, MCPTT client 3 to MCPTT client n are on an ongoing ad hoc group call.

3.	The MCPTT user 1 requests to remove the user of MCPTT client 3 from the ad hoc group call and add user of MCPTT client 2 into the on-going ad hoc group call.

Figure 10.19.3.1.4-1: Modification of ad hoc group call participants by the initiator

1.	The MCPTT client 1 sends the modify ad hoc group call participants request to the MCPTT server in order to remove MCPTT client 3 from the ongoing ad hoc group call and add MCPTT client 2 into it.

2.	The MCPTT server verifies whether the MCPTT client 1 is authorized to add or remove (modify) the participants of the on-going ad hoc group call.

3.	The MCPTT server sends modify ad hoc group call participants response to the MCPTT client 1. If the ad hoc group was formed by using criteria, this request overrides the criteria and the MCPTT server stops checking whether MCPTT clients meet of no longer meet the criteria.

4.	The MCPTT server sends the ad hoc group call leave request to the MCPTT client 3 in order to remove it from the on-going ad hoc group call.

5.	The MCPTT client 3 notifies the user of the ad hoc group call leave request.

6.	The MCPTT client 3 sends the ad hoc group call leave response to the MCPTT server.

7.	The MCPTT server removes the MCPTT client 3 from the ongoing ad hoc group call.

8.	The MCPTT server sends the ad hoc group call request towards MCPTT client 2.

NOTE:	Steps 8 to 10 can occur at any time following step 3.

9.	The receiving MCPTT client 2 notifies the user about the incoming ad hoc group call.

10.	The MCPTT client 2 accepts the ad hoc group call request and send ad hoc group call responses to the MCPTT server. The response may also contain a functional alias of the responding MCPTT user, which is verified (valid and activated for the user) by the MCPTT server.

11.	The MCPTT server may notify the initiating MCPTT user of all the users who are added to the on-going ad hoc group call. This notification may be sent to the initiating MCPTT user by the MCPTT server more than once during the call when MCPTT users join or leave the ad hoc group call.

12.	The MCPTT server may notify the participants about the change in the participants list of on-going ad hoc group call.

# * * * * End of changes * * * *