3GPP TSG-SA WG6 Meeting #58	S6-233659

Chicago, USA, 13th - 17th November 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.283 | CR | 0068 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Functional alias support for group call interworking with GSM-R |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In current 23.283, the functional alias is only supported in the 10.3.2.2 IWF group call request, 10.3.2.4 IWF Group-broadcast group call setup request, 10.3.2.8 IWF group join request. For the other call messages, such functional alias is also needed to align with corresponding functional alias usage in MCPTT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the functional alias to the IWF call messages. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The functional alias is partially supported for the call interworking with GSM-R. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.2.3, 10.3.2.5, 10.3.2.6, 10.3.2.7, 10.3.2.9, 10.3.2.10, 10.3.2.11, 10.3.2.12, 10.3.2.13, 10.3.2.14, 10.3.2.15, 10.3.2.16, 10.3.2.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /******************** First Changes ********************/

10.3.2.3	IWF group call response (IWF – MCPTT server)

Table 10.3.2.3-1 describes the information flow IWF group call response from the MCPTT server to the IWF and from the IWF to MCPTT server.

Table 10.3.2.3-1: IWF group call response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the target MCPTT group member |
| Functional alias | O | The functional alias of the MCPTT group member |
| MCPTT group ID | M | The MCPTT group ID of the group on which the call is requested |
| SDP answer | M | Media parameters selected |

# /******************** Next Changes ********************/

10.3.2.5	IWF Group-broadcast group call setup response

Table 10.3.2.5-1 describes the information flow IWF group-broadcast group call setup response from the IWF to the MCPTT server and from the MCPTT server to the IWF.

Table 10.3.2.5-1: IWF Group-broadcast group call setup response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the MCPTT group member |
| MCPTT group ID | M | The MCPTT group ID of the group on which the call is requested |
| SDP answer | M | Media parameters selected |

10.3.2.6	IWF Group-broadcast group call release request

Table 10.3.2.6-1 describes the information flow IWF group-broadcast group call release request from the MCPTT server to the IWF and from the IWF to the MCPTT server.

Table 10.3.2.6-1: IWF Group-broadcast group call release request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M (see NOTE) | The MCPTT ID of the MCPTT group member |
| Functional alias | O | The functional alias of the calling party |
| MCPTT group ID | M | The MCPTT group ID of the group on which the call is released |
| NOTE: If the LMR system does not provide the calling party identity when the group-broadcast group call release request is originated from the LMR system, then this information element may be set to a MCPTT ID reserved for the LMR user at the IWF. |  |  |

10.3.2.7	IWF group-broadcast group call release response

Table 10.3.2.7-1 describes the information flow IWF group-broadcast group call release request from the IWF to the MCPTT server and from the MCPTT server to the IWF.

Table 10.3.2.7-1: IWF Group-broadcast group call release response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the target MCPTT group member |
| Functional alias | O | The functional alias of the MCPTT group member |
| MCPTT group ID | M | The MCPTT group ID of the group on which the call is released |

# /******************** Next Changes ********************/

10.3.2.9	IWF group join response

Table 10.3.2.9-1 describes the information flow group join response from the MCPTT server to the IWF and from the IWF to the MCPTT server.

Table 10.3.2.9-1: IWF group join response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the originator of the request. (see NOTE) |
| Functional alias | O | The functional alias of the MCPTT group member |
| MCPTT group ID | M | The MCPTT group ID of the group to which the group communication is requested |
| SDP answer | M | Media parameters selected |
| NOTE: The IWF is configured with an MCPTT ID for use when the IWF is affiliating itself to the group on behalf of the LMR system. |  |  |

10.3.2.10	IWF group call leave request

Table 10.3.2.10-1 describes the information flow IWF group call leave request from the MCPTT server to the IWF and from the IWF to the MCPTT server.

Table 10.3.2.10-1: IWF group call leave request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the MCPTT group member |
| Functional alias | O | The functional alias of the calling party |
| MCPTT group ID | M | The MCPTT group ID of the group from which the user is leaving |

10.3.2.11	IWF group call leave response

Table 10.3.2.11-1 describes the information flow IWF group call leave response from the IWF to the MCPTT server and from the MCPTT server to the IWF.

Table 10.3.2.11-1: IWF group call leave response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the MCPTT group member |
| Functional alias | O | The functional alias of the MCPTT group member |
| MCPTT group ID | M | The MCPTT group ID of the group from which the user is leaving |

10.3.2.12	IWF group call release request

Table 10.3.2.12-1 describes the information flow IWF group call release request from the IWF to the MCPTT server and from the MCPTT server to the IWF.

Table 10.3.2.12-1: IWF group call release request information elements

| Information element | Status | Description |
| --- | --- | --- |
| MCPTT ID (see NOTE) | O | The MCPTT ID of the initiating MCPTT group member |
| Functional alias | O | The functional alias of the calling party |
| MCPTT group ID | M | The MCPTT group ID of the group on which the call is released |
| Release reason | O | The reason why the call is released |
| NOTE: This IE is not included if the group call release is initiated by the server (e.g. due to timeout) |  |  |

10.3.2.13	IWF group call release response

Table 10.3.2.13-1 describes the information flow IWF group call release response from the IWF to the MCPTT server and from the MCPTT server to the IWF.

Table 10.3.2.13-1: IWF group call release response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the target MCPTT group member |
| Functional alias | O | The functional alias of the MCPTT group member |
| MCPTT group ID | M | The MCPTT group ID of the group on which the call is released. |

10.3.2.14	IWF pre-configured regroup request

Table 10.3.2.14-1 describes the information flow IWF pre-configured regroup request from the IWF to the MCPTT server and from the MCPTT server to the IWF.

Table 10.3.2.14-1 IWF pre-configured regroup request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the requester |
| Functional alias | O | The functional alias of the calling party |
| MCPTT group ID | M | MCPTT group ID of the regroup group |
| MCPTT group ID | M | MCPTT group ID of the MCPTT group from which configuration is to be taken |
| MCPTT group ID list | O (see NOTE) | List of MCPTT groups to be regrouped into the pre-configured regroup group |
| MCPTT ID list | O (see NOTE) | List of MCPTT IDs to be regrouped into the pre-configured user regroup group |
| NOTE: One and only one of these shall be present. |  |  |

10.3.2.15	IWF pre-configured regroup response

Table 10.3.2.15-1 describes the information flow IWF pre-configured regroup response from IWF to the MCPTT server and from the MCPTT server to the IWF.

Table 10.3.2.15-1 IWF pre-configured regroup response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the requester of the regrouping operation |
| Functional alias | O | The functional alias of the MCPTT group member |
| MCPTT group ID | M | MCPTT group ID of the regroup group |
| Result | M | Result of the regrouping operation |

10.3.2.16	IWF pre-configured regroup cancel request

Table 10.3.2.16-1 describes the information flow pre-configured regroup cancel request from the IWF to the MCPTT server and from the MCPTT server to the IWF.

Table 10.3.2.16-1 IWF pre-configured regroup cancel request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the requester |
| Functional alias | O | The functional alias of the calling party |
| MCPTT group ID | M | MCPTT group ID of the regroup group |

10.3.2.17	IWF pre-configured regroup cancel response

Table 10.3.2.17-1 describes the information flow IWF pre-configured regroup cancel response from the IWF to the MCPTT server and from the MCPTT server to the IWF.

Table 10.3.2.17-1 IWF pre-configured regroup cancel response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the requester of the regroup removal |
| Functional alias | O | The functional alias of the MCPTT group member |
| MCPTT group ID | M | MCPTT group ID of the regroup group |
| Result | M | Result of the regroup removal operation |