3GPP TSG-SA WG6 Meeting #58	S6-233787

Chicago, USA, 13th – 17th November 2023	(revision of S6-233536)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0396 | rev | 1 | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to modification of ad hoc group call participants procedure - mcptt |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MC_AHGC |  |  |  |  |  | Date: |  |  | 2023-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Any authorized user can modify ad hoc group call participants. However, the existing procedure allows only the initiator of the call to modify the ad hoc group call participants. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modify ad hoc group call participants request and Modify ad hoc group call participants response – Information flow is updated to indicate that the MCPTT ID included is of the user requesting for modification of ad hoc group call participants. - The modification of the ad hoc group call participants procedure title is updated to indicate that it is applicable for authorized users and the pre-conditions are updated to indicate that the ad hoc group call is already established and initiated by one of the ad hoc group participants with additional changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Only the initiator of the call will be able to modify ad hoc group call participants and no way for an authorized user (e.g. Dispatcher) to modify. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.19.2.11, 10.19.2.12, 10.19.3.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: 1) In "Modify ad hoc group call participants request" and "Modify ad hoc group call participants response" information flows: Proposed text is reworded to emphasize that requesting user is authorized user. 2) In 10.19.3.1.4: - In pre-condition 1: ‘is’ word added in "participants list is provided". - In pre-condition 2: ‘s’ removed from the word "belongs" - In figure: Step 9: Reworded to "Notify user" and Removed Step 7 - In Step 1 & 9: added additional text to align with the CR 0399 - Removed Step 7 3) Removed "list" word from "participants list" wherever applicable. 4) In cover page: Corrected the WID code to MC_AHGC |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 10.19.2.11	Modify ad hoc group call participants request (MCPTT client – MCPTT server)

Table 10.19.2.11-1 describes the information flow Modify ad hoc group call participants request from the MCPTT client to the MCPTT server.

Table 10.19.2.11-1: Modify ad hoc group call participants request

| Information element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the requesting MCPTT user who is authorized to modify the ad hoc group call participants |
| Functional alias | O | The functional alias of the requesting MCPTT user who is authorized to modify the ad hoc group call participants |
| MCPTT ad hoc group ID | M | The MCPTT group ID of ad hoc group call whose participants needs to be modified |
| MCPTT ID list (see NOTE) | O | List of additional MCPTT users to be added to the on-going ad hoc group call |
| MCPTT ID list (see NOTE) | O | List of MCPTT users to be removed from the on-going ad hoc group call |
| NOTE: Either one or both of these information elements is present |  |  |

#### 10.19.2.12	Modify ad hoc group call participants response (MCPTT server – MCPTT client)

Table 10.19.2.12-1 describes the information flow Modify ad hoc group call participants response from the MCPTT server to the MCPTT client.

Table 10.19.2.12-1 Modify Ad hoc group call participants response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the requesting MCPTT user who is authorized to modify the ad hoc group call participants |
| Functional alias | O | The functional alias of the requesting MCPTT user who is authorized to modify the ad hoc group call participants |
| MCPTT ad hoc group ID | M | The MCPTT group ID of ad hoc group call whose participants needs to be modified |
| Result | M | Result of the modify ad hoc group call participants request (success of failure) |
| MCPTT ID list | O | List of MCPTT users who are not allowed to be added to the on-going ad hoc group call. This list is provided if the operation is partially success. |

* * * Second Change * * * *

##### 10.19.3.1.4	Modification of ad hoc group call participants by an authorized user

Figure 10.19.3.1.4-1 below illustrates the modification of ad hoc group call participants procedure by an authorized user.

Pre-conditions:

1.	An ad hoc group call is already in progress and the participants list is provided by the originating MCPTT user while initiating the MCPTT ad hoc group call.

2.	The participants of the MCPTT ad hoc group call belong to the single MCPTT system.

3.	The MCPTT users on MCPTT client 1, MCPTT client 3 to MCPTT client n are on an ongoing ad hoc group call.

4.	The MCPTT user at MCPTT client 1 determines to remove the user of MCPTT client 3 from the ad hoc group call and add user of MCPTT client 2 into the on-going ad hoc group call.

Figure 10.19.3.1.4-1: Modification of ad hoc group call participants by an authorized user

1.	The MCPTT user at the MCPTT client 1 is authorized and requests to modify ad hoc group call participants. The MCPTT client 1 sends the modify ad hoc group call participants request to the MCPTT server in order to remove MCPTT client 3 from the ongoing ad hoc group call and add MCPTT client 2 into it.

2.	The MCPTT server verifies whether the MCPTT client 1 is authorized to add or remove (modify) the participants of the on-going ad hoc group call.

3.	The MCPTT server sends modify ad hoc group call participants response to the MCPTT client 1.

4.	The MCPTT server sends the ad hoc group call leave request to the MCPTT client 3 in order to remove it from the on-going ad hoc group call.

5.	The MCPTT client 3 notifies the user of the ad hoc group call leave request.

6.	The MCPTT client 3 sends the ad hoc group call leave response to the MCPTT server.

7.	The MCPTT server sends the ad hoc group call request towards MCPTT client 2.

NOTE:	Steps 7 to 9 can occur at any time following step 3.

8.	The receiving MCPTT client 2 notifies the user about the incoming ad hoc group call.

9.	The MCPTT client 2 accepts the ad hoc group call request and send ad hoc group call responses to the MCPTT server. The response may also contain a functional alias of the responding MCPTT user, which is verified (valid and activated for the user) by the MCPTT server. The MCPTT server considers the MCPTT user as implicitly affiliated to the ad hoc group.

10.	The MCPTT server may notify the initiating MCPTT user of all the users who are added to the on-going ad hoc group call. This notification may be sent to the initiating MCPTT user by the MCPTT server more than once during the call when MCPTT users join or leave the ad hoc group call.

11.	The MCPTT server may notify the participants about the change in the participants of on-going ad hoc group call.

* * * * End of changes * * * *