3GPP TSG-SA WG6 Meeting #58	S6-240141

Athens, Greece 26th February – 1st March 2024	(revision of S6-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0282 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MBS service area handling at NRM server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, eSEAL2 |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In SA2#160, the CR addressing the MBS service area update issue was agreed in S2-2311901(R17 CR) and S2-2311276 (R18 mirror CR) In the CR, SA2 concluded that: - Case #1- TMGI allocation: NEF reject the request if cannot satisfy the service area request. - Case #2 - MBS Session Creation without TMGI: NEF reject the request if cannot satisfy the service area request. - Case #3 - MBS Session Creation with TMGI: MB-SMF partial accept the request, and provide non-supported MBS service area and indication for this condition; - Case #4 - MBS Session Update: MB-SMF partial accept the request, and provide non-supported MBS service area and indication for this condition. Further in CT4 and CT3, the following respone were defined accordingly: - During TMGI allocation or MBS session creation: "MBS_SERVICE_AREA_NOT_SUPPORTED", HTTP "403 Forbidden" – MBS service area is not supported by any MB-SMF. "MBS_SERVICE_AREA_TOO_LARGE"+ ReducedMbsServArea, HTTP "403 Forbidden"- cannot be covered by the MB-SMF Service Area of a single MB-SMF. "ReducedMbsServArea", HTTP "201 Created" - partially accept. - Duing the MBS session update: HTTP "200 OK" with reducedMbsServArea - partially accept. In SA6, how the NRM server (acting as the AF) handles the above response from the network needs to be defined in order to efficiently make use of the MBS feature: - HTTP "403 Forbidden"+"MBS_SERVICE_AREA_NOT_SUPPORTED", the NRM server shall directly use the unicast delivery. - HTTP "403 Forbidden"+"MBS_SERVICE_AREA_TOO_LARGE"+ ReducedMbsServArea, the NRM server can retry with the ReducedMbsServArea or use the unicast delivery. - HTTP "200 OK" with reducedMbsServArea, the NRM server can retry with the reducedMbsServArea or use the unicast delivery. For the service area which is not supported by the network (i.e., not covered by the reducedMbsServArea) the NRM server can retry with a new MBS session or use the unicast delivery. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the clause to describe how the NRM server behave when the MBS service area update with requested MBS service area is failed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NRM server can not handle the MBS service area update failure correctly, and the MBS may not be used as much as impossible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, (new)14.3.4A.3.x, 14.3.2.42, 14.3.2.43 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /******************** First Changes ********************/

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains".

[3]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[4]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[5]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[6]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[7]	3GPP TS 23.286: "Application layer support for V2X services; Functional architecture and information flows".

[8]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[9]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[10]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[11]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[12]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".

[13]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[14]	3GPP TS 23.002: "Network Architecture".

[15]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[16]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

[17]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[18]	3GPP TS 23.203: "Policy and charging control architecture".

[19]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[20]	3GPP TS 26.348: "Northbound Application Programming Interface (API) for Multimedia Broadcast/Multicast Service (MBMS) at the xMB reference point".

[21]	3GPP TS 29.214: "Policy and charging control over Rx reference point".

[22]	3GPP TS 29.468: "Group Communication System Enablers for LTE (GCSE_LTE); MB2 Reference Point; Stage 3".

[23]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[24]	IETF RFC 6733 (October 2012): "Diameter Base Protocol".

[25]	ETSI TS 102 894-2 (V1.2.1): "Intelligent Transport Systems (ITS); Users and applications requirements; Part 2: Applications and facilities layer common data dictionaryMultimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[26]	ETSI TS 102 965 (V1.4.1): "Intelligent Transport Systems (ITS); Application Object Identifier (ITS-AID); Registration".

[27]	ISO TS 17419: "Intelligent Transport Systems - Cooperative systems - Classification and management of ITS applications in a global context".

[28]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[29]	3GPP TS 33.434: "Service Enabler Architecture Layer (SEAL); Security aspects for Verticals".

[30]	3GPP TS 29.549: "Service Enabler Architecture Layer for Verticals (SEAL); Application Programming Interface (API) specification; Stage3".

[31]	3GPP TS 23.285: "Architecture enhancements for V2X services".

[32]	IETF RFC 7252: "The Constrained Application Protocol (CoAP)".

[33]	IETF RFC 8323: "CoAP (Constrained Application Protocol) over TCP, TLS, and WebSockets".

[34]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[35]	IEEE Std 802.1Qcc-2018: "Standard for Local and metropolitan area networks - Bridges and Bridged Networks - Amendment: Stream Reservation Protocol (SRP) Enhancements and Performance Improvements".

[36]	IEEE 802.1Q-2018: "IEEE Standard for Local and Metropolitan Area Networks—Bridges and Bridged Networks".

[37]	IEEE Std 802.1CB-2017: "Frame Replication and Elimination for Reliability".

[38]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[39]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[40]		3GPP TS 23.435: "Procedures for Network Slice Capability Exposure for Application Layer Enablement Service".

[41]		3GPP TS 28.531: "Management and orchestration; Provisioning".

[42]		3GPP TS 28.533: "Management and orchestration; Architecture framework".

[43]		3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[44]		3GPP TS 28.532: "Management and orchestration; Generic management services".

[45]		3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[46]		3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[47]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[48]	3GPP TS 23.433: "Service Enabler Architecture Layer for Verticals (SEAL); Data Delivery enabler for vertical applications".

[49]	3GPP TS 23.436: "Procedures for Application Data Analytics Enablement Service".

[50]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2"

[51]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3"

[52]	3GPP TS 23.256 "Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2".

[53]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[54]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[55]	3GPP TS 23.545 "Application layer support for Factories of the Future (FF)".

[29.532]	3GPP TS 29.532: "5G Multicast-Broadcast Session Management Services; Stage 3".

[29.522]	3GPP TS 26.502: "Network Exposure Function Northbound APIs; Stage 3".

# /******************** Next Changes ********************/

14.3.4A.3.x	MBS service area handling

The MBS service area requested by the NRM server in the TMGI allocation or MBS session procedure towards the 5G core network may be rejected or partially accepted, the NRM server may take different actions as follows:

-	If the MBS service area requested in the TMGI allocation or MBS Session Creation without TMGI procedure as specified in 3GPP TS 23.247 [15] is rejected with the error "MBS_SERVICE_AREA_NOT_SUPPORTED" as specifined in 3GPP TS 29.532 [29.532] and 3GPP TS 29.522 [29.522], the NRM server uses the unicast delivery.

-	If the MBS service area requested in the TMGI allocation or MBS Session Creation with TMGI procedure was rejected with the error "MBS_SERVICE_AREA_TOO_LARGE" and the ReducedMbsServArea indicating the MBS Service Area that can be supported by the network as specifined in 3GPP TS 29.532 [29.532] and 3GPP TS 29.522 [29.522], the NRM server may retry with the returned ReducedMbsServArea. Alternatively, the NRM server may also determine to use the unicast delivery for the group communication in that area.

-	If the MBS service area requested in MBS Session Creation with TMGI or MBS Session Update procedure as specified 3GPP TS 23.247 [15] is partially accepted with the ReducedMbsServArea as specifined in 3GPP TS 29.532 [29.532] and 3GPP TS 29.522 [29.522], the NRM server may try another new MBS session with the MBS service area which is not covered by the ReducedMbsServArea, or alternatively use the unicast delivery for that area.

The figure 14.3.4A.3.x-1 provides a high level procedure of the NRM server determines to create an extra MBS Session with the MBS service area not covered by the ReducedMbsServArea.

NRM client

3GPP

NRM server

VAL server

VAL client

UE

Created MBS session with initial MBS service area

1a. MBS session update request with requested service area

1b. New service area based on UE location report

2. MBS session update with the requested service area is partially accepted

4. MBS session creation with rest service area which is not accepted, and obtain new MBS session ID and ingress address

3. Based on the return information, try to create a new MBS session

5. Announce the new MBS session, UE join the MBS session if multicast type, UE report listening status.

6. MBS session update response

7. User plane delivery mode

Figure 14.3.4A.3.x-1: MBS service area update failure handling

1.	The NRM server determines to update service area of the existing MBS session. This may be triggered by the MBS session update request with a new service area from the VAL server (step 1a), or the NRM server monitors the UE’s mobility and generates the new service area (step 1b) based on e.g., SEAL LMS mechanism.

2. The NRM servers performs the MBS session update with updated service area towards the 5GC as described in 3GPP TS 23.247 [39]. The MBS session update is partically accepted and ReducedMbsServArea indicating the reduced MBS Service Area in which the MBS session has been updated due to the requested MBS Service Area cannot be covered by the MB-SMF Service Area of a single MB-SMF, and the MB-SMF reduced the MBS Service Area accordingly as specifined in 3GPP TS 29.532 [29.532] and 3GPP TS 29.522 [29.522].

3.	Upon receiving the partially accept response, the NRM determines to create a new MBS session with the rest MBS Service Area that is not covered by the ReducedMbsServArea.

4.	The NRM server performs an MBS session creation procedure with the rest MBS service area.

If MBS session creation is successful, the new MBS session ID and new ingress address are returned. The procedure proceeds with step 5 to step 7.

If the MBS session creation fails, the procedure proceeds with step 7.

5.	The NRM server obtains the new MBS session ID and new ingress address of the new MBS session. The NRM server announces the new session information to the corresponding UEs. The UE may join the MBS session if it is multicast type. The UE may report the MBS listening status to the NRM server.

6.	 The NRM server returns the MBS session update response to the VAL server. The new MBS session ID are included. The ingress address of the new MBS session may be also included if it is returned in step 4.

7.	The NRM server also indicates the VAL server to deliver the data over the new MBS session if created in step 4. If the new MBS session creation is failed in step 4, the NRM server indicates the VAL server to use unicast delivery for the rest MBS service area.

# /******************** Next Changes ********************/

14.3.2.42	Multicast/broadcast resource update request

Table 14.3.2.42-1 describes the information flow for the multicast/broadcast resource update request from VAL server to NRM server.

Table 14.3.2.42-1: Multicast/broadcast resource update request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| Identity of the multicast/broadcast resource | M | Identity of the multicast/broadcast resource to be updated |
| Service requirement description | O (NOTE 3) | Indicates the service characteristics, e.g., Media type, Media format, bandwidth requirements, flow description, Application Identifier, Priority indicator, emergency indicator Application service provider. |
| Multicast/Broadcast area information | O(NOTE 3) | Indicate the area where the multicast/broadcast resource is requested for |
| Local MBMS information (NOTE 1, NOTE 2) | O(NOTE 3) |  |
| > MB2-U information | O | IP address, UDP port number of the MB2-U interface |
| > xMB-U information | O | IP address, UDP port number of the xMB-U interface |
| > M1 interface information | O | M1 interface information for local MBMS |
| Local MBMS activation indication (NOTE 1, NOTE 2) | O | Indicates whether to request the NRM server to use Local MBMS information. |
| NOTE 1: The VAL server may provide either the Local MBMS information or the Local MBMS activation indication. This IE is present when the local MBMS is required for VAL services like V2X service. NOTE 2: This information element is only applicable when EPS is specified in the network system indicator IE. NOTE 3: At least one of those IEs shall be present. |  |  |

14.3.2.43	Multicast/broadcast resource update response

Table 14.3.2.43-1 describes the information flow for the Multicast/broadcast resource update response from NRM server to VAL server.

Table 14.3.2.43-1: Multicast/broadcast resource update response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result indicates success or failure of the multicast/broadcast resource update request operation. |
| Identity of the multicast/broadcast resource | M (NOTE 1) | The identity of the multicast/broadcast MBS session, or/and the identity of the MBMS bearer to be updated. |
| New Identity of the multicast/broadcast resource | O (NOTE 3) | The identity of the new multicast/broadcast MBS session created during update. |
| Session description information | O (NOTE 2) | Indicates Multicast/broadcast related configuration information as defined in 3GPP TS 26.346 [28] (e.g. radio frequency and MBMS Service Area Identities) or/and 3GPP TS 23.247 [39] (e.g. MBS FSA ID(s), Area Session ID, service requirements) |
| NOTE 1: This IE may not be required if the session announcement mode indicates that the request is sent by the NRM server. NOTE 2: If the Result Information element indicates failure then the values of the other information elements shall not present. NOTE 3: This IE is present when the NRM server creates additional MBS session during the update |  |  |