3GPP TSG-SA WG6 Meeting #59	S6-240307

Athens, Greece 26th February – 1st March 2024	(revision of S6-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0041 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add release operation for URLLC connection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD |  |  |  |  |  | Date: |  |  | 2024-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For SEALDD client-side initiated connection, single connection has release procedure in cl.9.2.2.5, but URLLC connection doesn’t have any procedure to support connection release. In addition, the API definition needs to follow naming conversion used in other SEALDD APIs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add release operation; Update the API definition for Sdd_URLLCTransmissionConnection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The API definition is not complete for URLLC connection in SEALDD layer, which may create confusion in stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2.x (new), 9.3.3.x (new), 9.3.3.y (new), 9.3.4.1, 9.3.4.3, 9.3.4.4, 9.3.4.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 9.3.2.x	SEALDD client initiated connection release

Figure 9.3.2.x-1 illustrates the procedure for SEALDD client initiated connection release procedure from the SEALDD client to the SEALDD server.

Figure 9.3.2.x-1: SEALDD client initiated connection release

1.	The SEALDD client sends the SEALDD URLLC connection release request to the SEALDD server to release the established connection.

2.	The SEALDD server releases the SEALDD-UU redundant data transmission connection (which was established by SEALDD client or SEALDD server) and sends the response in the SEALDD URLLC connection release response message. Upon receiving the acknowledgement, the SEALDD client can release the connection resources.

* * * Next Change * * * *

#### 9.3.3.x	SEALDD URLLC transmission connection release request

See clause 9.2.3.5 for the details of information flow.

#### 9.3.3.y	SEALDD URLLC transmission connection release response

See clause 9.2.3.6 for the details of information flow.

* * * Next Change * * * *

#### 9.3.4.1	General

Table 9.3.4.1-1 illustrates the APIs exposed by SEALDD server for URLLC transmission.

Table 9.3.4.1-1: List of SEALDD server APIs for redundant transmission

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Sdd_URLLCTransmission | Request | Request/Response | VAL server |
| Sdd_URLLCTransmissionConnnection | Establish | Request/Response | SEALDD client |
|  | Update | Request/Response | SEALDD client |
|  | Release | Request/Response | SEALDD client |

* * * Next Change * * * *

#### 9.3.4.3	Sdd_URLLCTransmissionConnection_Establish operation

API operation name: Sdd_URLLCTransmissionConnection_Establish

Description: The consumer requests for URLLC transmission connection establishment.

Inputs: See clause 9.3.3.3.

Outputs: See clause 9.3.3.4.

See clause 9.3.2.1 for details of usage of this operation.

#### 9.3.4.4	Sdd_URLLCTransmissionConnection_Update operation

API operation name: Sdd_URLLCTransmissionConnection_Update

Description: The consumer requests for URLLC transmission connection update.

Inputs: See clause 9.3.3.5.

Outputs: See clause 9.3.3.6.

See clause 9.3.2.1 for details of usage of this operation.

* * * Next Change * * * *

#### 9.3.4.x	Sdd_URLLCTransmissionConnection_Release operation

API operation name: Sdd_URLLCTransmissionConnection_Release

Description: The consumer requests for URLLC transmission connection release.

Inputs: See clause 9.3.3.x.

Outputs: See clause 9.3.3.y.

See clause 9.3.2.x for details of usage of this operation.

* * * End of Change * * * *