3GPP TSG-SA WG6 Meeting #59	S6-240563

Athens, Greece 26th February – 1st March 2024	(revision of S6-240525, 240271)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0032 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Seamless SEALDD relocation enhancement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2024-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Rel-18, the SEALDD relocation has been introduced in clause 9.6, the SEALDD server can request the 5GC to perform IP rplacement to impove the service continuity. It mainly focuses on the connectivity-granularity transmission. To further imporve the seamless relcoation, the new SEALDD server and the SEALDD client should be aware of the packet sequence number has been recived, and continue to transmit with these information to provide the packet-granularity service continuity. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper proposes the seamless SEALDD relcoation enhancement procedure by interacting with the packet sequence number has been recived, to provide the packet-granularity service continuity. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SEALDD relcoation in clause 9.6 of current specification cannot provide the packet-granularity service continuity. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.6.2.x (new), 9.6.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.6.2.x	SEALDD seamless relocation based on data breakpoint information

Figure 9.6.2.x-1 describes the SEALDD seamless relocation procedure, the data traffic between the SEALDD client and the new SEALDD server is transmitted based on the uplink/downlink data breakpoint information.

Pre-conditions:

- The old SEALDD server discovers and selects the new SEALDD server by using CAPIF or EDGEAPP mechanism, triggered by UE mobility or SEALDD server overload.

- The old SEALDD server and the new SEALDD server have stored the same data/content from the VAL server.

Figure 9.6.2.x-1: SEALDD seamless relcoation procedure

1. To improve the seamless SEALDD relocation, the old SEALDD server stops the downlink data transmission and sends the SEALDD traffic pending request to the SEALDD client, with the SEALDD flow ID, and optionally, the uplink data transmission pending indication.

2. The SEALDD client stops the uplink data transmission, and responds to the old SEALDD server with the downlink data breakpoint information (e.g. downlink packet sequence number already received in SEALDD client).

Editor's Note:	Whether the old SEALDD server can obtain the downlink data breakpoint information by itselt, without steps 1-2 is FFS.

3. The old SEALDD server pushs the SEALDD context to the new SEALDD server with the downlink data breakpoint information received in step 2, as defined in clause 9.6.3.1.

4. After completing SEALDD context transfer, the old SEALDD server sends the SEALDD traffic restart request to the SEALDD client with the connectivity information for new SEALDD server (e.g. SEALDD-UU address/port), the uplink data breakpoint information (e.g. uplink packet sequence number already received in old SEALDD server), and optionally, the uplink data transmission restart indication,

5. The SEALDD client responds to the old SEALDD server with the successful or failed result.

Editor's Note:	For steps 1, 2, 4 and 5, whether it can be replaced by other procedures in EDGEAPP is FFS.

6. The SEALDD client connects to the new SEALDD server, and transmits the uplink traffic to the new SEALDD server based on the uplink data breakpoint information. Similarly, the new SEALDD server transmits the downlink traffic to the SEALDD client based on the downlink data breakpoint information.

Editor's Note:	Whether the packet sequence number can be used to describe the data breakpoint information is FFS.

Editor's Note:	 The detailed information flows and APIs for steps 1, 2, 4, and 5 are FFS.

* * * Next Change * * * *

9.6.3.1	SEALDD context push request

Table 9.6.3.1-1 describes the information flow from the old SEALDD server to the new SEALDD server to push the SEALDD context.

Table 9.6.3.1-1: SEALDD context push request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | Identifies the requestor (i.e. old SEALDD server). |
| SEALDD-Uu Context | M | Identifies the context related to SEALDD-Uu connection, which is created upon SEALDD connection establishment. |
| SEALDD-S Context | M | Identifies the context related to SEALDD-S subscription. |
| Transport layer context | O | Identifies the context related to Transport layer (e.g. TCP/TLS/QUIC) for SEALDD-Uu user plane communication, also includes the downlink data breakpoint information. |