3GPP TSG-SA WG6 Meeting #59	S6-240589,

Athens, Greece 26th February – 1st March 2024	(revision of S6-240173)

Source:	China Mobile

Title:	pCR on Solution of downloading data channel application to UE

Spec:	3GPP TR 23.700-92 v0.3.0

Agenda item:	8.2

Document for:	Approval

Contact:	liuyueyjy@chinamobile.com

1. Introduction

This pCR proposes solution of downloading data channel application to UE which solves the Key Issue#2.

2. Reason for Change

Contribution with new text.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-92 v0.3.0.

* * * First Change * * * *

8.X	Solution #X: downloading data channel application to UE

8.X.1	Description

This solution resolves Key Issue #2 about downloading data channel application to UE.

In this procedure, the application list associated with necessary information of each Data Channel Applications is stored in the eMMTel Enabler Server based on the configuration information set by data channel application provider or eMMTel service provider via Controlling Application Server.

When the bootstrap data channels have been established the eMMTel Client in the UE send application request messages to eMMTel Enabler Server to request a data channel application or an application list if multiple DC applications are available, via the established bootstrap data channel.The detailed procedure to download data channel applications to the UE and the related information flow are specified in this solution.

NOTE:	The downloading procedures may goes over the top of DCSF. The detailed procedures to download data channel applications to the UE are out of scope of 3GPP TS 23.228 [3]

The functional model for downloading data channel application to UE is illustrated in Figure 8.x.1-1.

Figure 8.x.1-1: functional model for downloading data channel application to UE

The the application request messages sent by eMMTel Client in the UE goes though the bootstrap data channel to eMMTel Enabler Server via eMMTel-1 reference point.

8.X.2	Procedures

8.X.2.1	Downloading data channel application list to UE

The procedures in this clause are detailed procedures to download data channel applications list which apply to the step 20-2, 21-2, 22-2 and 23-2 of Annex AC.7.1 of 3GPP TS 23.228 [3].

The procedure of eMMTel Client in the UE sending application request messages to eMMTel Enabler Server via bootstrap data channel to request the application list is illustrated in Figure 8.x.2.1-2.

NOTE 1:	The eMMTel Client interact with DCMTSI Client in the same UE to control the downloading behavior on DCMTSI Client based on the application policy provided by the eMMTel Enabler Server. The interaction between eMMTel Client and DCMTSI Client.is internal interaction in the dialler application on the UE and implementation specific.

NOTE 2:	the DCMTSI client is responsible for creating bootstrap channel and actually downloading the data channel applications. To simply the procedures, the DCMTSI client is not shown in the procedures in this solution.

Pre-conditions:

- The bootstrap data channels have been established as specified in 3GPP TS 23.228 [3].

Figure 8.x.2.1-2: eMMTel Client in the UE requests the Data Channel Application list

When the eMMTel service provider provides a Root application which specified in GSMA NG.134 [x], containing layout, and/or home page etc. of the Data Channel Application list, step 1, step 2 and step 3 apply: Otherwise the step 1, step 2 and step 3 are skipped.

NOTE 1:	The user experience of using the eMMTel service may significantly different in devices provided by different vendors if the layout, and/or home page etc.are all implementation specific. E.g. in some devices, the available Data Channel Applications may be selected from a menu locally implemented in the device, but in some other devices only the first Data Channel Application in the Data Channel Application list can be selected and used automatically by the device. Layout, and/or home page etc. of the Data Channel Application list provided by eMMTel service provider may reduce the difference of user experience.

1.	The eMMTel Client in the UE sends a Get Root application request to eMMTel Enabler Server to get the Root application (e.g. layout, and/or home page etc. of the Data Channel Application list). The request message includes information elements as specified in Table 8.x.2.1-1.

Table 8.x.2.1-1: Information elements in Get Root application request

| Information element | Status | Description |
| --- | --- | --- |
| UE Information (see NOTE) | M | The UE related information required to identify the Root application (e.g. device type, device vendor, etc) |
| Root application version | O | The newest version of the Root application stored in the eMMTel Client in the UE. This IE presents if Root application is locally stored in the eMMTel Client. |
| NOTE: The eMMTel service provider can provide the eMMTel Client with different Root application based on this IE. |  |  |

2.	The eMMTel Enabler Server checks whether the Root application version included in the Get Root application request is the newest version of the Root application.If the Root application version IE is not included, the Root application version is not considered as newest.

3.	The eMMTel Enabler Server sends a Get Root application response to the eMMTel Client in the UE. The response message includes information elements as specified in Table 8.x.2.1-2.

Table 8.x.2.1-2: Information elements for Get Root application response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of Get Root application request. |
| Update needed | M | Indicates whether the Root application is needed to be updated by the eMMTel Client in the UE |
| Root application version | O | The newest version of the Root application. This IE presents only if the Root application is needed to be downloaded or updated by the eMMTel Client in the UE. |
| Root application (see NOTE) | O | The Root application provided by the eMMTel service provider, e.g. layout, and/or home page etc of the Data Channel Application list. This IE presents if the update of the Root application in the UE is needed, i.e. the Root application version included in the Get Root application request is not equal to the newest version on the eMMTel Enabler Server. |
| NOTE: The detailed information of this IE is implementation specific and out of scope of the present document.. |  |  |

- The eMMTel Client in the UE interects with the DCMTSI client in the same UE to send a Get Application List request to eMMTel Enabler Server to get the Data Channel Application list available for this user. The request message includes information elements as specified in Table 8.x.2.1-3.

Table 8.x.2.1-3: Information elements in Get Application List request

| Information element | Status | Description |
| --- | --- | --- |
| eMMTel Client version (see NOTE) | M | The version of eMMTel Client |
| Begin index | O | The starting index of the first Data Channel Application required in the Data Channel Application list. This IE presents if the requested Data Channel Application is not the first one in the Data Channel Application list, e.g. when the Data Channel Application list is needed to be shown in multiple pages, this IE is needed to be included when request the pages other than the first page. The default value of this IE is 0 if this IE is absent. |
| Required application number | O | The total number of Data Channel Applications required in this request |
| NOTE: The eMMTel service provider can provide the eMMTel Client with different Data Channel Application list based on this IE. |  |  |

- The eMMTel Enabler Server sends a  Get Application List response to the eMMTel Client in the UE. The response message includes information elements as specified in Table 8.x.2.1-4.

Table 8.x.2.1-4: Information elements in Get Application List response

| Information element | Status | Description |
| --- | --- | --- |
| Data Channel Application list | M | A list of Data Channel Applications available for this user. Each element in this list contains a DC application profile of this Data Channel Application. The detailed information elements of DC application profile are listed in Table 8.x.2.1-5. |

Table 8.x.2.1-5: Information elements in DC application profile

| Information element | Status | Description |
| --- | --- | --- |
| Application ID | M | Identifier of the Data Channel Application |
| Application Name | O | Name of the Data Channel Application |
| Application Icon | O | Icon of the Data Channel Application |
| Application Loading Phase | O | Indicates when this Data Channel Application is allowed to be used. The values of this IE include: - Precall:the Data Channel Application is allowed to be used before the eMMTel call session is established, i.e. after the 18x response is sent/received and before the 200 OK of the initial SIP INVITE request is sent/received. - Incall:the Data Channel Application is allowed to be used after the eMMTel call session is established, i.e. after the 200 OK of the initial SIP INVITE request is sent/received. |
| Autoload | O | Indicates whether this Data Channel Application is needed to be load to the UE automatically. |
| Autolaunch (see NOTE) | O | Indicates whether this Data Channel Application is needed to be downloaded to the UE and run automatically. |
| If Work Without Peer DC | O | Indicates whether this Data Channel Application can be used if Data Channel is not supported by the other party of the call. |
| Supported Scenario | O | Indicates supported media type required for this Data Channel Application. The values of this IE include: - Voice call only: this Data Channel Application can be used if and only if the corresponding call is a voice call. - Video call only: this Data Channel Application can be used if and only if the corresponding call is a video call. - Voice and video call: this Data Channel Application can be used in both voice call and video call. |
| 3gpp Qos Hint | O | Indicates the QoS requirement of this Data Channel Application which is used to set the value of “a=3gpp-qos-hint” attribute in SDP for the data channel(s) used by the application. |
| NOTE: The Autolaunch IE can be applied only if the specific service using this data channel application is agreed to be used by the user based on the subscription of this specific service, or explicitly agreed to be autolaunched by the user on the UE. |  |  |

8.X.2.2	Downloading data channel application to UE

After the Data Channel Application list downloaded to the eMMTel Client in the UE, the eMMTel Client in the UE may download a specific data channel application from the Data Channel Application list and based on the user’s selection or the Autoload/Autolaunch IE included in the DC application profile.

These procedures are detailed procedures to download data channel applications which apply in the step 20-3, 21-3, 22-3 and 23-3 of Annex AC.7.1 of 3GPP TS 23.228 [3].

The procedure of eMMTel Client in the UE downloading a Data Channel Application from the eMMTel Enabler Server via bootstrap data channel is illustrated in Figure 8.x.2.2-1.

Figure 8.x.2.2-1: eMMTel Client in the UE downloads an Data Channel Application from the eMMTel Enabler Server

1.	The eMMTel Client in the UE sends a Data Channel Application download request to the eMMTel Enabler Server to download the Data Channel Application. The request message includes information elements as specified in Table 8.x.2.2-1.

Table 8.x.2.2-1: Information elements in Data Channel Application download request

| Information element | Status | Description |
| --- | --- | --- |
| Application ID | M | Identifier of the Data Channel Application requested to be downloaded. |
| eMMTel Client version (see NOTE) | M | The version of eMMTel Client |
| Data Channel Application version | O | The version of the Data Channel Application locally stored on the eMMTel Client. This IE presents if a Data Channel Application with the requested Application ID is locally stored by the eMMTel Client. |
| NOTE: The eMMTel service provider can provide the eMMTel Client with different version of Data Channel Application based on this IE. |  |  |

2.	The eMMTel Enabler Server checks whether the Data Channel Application version included in the Data Channel Application download request is the newest version of the requested Data Channel Application. If the Data Channel Application version IE is not included, the Data Channel Application version is not considered as newest.

3.	The eMMTel Enabler Server sends a Data Channel Application download response to the eMMTel Client in the UE. The response message includes information elements as specified in Table 8.x.2.2-2.

Table 8.x.2.2-2: Information elements for Data Channel Application download response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of Data Channel Application download request. |
| Reason | O | Indicates the reason for failure. |
| Update needed | M | Indicates whether the Data Channel Application is needed to be updated by the eMMTel Client in the UE |
| Data Channel Application version | O | The newest version of the Data Channel Application. This IE presents only if the Data Channel Application is needed to be downloaded or updated by the eMMTel Client in the UE. |
| Data Channel Application (see NOTE) | O | The Data Channel Application requested to be downloaded This IE presents if the downloading of the Data Channel Application to the UE is needed, i.e. the Data Channel Application version included in the Data Channel Application download request is not equal to the newest version on the eMMTel Enabler Server. |
| NOTE: The detailed information of Data Channel Application is out of scope of the present document. |  |  |

8.X.3	Impacts on Existing Nodes and Functionality

This solution detailed specifies the procedures of downloading Data Channel Application list and Data Channel Application from network.

In this solution, the procedures may be handled by eMMTel Enabler Server and the related requests/responses are go over the top of the DCSF and do not impact the existing functionalities of DCSF.Therefore, this solution has no impact on 5GC/IMS.

* * * Next Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system".

[3]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS)".

[4]	OMA-AD-NGSI-V1_0-20120529-A: "Next Generation Service Interfaces Architecture".

[5]	OMA-TS-REST_NetAPI_ThirdPartyCall-V1_0-20130212-C: "RESTful Network API for Third Party Call".

[6]	OMA-TS-REST_NetAPI_CallNotification-V1_0-20200226-C: "RESTful Network API for Call Notification".

[7]	OMA-TS-REST_NetAPI_AudioCall-V1_0-20200226-C: "RESTful Network API for Audio Call".

[8]	3GPP TS 23.198: "Open Service Access (OSA)".

[9]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[10]	3GPP TR 22.873: "Study on evolution of the IP Multimedia Subsystem (IMS) multimedia telephony service".

[11]	GSMA NG.129: "IMS Data Channel White Paper v0.1".

[12]	GSMA TGY.02: "Business Voice Calling".

[13]	3GPP TR 23.700-87: "Study on system architecture enhancement for next generation real time communication".

[x]	GSMA NG.134: "IMS Data Channel".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".