3GPP TSG-SA WG6 Meeting #59	S6-240765

Athens, Greece, 26th February – 1st March 2024

Source:	Ericsson, Samsung, TNO

Title:	AI/ML Model storage and discovery procedures improvements

Spec:	3GPP TS 23.700-82 V0.2.0

Agenda item:	8.3

Document for:	Approval

1. Introduction

The current definition of the ML model storage and discovery procedures is not complete. This pCR provides information flows and updates for the procedures:

- AI/ML model information storage request:

- - the VAL service ID(s) is added to identify the VAL service that the ML model belongs;

- - the ML Model profile content is clarified, i.e., it may contain analytics ID, ML model interoperability indicator and/or information. The definitions of the ML model interoperability indicator and information are based on 23.288 and needed to avoid interopablity issues.

- - Clarification : it is impossible that one ML model may be related to different AnalyticsID, that is not aligned with SA6 design (see 23.436) ; thus, it is proposed to align SA6 solution with SA2 design principle.

- - the ML model storage requirements are added to indicate the ML model storage duration and security requirements

- In AI/ML model information discovery request, the alignment is done with AI/ML model information storage request.

- In clause 8.5.3, the statement "If ADAE is the AI/ML model consumer, then step 1 is skipped" implies that ADAE and AIML Enabler are the same service. This is a limitation of the solution and should be discussed during normative work when architechure is stable. Thus, it is proposed to remove this statement.

2. Reason for Change

The current definition of the ML model storage and discovery procedures is not complete.

3. Conclusions

This pCR proposes the information flows and clarifications on the ML model storage and discovery procedures.

4. Proposal

It is proposed to agree the following changes to 3GPP 23.700-82 V0.2.0.

* * * First Change * * * *

### 8.5.1	General

The following clauses specify procedures of AI/ML model management.

### 8.5.2	AI/ML model information storage procedure

Pre-conditions:

Model repository consumer wants to store the ML model in the Model repository.

Figure 8.5.2-1: AI/ML model information storage procedure

1.	The model repository consumer (e.g.  ADAE or AIML server with MTME function) sends an AI/ML model information storage request to the model repository to store a model. The model included in the request can be one trained by the repository consumer or its information can be provisioned to the repository consumer. The request contains information elements as described in table 8.5.4.1-1.

NOTE 1:	The detailed definition of the security requirements within the ML model storage requirements is within SA3 scope.

2.	Upon receiving the request, the model repository verifies if the model repository consumer is authorized to store the ML model identified by Analytics ID and/or ML model interoperability indicator provided within the ML model profile attribute. If the AI/ML model repository consumer is authorized the model repository processes the request and records information of the AI/ML model (e.g., by creating an AI/ML model profile). The model repository sends a response to the model repository consumer (e.g., MTME-enhanced ADAE or AIML server) with an identifier of the created AI/ML model profile.

NOTE 2:	The model repository may be implemented by A-ADRF.

NOTE 3:	It is to be determined in the evaluation phase which entities are can have direct interaction with the model repository, e.g., only AI/ML enabler and/ or MTME-enhanced ADAE or AIML, etc. This procedure is assumed to also enable other AI/ML service consumers to also access this functionality indirectly, via these enablers with direct access.

* * * Next Change * * * *

### 8.5.3	AI/ML model information discovery procedure

Figure 8.5.3-1: AI/ML model information discovery procedure

1.	The AI/ML model repository consumer (e.g., VAL server) sends an AI/ML model discovery request to the model repository. The request contains information elements as described in table 8.5.4.3-1.

2.	Upon receiving the discovery request, the model repository verifies if the AI/ML model repository consumer is authorized to discover the ML model(s) identified by the ML model ID, Analytics ID and/or List of allowed vendors. If the AI/ML model repository consumer is authorized, the model repository processes the request. If the model address is provided in step 1, the Model repository shall download the model from the consumer. Sends the response message to the repository consumer. The response includes information of the discovered AI/ML model. Optionally, the model repository can initiate a model transfer process to the consumer.

NOTE:	It is to be determined in the evaluation phase which entities are can have direct interaction with the model repository, e.g., only AI/ML enabler and/ or MTME-enhanced ADAE, etc. This procedure is assumed to also enable other AI/ML service consumers to also access this functionality indirectly, via these enablers with direct access.

* * * Next Change * * * *

### 8.5.4	Information flows

#### 8.5.4.1	AI/ML model information storage request

Table 8.5.4.1-1 describes the information flow from the repository consumer to the model repository as a request for the AI/ML model storage.

Table 8.5.4.1-1: AI/ML model information storage request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M (NOTE 1) | The identity of the model repository consumer performing the request. |
| VAL service ID(s) | O | Identify the VAL service ID(s). |
| ML model | O (NOTE 2) | Provides the trained AI/ML model. |
| ML model address | O (NOTE 2) | Represents the ML model address that can be used by Model repository to download the ML model. |
| ML model profile | O (NOTE 2) | Provides the AI/ML model profile. |
| > Analytics ID | O (NOTE 3) | Represents analytics ID for which the model can be used. |
| > List of allowed vendors | O (NOTE 3) | Indicates which vendors that are allowed to retrieve the ML model and thereby also are interoperable to the model. |
| > ML model interoperablility information | O (NOTE 3) | Represents the vendor-specific information that conveys, e.g., requested model file format, model execution environment, etc. The encoding, format, and value of ML Model Interoperable Information is not specified since it is vendor specific information, and is agreed between vendors, if necessary for sharing purposes. |
| > Training data information | O (NOTE 3) | Describes the information of the training data with which the model is trained. |
| > Base model ID | O (NOTE 3) | Represents the identifier of the base model if the model has been trained using Transfer Learning |
| > ML model metadata | O (NOTE 3) | Represents the metadata of the ML model training process. |
| > ML model storage requirements | O (NOTE 3) | Represents the requirements for the ML model storage. |
| >> Storage duration | O | Represents the ML model storage duration time. When the storage duration time is expired, the stored ML model and the related information shall be deleted. |
| >> Security requirements | O | Represents the information security requirements for storing the ML model information. |
| NOTE 1: This information element shall not be updated. NOTE 2: At least one of these information elements shall be provided. NOTE 3: At least one of these information elements shall be provided. |  |  |

NOTE:	The security requirements for downloading the ML model according to the ML model address IE are within SA3 scope.

Editor's Note: The ML model retrieval access control (e.g., when the ML model is in training or re-trainting phase) is FFS.

Editor's Note: Whether additional IE are required to support Transfer Learning is FFS.

Editor's Note: Whether the Training data information, ML model metadata is needed is FFS.

#### 8.5.4.2	AI/ML model information storage response

Table 8.5.4.2-1 describes the information flow from the model repository to the repository consumer as a response for the AI/ML model storage request.

Table 8.5.4.2-1: AI/ML model information storage response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of the request |
| > ML model ID | O | Represents the identifier of the stored ML model. This information element shall be provided if the result is success. |

#### 8.5.4.3	AI/ML model information discovery request

Table 8.5.4.3-1 describes the information flow from the repository consumer to the model repository as a request for the AI/ML model discovery.

Table 8.5.4.3-1: AI/ML model information discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the model repository consumer performing the request. |
| VAL service ID | M | Identify the VAL service ID. |
| Filtering criteria | M | Represents the filtering criteria |
| > ML model ID | O (NOTE 1) | Represents the identifier of the stored ML model. |
| > ML model filter criteria | O (NOTE 1) | Provides the AI/ML model profile. See Table 8.5.4.1-1 for the detailed definition. |
| >> Analytics ID | O (NOTE 2) | Represents analytics ID for which the model can be used. |
| >> List of allowed vendors | O (NOTE 2) | Indicates which vendors that are allowed to retrieve the ML model and thereby also are interoperable to the model. |
| >> ML model interoperablility information | O (NOTE 2) | Represents the vendor-specific information that conveys, e.g., requested model file format, model execution environment, etc. The encoding, format, and value of ML Model Interoperable Information is not specified since it is vendor specific information, and is agreed between vendors, if necessary for sharing purposes. |
| NOTE 1: At least one of these information elements shall be provided. NOTE 2: At least one of these information elements shall be provided. |  |  |

#### 8.5.4.4	AI/ML model information discovery response

Table 8.5.4.4-1 describes the information flow from the model repository to the repository consumer as a response for the AI/ML model discovery request.

Table 8.5.4.4-1: AI/ML model information discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of the request |
| > ML model | O (NOTE) | Represents the ML model. This information element may be provided if the result is success. |
| > ML model address | O (NOTE) | Represents the ML model address at Model repository that can be used to download the ML model. This information element may be provided if the result is success. |
| NOTE: Only one of these information elements shall be provided. |  |  |

# *** End of Changes ***