3GPP TSG-SA WG6 Meeting #60	S6-241108

Changsha, China, 15th - 19th April 2024	(revision of S6-24xxxx)

Title:	[draft] LS on the support of ECN marking L4S in MCVideo services

Response to:

Release:	Rel-19

Work Item:	enhMC

Source:	SA6

To:	SA4

Cc:	CT1

Contact person:	Rana Alhalaseh

Rana.alhalaseh@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA6 has discussed the 5G network capabilities related to high data rate and low latency services in the focus of MCVideo services enhancements. In specific, the support of Low Latency, Low Loss and Scalable Throughput (L4S), which exposes the congestion information by marking the Explicit Congestion Notification (ECN) bits in the IP header of the user`s IP packets between the UE and the application server so that the respective MCVideo sender can adapt the application layer bit rate accordingly.

SA6 believes that ECN marking for L4S capability is essential for MCVideo services to maintain reliable and low-latency video services for mission critical users even in congestion scenarios. In case of accidents when large number of MC service users (e.g., police, firefighters, etc.) are located in a certain area to assist in the event, congestion can occur and the quality of the MCVideo services decreases due to the existing conditions. Therefore, ECN marking for L4S can play a significant role in adjusting the bit rate to maintain the maximum viable MCVideo services quality, meet the MC service availability requirements and MC KPIs for these MC service users.

Furthermore, the ECN marking for L4S can create the possibility to provide situational awareness for MC service users to report the current downlink congestion status back to the network and MC service server.

Therefore, SA6 would like to liaison with SA4 on the 5G network capability mentioned above for potential updates and alignment in 3GPP TS 23.281 and 3GPP TS 26.281 within Release 19.

# 2	Actions

To SA4

ACTION: As media handling, codec negotiation, application layer bit rate and related details are within the regime of SA4, SA6 kindly ask SA4 for the possible investigation of the network capability mentioned above for MCVideo services and potentially update 3GPP TS 26.281 accordingly, in order for SA6 to align application layer aspects appropriately.

# 3	Dates of next TSG SA WG 6 meetings

SA6#61                             20th May – 24th May 2024 	Jeju, Korea

SA6#62-Ad-Hoc-e             10th July – 18th July 2024 	e-meeting