3GPP TSG-SA WG6 Meeting #60	S6-241141

Changsha, China 9th – 13th April 2024	(revision of S6-241xxx)

Source:	Huawei, HiSilicon

Title:	pCR on Solution of updating data channel application profiles to UE

Spec:	3GPP TR 23.700-92 v0.4.0

Agenda item:	8.2

Document for:	Approval

Contact:	wanghan28@huawei.com

1. Introduction

This pCR proposes solution of updating data channel application profiles to UE which solves the Key Issue#2.

2. Reason for Change

Contribution with new text.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-92 v0.4.0.

* * * First Change * * * *

## 8.X	Solution #X: updating of data channel application profiles to UE

### 8.X.1	Description

This solution resolves Key Issue #2 about download data channel application to UE.

There are two modes in this procedure: one is the eMMTel Enabler Server notifies the eMMTel Client in the UE to send application request messages to the eMMTel Enabler Server to request the profiles of data channel applications; another is the eMMTel Enabler Server sends the profiles to the UE directly after the UE subscribes to the profile notification. The detailed procedure of updating data channel application profiles to the UE and the related information flow are specified in this solution below.

NOTE 1:	The updating procedures may go over the top of DCSF. The detailed procedures to update data channel applications to the UE are out of scope of 3GPP TS 23.228 [3].

The functional model for updating data channel application profiles to UE is illustrated in Figure 8.x.1-1.

Figure 8.x.1-1: functional model for updating data channel application to UE

The notification management is a SEAL service that offers the notification functionality to one or more verticals, the notification management server is a functional entity that handles the notification management aspects by interacting with the notification management client as specified in clause 17 of TS 23.434 [x]. The notification messages sent by the eMMTel Enabler Server to the eMMTel Client via NM-UU reference point.

### 8.X.2	Procedures

#### 8.x.2.1	The eMMTel Client request profiles based on notification from the eMMTel Enabler Server

The difference between this procedure and solution#3 of the current specification was the first step which indicates the eMMTel Enabler Server can notify the eMMTel Client in the UE to send application request messages to the eMMTel Enabler Server to request the profiles of data channel applications, the detailed procedure is illustrated in Figure 8.x.2.1-1.

NOTE:	The eMMTel Enabler Server can instruct the eMMTel Client to request root application and data channel application profiles as required. e.g. root application version update, offline of a data channel application, etc.

Pre-condition:

1)	The bootstrap data channels have been established as specified in 3GPP TS 23.228 [3].

Figure 8.x.2.1-1: eMMTel Server notifies eMMTel Client to request the Data Channel Application profile list

- The notification channel has been established between notification management client and notification management Server as specified clause 17.3.3 of TS 23.434 [x].

- The eMMTel client subscribes to the eMMTel Enabler server for receiving notifications.

- The eMMTel Enabler Server notifies the eMMTel Client in the UE to get the Root application or data channel profile. The notification includes information elements as specified in Table 8.x.2.1-1. The remaining steps are the same as those in solution#3 of the current specification.

Table 8.x.2.1-1: Information elements in notification

| Information element | Status | Description |
| --- | --- | --- |
| Urgent | O | Indicates whether the operation is urgent to UE. |
| Root application version | O | The newest version of the Root application. |

#### 8.x.2.2	The eMMTel Enabler Server notifies the DC application profile to the eMMTel Client directly

The eMMTel Enabler Server notifies the data channel application profile to the eMMTel Client is illustrated in Figure 8.x.2.2-1.

Pre-condition:

1)	The bootstrap data channels have been established as specified in 3GPP TS 23.228 [3].

Figure 8.x.2.2-1: eMMTel Enabler Server notifies the Data Channel Application profile list to UE

0.	The notification channel has been established between notification management client and notification management Server as specified clause 17.3.3 of TS 23.434 [x].

1.	The eMMTel client subscribes to the eMMTel Enabler server for receiving notifications.

2.	The eMMTel Enabler Server check the provision of Root application and data channel application profile list is needed, e.g. the UE has IMS DC capability, an application version or application profile update is available, etc.

NOTE:	The detail additional checklists in this step are out of scope of the present document.

3.	The eMMTel Enabler Server sends a Root application notification message to the eMMTel Client in the UE. The notification message includes information elements as specified in Table 8.x.2.2-1.

Table 8.x.2.2-1: Information elements for Root application notification

| Information element | Status | Description |
| --- | --- | --- |
| Update needed | M | Indicates whether the Root application is needed to be updated by the eMMTel Client in the UE |
| Root application version | O | The newest version of the Root application. This IE presents only if the Root application is needed to be updated by the eMMTel Client in the UE. |
| Root application (see NOTE) | O | The Root application provided by the eMMTel service provider, e.g. layout, and/or home page etc of the Data Channel Application list. This IE presents if the update of the Root application in the UE is needed, i.e. the Root application version included in the Get Root application request is not equal to the newest version on the eMMTel Enabler Server. |
| NOTE: The detailed information of this IE is implementation specific and out of scope of the present document. |  |  |

- The eMMTel Enabler Server sends a data channel Application Profile List notification to the eMMTel Client in the UE. The notification message includes information elements are as same as the elements in Table 8.3.2.1-4.

### 8.X.3	Impacts on Existing Nodes and Functionality

In this solution, the procedure is handled by eMMTel client and eMMTel Enabler Server and go over the top of the DCSF.Therefore, this solution has no impact on 5GC/IMS.

* * * End of Changes * * * *