3GPP TSG-SA WG6 Meeting #60	S6-241248

Changsha, China 15th – 19th April 2024	(revision of S6-241xxx)

Source:	KPN N.V.

Title:	Solution #10 update: Service Permission Level

Spec:	3GPP TR 23.700-82 v0.3.0

Agenda item:	8.3

Document for:	Approval

Contact:	Yonatan Shiferaw, yonatan.shiferaw@tno.nl

1. Introduction

This contribution updates Solution #10 to add Service Permission Level corsponding to the VAL Serivces in the list of configurations.

2. Reason for Changes

In clause 8.10.3.1, Table 8.10.3.1-1 details the AI/ML member participation configurations proviosioing and management request. In the request the AI/ML member includes the list of VAL service ID(s) for which the configuration applies and other parameters that can be used to select the AI/ML member for an on going AI/ML operation. This contribution adds Service Permission Level in the list of parameters. This is useful because an AI/ML member may not provide the same service for all VAL services: for some VAL services it may be able to give majority of the resouce available while in others it may be able to give a fraction,  for some VAL services it may be able to provide the raw data in the AI/ML operation while in other it only provides processed data. In order to be able to take into consideration the Service Permission Level, this contribution proposes to add Service Permission Level in Table 8.10.3.1-1

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-82 v0.3.0.

* * * First Change * * * *

## 8.10	Solution #10: AI/ML member participation configurations provisioning and management

### 8.10.1	General

The following clauses specify procedures, information flows and APIs for Key issue #A to provision the AI/ML member participation configurations for the AI/ML Enabler services.

Editor's note:	 The relationship between the solution and KI#3 solutions needs to be evaluated and is FFS.

Assumptions:

-	AI/ML member (e.g., AI/ML Enabler Client) is registered on the AI/ML Enabler Server to participate in the AI/ML operations (e.g., ML model training).

NOTE:	The AI/ML member configurations provisioning service operation can be a part of the ML client config provisioning procedure defined in clause 8.3.2.

### 8.10.2	Procedures

#### 8.10.2.1	AI/ML member participation configurations provisioning and management

Figure 8.10.2.1-1: AI/ML member participation configurations provisioning and management procedure

1.	The AI/ML member (e.g. AI/ML Client) sends an AI/ML member participation configurations provisioning and management request as defined in clause 8.10.3.1.

2.	Upon receiving the request, the AI/ML Enabler server performs an authorization check of the AI/ML member.

3.	If the AI/ML member is authorised, the AI/ML Enabler Server stores the AI/ML member configurations and applies the provisioned configurations to ongoing and further the AI/ML operations (e.g., ML model training).

4.	AI/ML Enabler Server provides the response to the AI/ML member with a status via AI/ML member participation configurations provisioning and management response defined in clause 8.10.3.2.

NOTE:	The service operation defined in clause 8.10.2.1 can be utilized for AI/ML member participation configurations management (i.e., update/delete) functionality.

Editor's Note: It is FFS how an FL client repository function is used in this procedure.

### 8.10.3	Information flows

#### 8.10.3.1	AI/ML member participation configurations provisioning and management request

Table 8.10.3.1-1 describes the information flow from the AI/ML member to the AI/ML Enabler server as a request for the AI/ML member participation configurations provisioning and management.

Table 8.10.3.1-1: AI/ML member participation configurations provisioning and management request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M (NOTE) | The identity of the AI/ML member performing the request. |
| List of configurations | M | Identify the list of configurations. |
| > List of VAL service ID(s) | M | Identify the list of VAL service IDs for which the configurations are applicable. |
| > List of time-schedule configurations | O | Provides the list of time-schedule configurations, e.g., the AI/ML member is (not) available to participate in the AI/ML operations in the given time slot(s) and/or day(s) of the week. |
| > List of location-based configurations | O | Provides the list of location-based configurations, e.g., the AI/ML member is (not) available to participate in the AI/ML operations in the given locations represented by coordinates, civic addresses, network areas, or VAL service area ID. |
| > List of compute-utilization based configurations | O | Provides the AI/ML member participation availability based on list of compute (GPU, CPU, AIPU, NPU) utilization based configurations, e.g. the AI/ML member may participate the AI/ML operation if the GPU load is less than the given percent. |
| > Mode of member participation | O | Indicates the AI/ML member availability for the participation based on the modes like training (e.g., FL, DML, ML, and/or Split Learning), aggregator, coordinator and/or inference. |
| > Service permission level | O | Indicates level of service permission for the corsponding VAL service ID e.g. premium resource usage, limited resource usage, raw data, processed data. |
| NOTE: This information element shall not be updated in the AI/ML member participation configurations management request. |  |  |

NOTE:	In Table 8.10.3.1-1, the configurations identify the conditions and criteria, when satisfied the AI/ML member can participate in the AI/ML operations initiated by VAL service(s).

Editor's note: Whether ML model requirement, dataset requirement and other AI/ML capability are needed is FFS.

#### 8.10.3.2	AI/ML member participation configurations provisioning and management response

Table 8.10.3.2-1 describes the information flow from the AI/ML Enabler server to the AI/ML member as a response for the AI/ML member participation configurations provisioning and management.

Table 8.10.3.2-1: AI/ML member participation configurations provisioning and management response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of the request. |

* * * End of Change * * * *