3GPP TSG-SA WG6 Meeting #60	S6-241498

Changsha, China 15th-19th April 2024	(revision of S6-241190 was 0706)

Source:	Convida Wireless, LLC

Title:	Multi-modal SEALDD policy configuration

Spec:	3GPP TR 23.700-23 V0.2.0

Agenda item:	8.5

Document for:	Approval

Contact:	Catalina Mladin, mladin.catalina@convidawireless.com

1. Introduction

This contribution provides a solution for KI #2 - E2E Multi-Modal Communication Flows.

2. Reason for Change

To effectively support multi-modal application traffic flows with SEALDD, SEALDD servers and clients must have awareness of the individual application flows having multi-modal dependencies on each another. To provide this awareness, the contribution proposes extending the current SEALDD policy functionality to support multi-modal SEALDD policy functionality.

3. Proposal

It is proposed to agree the following changes to 3GPP 23.700-23 V0.2.0.

# *** All new text ***

## 7.x	Solution #X: SEALDD Policy Configuration for multi-modal flows

### 7.x.1	Architecture Impacts

This solution adds support for multi-modal SEALDD policy configuration functionality by enhancing the existing SEALDD policy configuration functionality supported in clause 9.10 of TS 23.433[x]. There are no impacts to the  SEALDD architecture.

### 7.x.2	Solution description

This solution is targeting KI #2: E2E Multi-Modal Communication Flows and specifies enhancements to the SEALDD policy configuration information flows. In this procedure, the VAL server is a specific server for configuring multi-modal SEALDD policies and is different from VAL servers used for VAL application processing.

The SEALDD policy configuration procedure detailed in 3GPP TS23.433[x]  clause 9.10.2.1 is reused with the enhanced information flows below

NOTE: Corresponding enhancements to multi-modal SEALDD policy update and delete procedures will be completed during the normative phase.

### 7.x.3	Information flows

#### 7.x.3.1	SEALDD policy configuration request for multi-modal flows

Table 7.x.3.1-1 describes the new information elements to be added to the SEALDD policy configuration request in 3GPP TS 23.434[x] Table 9.10.3.1-1  in order to support multi-modal flows.

Table 7.x.3.1-1:  SEALDD policy configuration request IEs for multi-modal flows

| Information element | Status | Description |
| --- | --- | --- |
| Multi-modal VAL flows list | M | List of individual VAL flows applicable to this multi-modal SEALDD policy. |
| > Application traffic identifiers | M | Identifier of the application traffic for this individual VAL flow (e.g. VAL server ID, VAL service ID) |
| > VAL UE identity | O | Identifier of the VAL UE for this individual VAL flow |
| > SEALDD policy | M | SEALDD policy associated with this individual VAL flow as defined in Table 9.10.3.1-1 of TS 23.433 |
| Multi-modal SEALDD policy | M | Multi-modal SEALDD policy applicable to Multi-modal VAL flows list. This IE is used for complementing other SEALDD measurement policies for multi-modal flows |
| > Burst policy | O (See NOTE 1) | Policy used by the SEALDD layer to determine multi-modal flow configuration |
| > Synchronization policy | O (See NOTE 1) | Policy used by the SEALDD layer to perform multi-modal synchronization operations. |
| > Policy expiration time | O | Indicates the validity period of the policy |
| NOTE 1: At least one of these multi-modal SEALDD policies shall be present in the request |  |  |

NOTE 2: 	The burst policy and the synchronization policy, and their use, are to be harmonized with KI#1 solutions.

Editor’s Note:	How to harmonize the burst related parameters considering burst policy in DD policy is FFS.

### 7.x.4	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

# *** End of Changes ***