3GPP TSG-SA WG6 Meeting #60	S6-241501

Changsha, China, 15th – 19th April, 2024

Source:	Ericsson

Title:	Support multi-modal service identifier in SEALDD

Spec:	3GPP TS 23.700-23 V0.2.0

Agenda item:	8.5

Document for:	Approval

1. Introduction

The XR traffic has Multi-Modal nature that requires the improvement in the existing SEAL Data Delivery service to support the multi-modal indication for the SEALDD enabled data transmission quality of service measurement.

2. Reason for Change

KI#2 has the following open issues:

-	Whether and how to support the interaction between the application enablement layer and 5G CN to manage E2E multi-modal communication flows between application clients and application servers?

-	Whether and how SEALDD may be enhanced to assist in managing E2E multi-modal communication flows between application clients and application servers involved in the same application service (e.g., support for multi-modal aware SEALDD flow management and policies)?

In order to support the XR scenario, the SEALDD layer require additional indication for addregation of multiple SEALDD connections into Multi-Modal Service, i.e., the VAL server may include multiple flows and multi-modal service ID into the Sdd_RegularTransmission request; and the SEALDD layer based on VAL Service ID and VAL Server ID can determine the multimodal service requirements for negotiation with 5GC.

This change enables the tracking of Multi-Modal Flows in the SEALDD layer and can be further used for harmonization of the SEALDD policies and QoS measurement/exposuse.

3. Conclusions

This pCR proposes a solution to address multi-modal service for XR traffic, which simplifies VAL server handling.

4. Proposal

It is proposed to agree the following changes to 3GPP 23.700-23 V0.1.0.

* * * First Change * * * *

## 7.1	Mapping of solutions to key issues

Table 7.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |
| Sol #2 |  |  |  |  |
| Sol #X |  | x |  |  |

* * * Next Change (all texts are new) * * * *

## 7.x	Solution #x: Support of Multi-Modal traffic indication in SEALDD layer

### 7.x.1	Architecture Impacts

This solution is based on architecture of SEALDD as described in 3GPP TS 23.433 [23433].

### 7.x.2	Solution description

#### 7.x.2.1	General

This solution addesses the improvement of the SEALDD layer to support Multi-Modal service.

The solution improves the SEALDD layer defined in 3GPP TS 23.433, i.e.:

1.	The multiple flows can be requested in one SEALDD enabled regular data transmission connection establishment.

2.	The SEALDD server uses 5GC capabilities to configure XR traffic flows.

In this solution, the following definitions are used:

1.	Multi-Modal Service Identifier is an identifier of the multi-modal service as defined in clause 6.1.3.27.3 of 3GPP TS 23.503 [23503].

2.	Multi-Modal Service Requirements is requirements for the multi-modal traffic handling in 5GC as defined in clause 6.1.3.27.3 of 3GPP TS 23.503 [23503].

In this solution, the following assumptions are used:

1.	The VAL service identified by the VAL Service ID serves only one XR application. Thus, the VAL Service ID functionally equal to the Multi-Modal Service Identifier and may be generated by the SEALDD server based on VAL service ID and used by the SEALDD server for negotiation with 5GC via N33/N5 interfaces for the XR traffic handling.

2.	The VAL server(s) within one VAL Service may provide different functional parts of the XR application, e.g., VAL Server A performes video flow(s) processing, VAL Server B performs audio flow(s) processing, and/or VAL Server C preforms both video and audio flows processing. Thus, the Multi-Modal Service Requirements for the XR flows may be determined and generated by the SEALDD server based on VAL service ID and/or VAL server ID for negotiation with 5GC via N33/N5 interfaces for the XR traffic handling.

#### 7.x.2.2	Impact to existing SEALDD procedures

The SEALDD procedures and information flows in 3GPP TS 23.433 [23433] can be enhanced (highlighted in bold italics) as follows.

NOTE:	SEALDD data transmission procedures with single SEALDD connection are described as typical example below.

#### 9.2.2.2 SEALDD enabled regular data transmission connection establishment procedure Figure 9.2.2.2-1 illustrate the procedure for establishing regular SEALDD data transmission connection. Pre-condition: - The VAL server can discover and select the SEALDD server by CAPIF functions. Figure 9.2.2.2-1: SEALDD enabled regular data transmission connection establishment procedure 1. The VAL server decides to use SEALDD service for application traffic transfer and allocates address/port as SEALDD-S Data transmission connection information for receiving the data packets from SEALDD server. The VAL server sends Sdd_RegularTransmission request to the SEALDD server discovered by CAPIF. The service request includes UE ID/address, VAL server ID, VAL service ID, list of requested flows, including SEALDD-S Data transmission connection information of the VAL server side, and optionally, the QoS information for the application traffic, e.g. QoS requirements. 2. Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, SEALDD server allocates address/port of the SEALDD server to receive the packets from the VAL server for application data transfer as SEALDD-S data transmission connection information of the SEALDD server side for each requested flow provided in step 1. The SEALDD server allocates a specific address or port used for SEALDD traffic transfer with the specific UE for the VAL server and responds with a SEALDD service response (including SEALDD-S data transmission connection information of the SEALDD server side) for each requested flow provided in step 1. The VAL server and SEALDD server can use SEALDD-S data transmission connection information to establish the data transmission connection between VAL server and SEALDD server for application data transfer. The SEALDD server may send the AF request to provide the required QoS information to 5GC via N33/N5 for each requested flow provided in step 1, as defined in clause 5.2.6.9 and in clause 5.2.5.3 of 3GPP TS 23.502 [6]. The AF request includes the application traffic descriptor containing the address or ports allocated by SEALDD server, and the QoS information for application traffic, and may include Multi-Modal Service ID together with Multi-modal Service Requirements. The QoS information may be determined by SEALDD server according to VAL service ID for different service type of application traffic if the QoS information is not provided by VAL server. The Multi-Modal Service ID may be determined by the SEALDD server according to VAL service ID. The Multi-modal Service Requirements may be determined by the SEALDD server according to VAL service ID and/or VAL server ID. The SEALDD server relies on the northbound Policy Authorization Service API exposed by the PCF as specified in 3GPP TS 23.502 [6] and 3GPP TS 23.503 [7], if the SEALDD server is connected to the PCF via the N5 reference point, or the northbound AF Session with QoS Service APIs and/or the PFD Management northbound APIs exposed by the NEF as specified in 3GPP TS 23.502 [6] and 3GPP TS 23.503 [7], if the SEALDD server is connected to the PCF via NEF. SEALDD may also rely upon the EES Session with QovS API as specified in 3GPP TS 23.558 [10] and/or the NRM QoS functionality as described in 3GPP TS 23.434 [4]. NOTE 1: The SEALDD-S data transmission connection information of the SEALDD server side is optional to respond to the VAL server, if the SEALDD server uses the downlink pull mode to obtain the data/content from the address provided by the VAL server in step 1, and uses the uplink push mode to send the data/content to the address provided by VAL server. 3. Data transmission sessions information is provisioned to the VAL client by the VAL server via application signalling. NOTE 2: The application signalling may be transmitted via direct application layer connection or via the SEALDD layer. 4. The VAL client sends a SEALDD service request to SEALDD client. The VAL client receives a SEALDD service response to the SEALDD client. The response indicates that whether the SEALDD service request is successful or not. 5. The VAL/SEALDD client discover and select the proper SEALDD server for the VAL application, as described in clause 9.4.3. After this step, the VAL server is discovered and selected along with the associated SEALDD server, the SEALDD client can get the SEALDD server's address. 6. The SEALDD client allocates a SEALDD flow ID mapping to the identifiers of the application traffic. The SEALDD client sends Sdd_RegularTransmissionConnection_Establish request to SEALDD server with the SEALDD client ID, the SEALDD flow ID, the SEALDD traffic descriptor of the SEALDD client side (the address/port of the SEALDD client for receiving the downlink SEALDD traffic), VAL server ID, VAL service ID. The request message also contains the selected VAL server endpoint information and UEID. The SEALDD server retrieves the location information of the VAL UE or SEALDD client from SEAL LM services defined in 3GPP TS 23.434[4] clause 9.3.12 and verifies with the Geofence policy configured by the VAL server to allow or restrict the data connection establishment. If the location information is allowed as per the configured geofence policy then the SEALDD server allows the data connection establishment, otherwise SEALDD server returns a failed result e.g. performs connection reject. NOTE 3: The SEALDD server can use or update the association between SEALDD-UU connection and SEALDD-S connection that associated with UE ID, VAL service ID, VAL server endpoint, which is used to correlate the SEALDD traffic and the VAL application traffic. NOTE 4: The SEALDD flow ID is used by the SEALDD client and SEALDD server to identify different VAL application traffic of the same SEALDD client. The SEALDD flow ID may be same with the identifiers of the application traffic or new simplified IDs allocated by SEALDD. 7. The SEALDD server responds to the SEALDD client with the SEALDD traffic descriptor of SEALDD server side (e.g. address/port allocated in step 2, transport layer protocol) mapping to the application traffic. 8. If the connection between VAL server and SEALDD server is not established in step 2, the SEALDD server establishes connection with VAL server for the VAL client to transmit application traffic mapping to the SEALDD traffic according to the SEALDD-S information negotiated in step 1-2. 9. The SEALDD client uses the SEALDD traffic descriptor of SEALDD server side for SEALDD connection establishment. After this step, the SEALDD client and SEALDD server both get the whole SEALDD traffic descriptor (including the UE's address/port and SEALDD server's address/port for the SEALDD traffic transmission). 10. If multiple requested flows are provided in step 1, the SEALDD client repeats steps 6-9 for each requested flow provided in step 1. NOTE 3: The way for the mulple flows configuration between SEALDD client and SEALDD server (i.e., re-use Sdd_RegularTransmissionConnection_Establish procedure or modification of the Sdd_RegularTransmissionConnection_Establish) will be decided in the normative stage. Editor’s Note: How to harmonize the burst related parameters (for PCF input) considering burst policy in DD policy is FFS. After the negotiation and establishment of the connections, the SEALDD client gets the mapping information between application traffic and SEALDD flow ID. The SEALDD server gets the mapping information between the SEALDD flow ID and the SEALDD-S connection. Upon receiving application traffic from VAL client, the SEALDD maps it to SEALDD traffic with SEALDD traffic descriptors as negotiated with SEALDD server in step 6 and step 7. The SEALDD traffic is sent to the SEALDD server. The SEALDD server maps the SEALDD traffic to the application traffic according to the stored SEALDD traffic descriptor, SEALDD client ID and SEALDD flow ID. The SEALDD server sends the recovered application traffic to the address provided by VAL server in step 1, via the connection established in step 2 or 8 according to the mapping information. The downlink application traffic sent from VAL server to VAL client is processed similarly. The SEALDD server receives any UE location change notification using SEAL LM services defined in 3GPP TS 23.434[4] clause 9.3.12, then the SEALDD server performs the data delivery in alignment with the geofence policy. If the UE is in the forbidden location or not allowed for the given VAL service to send/receive data as per the Geofence policy, then the SEALDD server performs action like releases the connection and informs the VAL server that UE is not reachable because in a forbidden location using connection event status procedure. If UE enters the allowed location area then the SEALDD server initiates the connection establishment using the procedure defined in clause 9.2.2.3. #### 9.2.3.1 SEALDD enabled regular transmission request Table 9.2.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting the regular application transmission service. Table 9.2.3.1-1: SEALDD enabled Regular transmission request Information element; Status; Description. VAL server ID; M; Identity of the VAL server. VAL service ID; O; Identity of the VAL service. UE identity; O; Identifier of specific UE, e.g. UE ID. List of requested flows; M; List of the requested multi-modal flows. > SEALDD-S Data transmission connection information; M; Address/port and/or URL of the VAL server to receive the application packets from the SEALDD server. > QoS information; O; QoS information provided by VAL server. > VAL server’s total bandwidth limit; O (See NOTE); The total bandwidth limit of VAL server, including UL/DL. > VAL users’ bandwidth limit; O (See NOTE); The bandwidth limits (i.e. minimum bandwidth requirement and maximum bandwidth limit) for VAL users, including UL/DL. NOTE: These IEs are used for the SEALDD enabled bandwidth control for different VAL users.. #### 9.2.3.2 SEALDD enabled regular transmission response Table 9.2.3.2-1 describes the information flow from the SEALDD server to the VAL server for responding to the regular application transmission. Table 9.2.3.2-1: SEALDD enabled regular transmission response Information element; Status; Description. Result; M; Success or failure.. List of SEALDD-S information Data transmission connection information; O; List of Address/port and/or URL of the SEALDD server to receive the packets from the VAL server for application traffic transfer (NOTE 2). Cause; O See NOTE; Indicates the reason for the failure, e.g. SEALDD policy mismatch.. NOTE 1: The IE is only present if the Result is failure. NOTE 2: The order of elements in this list matches the order of elements in the List of requested flows IE defined in table 9.2.3.1-1.

### 7.x.3	Solution evaluation

This solution addresses KI#2 for the following aspects:

-	Whether and how to support the interaction between the application enablement layer and 5G CN to manage E2E multi-modal communication flows between application clients and application servers?

-	Whether and how SEALDD may be enhanced to assist in managing E2E multi-modal communication flows between application clients and application servers involved in the same application service (e.g., support for multi-modal aware SEALDD flow management and policies)?

# *** End of Changes ***