3GPP TSG-SA WG6 Meeting #60	S6-241512

Changsha, China, 15th – 19th April 2024	was S6-241038

Source:	TNO, KPN N.V.

Title:	Solution #12 Update

Spec:	3GPP TS 23.700-82 V0.2.0

Agenda item:	8.3

Document for:	Approval

Contact:	Nassima Toumi, nassima.toumi@tno.nl

Yonatan Shiferaw, yonatan.shiferaw@tno.nl

1. Introduction

This contribution updates Solution #12 by completing the API information flows for AI/ML model performance degradation detection, and AI/ML model update.

2. Reason for Change

This pCR defines the corresponding APIs for supporting the proposed solution.

3. Proposal

It is proposed to agree the following changes to 3GPP 23.700-82 V0.2.0.

* * * First Change * * * *

### 8.12.1	Solution description

This solution addresses Key Issues #1 and #6. The solution provides a procedure to support AI/ML model lifecycle management for ML model re-training and update when model performance degradation is observed by AI/ML Enablement. The solution also supports using an existing model to re-train the model using Transfer Learning. Additionally, if the degraded model is related to other models due to e.g., Transfer Learning, the AI/ML Enablement may trigger the update of those related models as well.

Figure 8.12.1-1 depicts the procedure where the AI/ML Enablement capability can trigger model update upon detecting model performance degradation.

Pre-conditions:

1.	The AI/ML Enablement Server has provided a ML model to the AI/ML Enablement Consumer.

Figure 8.12.1-1: Support for AI/ML model lifecycle management

0.	The AI/ML Enablement Consumer detects a performance degradation of the AI/ML model.

.

Editor's Note: How the AI/ML Enablement Consumer detects the performance degradation is FFS

1.  The AI/ML Enablement Consumer sends  a AI/ML model update request to the AI/ML Enablement Server that includes the ID of the model

2.	The AI/ML Enablement Server determines whether an update of the AI/ML model is required.

3.	The AI/ML Enablement Server fetches the ML model information from the ML Model and Data Repository. The AI/ML Enablement Server may also perform ML model discovery to determine whether an existing ML model stored by the ML Model and Data Repository can be used to train the new model (e.g., using Transfer Learning).

The AI/ML Enablement Server can also discover models that are related due to Transfer Learning or the use of the same training data, to identify additional models that may require an update.

4.	The AI/ML Enablement Server performs ML model re-training, which corresponds to the ML model training procedure as described in other solutions in the TR.

If the degraded model is linked to other models (e.g., due to Transfer Learning, or the same training data has been used), the AI/ML Enablement Server may trigger the re-training and update of those related models as well.

5.	The AI/ML Enablement Server provides the updated ML model to the AI/ML Enablement Consumer either by sending it directly, or by providing information to retrieve it from the ML Model and Data Repository.

* * * Next Change * * * *

### 8.12.3	Corresponding APIs

Table 8.12.3-1 details the ML model performance degradation notification IEs.

Table 8.12.3-1: AI/ML model update request

| Information element | Status | Description |
| --- | --- | --- |
| AI/ML Enablement Consumer Identity | M | The identity of the AI/ML Enablement Consumer sending the notification. |
| AI/ML model ID | M | Provides the ID of AI/ML model for which the performance degradation has been detected. |
| Performance degradation information | O | Provides details about the detected performance degradation, such as the time, instances, or information on the degraded metrics (e.g. accuracy, recall, F1score). |
| AI/ML model information | O | Information on where the AI/ML is stored, such as the repository endpoint or ID. |

The ML model information retrieval process is similar to the one described in solution 5, where the model ID is provided in the request.

Table 8.12.3-2 details the ML model update IEs.

Table 8.12.3-2: AI/ML model update response

| Information element | Status | Description |
| --- | --- | --- |
| Model update indication | M | Indicates whether the model has been updated |
| AI/ML model ID | M | Provides the ID of the updated AI/ML model |
| AI/ML model | O (NOTE 1) | Provides the updated AI/ML model |
| AI/ML model information | O (NOTE 1) | Provides the information of the ML model and data repository, from which the AI/ML model can be retrieved, such as the repository endpoint or ID. |
| NOTE 1: At least one of these information elements shall be provided. |  |  |

* * * Next Change * * * *

### 8.12.4	Solution evaluation

This solution addresses Key Issues #1 and #6, and details the procedures for AI/ML model lifecycle management to detect performance degradation and trigger AI/ML model update. The solution also supports scenarios of Transfer Learning, by triggering proactive updates to related models if needed. This solution is feasible and doesn’t introduce any dependency to the 3GPP network systems.

* * * End of Changes * * * *