3GPP TSG-SA WG6 Meeting #60	S6-241569

Changsha, China, 15th– 19th April 2024

Source:	Ericsson

Title:	Solution on Enhance AIML Enablement Services for Assisting Edge Computing

Spec:	3GPP TR 23.700-82 V0.3.0

Agenda item:	8.3

Document for:	Approval

Contact:	Jing Yue (jing.yue@ericsson.com)

1. Introduction

As described in Key Issue #1, the study aspects for support of architecture enhancement and functions for application layer AI/ML services include:

- Whether and how to enhance the architecture and the above related functions to support application layer AI/ML services in edge computing scenarios.

An AI/ML task can be seen as a specical computing task, and edge comptuting can be used for completing the AI/ML tasks. A CAS, CES, ECS, EES or EAS may have different roles in different types of edge computing, e.g. hiearchitical computing (with one root node (e.g., CAS, EAS), the root has one or more children which also know as sub-root node(s) (e.g., EES or EAS), and multiple leaf nodes (e.g., EAS) which with no children) as shown in Figure 1-1, distributed computing (with one coordination node (e.g., CAS, CES, or ECS) and multiple execution nodes (e.g., EAS)) as shown in Figure 1-2.

- Figure 1-1: Example of architecture for hiearchitical computing

Consider a AI/ML task in a hiearchitical computing architecture, for example,

- For upward process in a hiearchitical computing architecture (e.g., Hiearchitical Learning), a leaf node (e.g., EAS) needs to delivery its intermediate outputs of AI/ML task to its connected sub-root nodes for furthur computing. Each sub-root node performs computing on the received intermediate outputs from the leaf node and generates new intermediate outputs, these new outputs need to be deliveries to the root node for final computation.

- For downward process in a hiearchitical computing architecture (e.g., Hiearchitial Inference), the computing process is opsize to that in upward process, i.e., root node delivers its intermediate outputs of AI/ML task to its connected sub-root nodes. After generates new intermediate outputs, each sub-root node delivers the new intermediate outputs to its connected leaf nodes for final computaton.

- For a distribution computing architecture (e.g. Horizental Federated Learning), a coordination node (e.g., CAS, CES, or ECS) needs to select proper execution nodes for computing sub-tasks (one AI/ML task is devided into multiple sub-tasks) and distributes the sub-tasks to the selected execution nodes.

- Figure 1-2: Example of architecture for distributed computing

Thus, to assist the opreations of an edge compting process for an AI/ML task, differnt assistance for management and configuration may be required. This solution proposes to enhance the AI/ML Enablement services for assisting edge computing.

2. Reason for Change

To assist the opreations of an edge compting process for an AI/ML task, differnt assistance for management and configuration are required. Study on mechanisms to assist edge computing is needed.

3. Conclusions

This pCR proposes solution on assistance for management and configuration edge computing for AI/ML tasks.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-82 V0.3.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[3]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[4]	3GPP TS 23.436: "Functional architecture and information flows for Application Data Analytics Enablement Service".

[5]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[6]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[7]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[8]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[9]	3GPP TS 28.105: "Management and orchestration; Artificial Intelligence/ Machine Learning (AI/ML) management".

[10]	3GPP TS 22.261: "Service requirements for the 5G system".

[11]	3GPP TR 26.927: “Study on Artificial Intelligence and Machine learning in 5G media services”.

[12]	3GPP TR 22.874: “Study on traffic characteristics and performance requirements for AI/ML model transfer”.

[A]	3GPP TS 23.558: “Architecture for enabling Edge Applications”.

* * * Next Change * * * *

## 8.0	Mapping of solutions to key issues

Table 8.1-1: Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | KI #7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 | X |  |  |  |  |  |  |
| Sol #2 |  |  | X |  |  |  |  |
| Sol #3 |  | X |  |  |  |  |  |
| Sol #4 |  | X |  |  |  |  |  |
| Sol #5 |  | X |  |  |  |  |  |
| Sol #6 |  |  | X |  |  |  |  |
| Sol #7 |  |  | X |  |  |  |  |
| Sol #8 |  |  | X |  |  |  |  |
| Sol #9 |  |  | X |  |  |  |  |
| Sol #10 |  |  |  |  |  |  | X |
| Sol #11 | X |  |  |  |  |  |  |
| Sol #12 | X |  |  |  |  | X |  |
| Sol #13 | X |  | X |  |  |  |  |
| Sol #14 |  |  |  |  |  |  | X |
| Sol #15 |  | X |  |  |  |  |  |
| Sol #16 |  |  | X |  |  |  |  |
| Sol #17 |  |  | X |  |  |  |  |
| Sol #18 |  |  |  | X |  |  |  |
| Sol #19 |  |  |  |  | X |  |  |
| Sol #X | X |  |  |  |  |  |  |

* * * Next Change (all new) * * * *

## Solution #X: Enhance AIML Services for Assisting Edge Computing

### 8.X.1	General

An AI/ML task can be seen as a specical computing task, and edge comptuting can be used for completing the AI/ML tasks. A CAS, CES, ECS, EES or EAS (which are defined in 3GPP TS 23.558 [A]) may have different roles in different types of edge computing, e.g. hiearchitical computing (with one root node (e.g., CAS, EAS), the root has one or more children which also know as sub-root node(s) (e.g., EAS), and multiple leaf nodes (e.g., EES or EAS) which with no children), distributed computing (with one coordination node (e.g., CAS, CES, or ECS) and multiple execution nodes (e.g., EAS)). Here, hierarchical computing represents a computation architecture with multiple compuatation entities involved and multiple levels of computations for a compuaton task.

The following clauses specify procedures, information flows and APIs for KI#1 to enhance AIML Services for Supporting Edge Computing.

Pre-conditions:

-	An AI/ML task be treated as a specical computing task, consumer (e.g. CAS, EAS) uses edge comptuting method for completing the AI/ML tasks.

-	The consumer decides its role in the edge computing architecture (e.g. hierarchical computing, distributed computing) for an AI/ML task, and also the type of edge computing operations, based on its local configuration.

-	The consumer decides that assistance from AI/ML Enablement Server for management and configuration to support the edge computing process is needed.

### 8.X.2	Procedures for assisting edge computing

Figure 8.X.2-1: Procedure for assisting an edge computing process

Figure 8.X.2-1 illustrates the procedure for subscribe/request assistance of edge computing. The corresponding procedure in detail is as follows:

- The Consumer (e.g. CAS, EAS) subscribes/requests to the AI/ML Enablement Server for assistance of an edge computing process. The request includes role of the consumer in an edge computing architecture (e.g. root node, sub-root node or leaf node of a hierarchical computing process, coordination node or execution node of a distributed computing process), type of edge computing operations (e.g. task distribution, intermediate output delivery, execution node selection) to indicate the operations that the assistance information be used for, assistance information type (e.g. time window(s) for computing task distribution and/or intermediate output delivery, candidate execution node list) to indicate the assistance information needed.

- The AI/ML Enablement Server derives information for the request, determines downstream entities and services needed (e.g. time window(s) recommendation for computing task distribution if the consumer is a root node in a hierarchical computing process or a coordination node in a distribution computing process, time window(s) recommendation for intermediate output delivery is the consumer is a leaf node in a hierarchical computing process, candidate execution node list provisioning if the consumer is a coordination node of a distributed computing process).

- The AI/ML Enablement Server performs operations according to the determination in step 2 to generate assistance information. For example, the AI/ML Enablement Server may subscribe/request analytics from ADAE Server (e.g. edge load analytics), request assistance information or configurations from 5GC NFs (e.g. Member UE selection).

- The AI/ML Enablement Server notifies/responds to the consumer with the assistance information for the edge computing process. The consumer (e.g. CAS, EAS) may use the assistance information for decision making on its edge computing operations.

### 8.X.3	Architecture Impacts

There is no impact on architecture in application enabler layer. Based on the existing architecture, the impacts on functionality are the following:

-	AI/ML Enablement Server is introduced to assist edge computing.

-	AI/ML Enablement Consumer (e.g. CAS, EAS) is introduced to consume the enhanced AI/ML Enablement Server services.

### 8.X.4	Corresponding APIs

Editor's note:	This clause provides the corresponding APIs for supporting the solution.

### 8.X.5	Solution evaluation

This solution addresses Key Issue #1 and introduces procedures to assist the opreations of edge compting process for AI/ML task. This solution reuses existing mechinisims of 5GC (e.g. Member UE selection) and ADAE analytics.

NOTE:	The possible impact of this solution to other solutions will be discussed and evaluated during the normative phase.

# *** End of Changes ***