3GPP TSG-SA WG6 Meeting #65	S6-250466

Athens, Greece 17th – 21st February 2025	(revision of S6-250095)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.482 | CR | 0012 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Solve the EN about the split AIML operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_App |  |  |  |  |  | Date: |  |  | 2025-2-8 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Considering the VAL server has requirement to find other processing nodes to achieve better AIML performance, it is proposed to allow VAL server to create,or update, or delete a split operation pipeline to resolve the following EN. Editor’s Note: Whether the procedures to create or update or delete a split operation pipeline can be performed by the VAL server is FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add VAL server as requestor of split operation pipeline discovery procedure and split operation pipeline creation, update and deletion. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VAL server cannot get the split operation pipeline discovery, split operation pipeline creation, update and deletion service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.14.2.2, 8.14.2.3, 8.14.2.6, 8.14.2.7, 8.14.3.2, 8.14.3.4, 8.14.3.19, 8.14.3.21 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.14.2.2	Split operation pipeline discovery

Figure 8.14.2.2-1 illustrates the procedure for an AIMLE client or VAL server to discover instance(s) of split AI/ML operation pipeline or processing nodes from the AIMLE server.

Pre-conditions:

1.	The AIMLE client or VAL server has received information (e.g. URI, IP address) related to the AIMLE Server;

2.	The AIMLE client or VAL server has received security credentials authorizing it to communicate with the AIMLE server;

Figure 8.14.2.2-1: Split operation pipeline discovery

1.	The AIMLE client or VAL server sends split operation pipeline discovery request to the AIMLE server; the request may be based on a VAL client requirement(s) and the split operation request includes information defined in Table 8.14.3.2-1.

2.	Upon receiving the request from the AIMLE client, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server may determine if existing instance(s) of a split operation pipeline satisfy the request parameters. If the AIMLE server determines existing instance(s) of split operation pipeline, the AIMLE server may proceed to step 3.

If no instance of a split operation pipeline satisfies the request parameters, the AIMLE server determines if an instance of a split operation pipeline can be created. The determination for split operation pipeline creation is based on availability of ML models and determination of available processing nodes (e.g., VAL server(s), NWDAF(s), etc.) to execute operations for such ML models (e.g., inference). VAL server nodes register with the AIMLE server to indicate their availability and capabilities as described in clause 8.14.2.4. If the AIMLE server determines that an instance of a split operation pipeline can be created, the AIMLE server creates a split operation profile accordingly as defined in Table 8.14.3.3-2.

3.	The AIMLE server sends a split operation pipeline discovery response to the requestor. If the AIMLE server has determined instance(s) of split operation pipeline (e.g., existing, or new), the response includes an indication of success and includes the corresponding split operation profile(s). Otherwise, the response includes an indication of failure and may include a reason for failure.

Upon receiving a response including split operation profile(s), the AIMLE client or VAL server stores the discovered split operation profile(s). The AIMLE client provides split operation profile information (e.g. endpoints, usage, etc.) to VAL client(s). The VAL client or VAL server may use the provided information to access the split operation pipeline.

If the VAL client or VAL server is a data provider, the VAL client or VAL server may use the endpoint information to send data (e.g., intermediate inference data) towards the split operation pipeline instance’s head endpoint.

If the VAL client or VAL server is a data consumer, the VAL client or VAL server may use the endpoint information to request results or subscribe to results from the split operation pipeline instance’s tail endpoint.

8.14.2.3	Split operation pipeline creation

Figure 8.14.2.3-1 illustrates the procedure for an AIMLE client or VAL server to create an instance of a split operation pipeline at the AIMLE server. If the required number of nodes are not discovered in using the discovery procedure (i.e., the number of nodes in the discovery response message are less than the required minimum number of nodes), then the AIMLE client or VAL server can discover the remaining required processing nodes from other AIMLE server(s).

Pre-conditions:

1.	The AIMLE client or VAL server has received information (e.g. URI, IP address) related to the AIMLE Server;

2.	The AIMLE client or VAL server has received security credentials authorizing it to communicate with the AIMLE server;

3.	The AIMLE client or VAL server has discovered information about processing nodes available with the AIMLE server;

4.	The VAL client (along with the AIMLE client) or VAL server selects the nodes, which can take part in the split operation and decides the order in which nodes should perform the split operation inference.

Figure 8.14.2.3-1: Split operation pipeline creation

1.	The AIMLE client or VAL server sends a split operation pipeline create request to the AIMLE server. The request includes information defined in Table 8.14.3.4-1.

2.	Upon receiving the request from the AIMLE client, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server validates if the requested split operation pipeline can be created based on the information included in the request.

If the requestor is authorized and the AIMLE server has determined that an instance of a split operation pipeline can be created, the AIMLE server creates a split operation profile along with its identifier and notifies the appropriate processing nodes indicated in the request about their inclusion in the split operation pipeline as described in clause 8.14.2.5.

3.	The AIMLE server sends a split operation pipeline create response message to the  requestor. If the AIMLE server has created an instance of a split operation pipeline, the response includes an indication of success, and the corresponding split operation profile. Otherwise, the response includes an indication of failure and may include a reason for failure.

Upon receiving a response including a split operation profile, the AIMLE client or VAL server can store the split operation profile. The AIMLE client provides split operation profile information to VAL client(s). Split operation profile information can be used to access the split operation as described in clause 8.14.2.2.

* * * Next Change * * * *

8.14.2.6	Split operation pipeline update

Figure 8.14.2.6-1 illustrates the procedure for an AIMLE client or VAL server to update an instance of a split operation pipeline at the AIMLE server. The AIMLE client, VAL client, or VAL server can determine that some of the nodes in the split operation pipeline may not be able to perform the required operation and decides to modify the pipeline either to add or remove the nodes.

Pre-conditions:

1.	The AIMLE client or VAL server has received information related to split operation pipeline profile as specified in clause 8.14.2.3.

Figure 8.14.2.6-1: Split operation pipeline update

1.	The AIMLE client or VAL server sends a split operation pipeline update request to the AIMLE server. The request includes information defined in Table 8.14.3.19-1.

2.	Upon receiving the request, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server validates if the requested split operation pipeline can be updated based on the split operation pipeline identifier included in the request.

If the requestor is authorized and a split operation pipeline profile is determined, the AIMLE server updates the split operation pipeline profile and notifies the appropriate processing nodes indicated in the request about their inclusion or exclusion in the split operation pipeline as described in clause 8.14.2.5. The AIMLE server also notifies the already existing nodes about modification of the existing pipeline as described in clause 8.14.2.5.

3.	The AIMLE server sends a split operation pipeline update response message to the requestor. If the AIMLE server has updated an instance of a split operation pipeline profile, the response includes an indication of success, and the corresponding split operation profile. Otherwise, the response includes an indication of failure and may include a reason for failure.

8.14.2.7	Split operation pipeline delete

Figure 8.14.2.7-1 illustrates the procedure for an AIMLE client or VAL server to delete an instance of a split operation pipeline at the AIMLE server. The AIMLE client, VAL client,  or VAL server can determine to delete a split operation pipeline when it is no longer needed.

Pre-conditions:

1.	The AIMLE client or VAL server has received information related to split operation pipeline profile as specified in clause 8.14.2.3.

Figure 8.14.2.7-1: Split operation pipeline delete

1.	The AIMLE client or VAL server sends a split operation pipeline delete request to the AIMLE server. The request includes information defined in Table 8.14.3.21-1.

2.	Upon receiving the request, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server validates if the requested split operation pipeline can be deleted based on the split operation pipeline identifier included in the request.

If the requestor is authorized and a split operation pipeline is determined, the AIMLE server deletes the split operation pipeline profile and notifies the appropriate processing nodes about deletion of the split operation pipeline as described in clause 8.14.2.5.

3.	The AIMLE server sends a split operation pipeline delete response message to the requestor. If the AIMLE server has deleted a split operation profile, the response includes an indication of success. Otherwise, the response includes an indication of failure and may include a reason for failure.

* * * Next Change * * * *

8.14.3.2	Split operation pipeline discovery request

Table 8.14.3.2-1 shows the request sent by a AIMLE client or VAL server to the AIMLE server for split operation pipeline discovery request.

Table 8.14.3.2-1: Split operation pipeline discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identity of the requestor (e.g., VAL client ID, AIMLE client ID, UE identifier, VAL server ID) |
| Security credentials | M | The security credentials of the requestor. |
| Split operation discovery filters | M | Split operation discovery filters. |
| > stage information | M | Information about the split operation stages (e.g., number, order, etc.) |
| > model information | M | Information about the ML models to be used in each stage (e.g., identifiers, versions, etc.) |
| > usage information | O | Information about the planned usage of the split operation (e.g., inputs frequency/size, output frequency/size, etc.) |
| > number of nodes | O | Minimum number of nodes required to support AIML operation splitting |

* * * Next Change * * * *

8.14.3.4	Split operation pipeline create request

Table 8.14.3.4-1 shows the request sent by a AIMLE client or VAL server to the AIMLE server for split operation pipeline create request.

Table 8.14.3.4-1: Split operation pipeline create request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identity of the requestor (e.g., VAL client ID, AIMLE client ID, UE identifier or VAL server ID). |
| Security credentials | M | The security credentials of the requestor. |
| Split operation requirements | M | Split operation requirements. |
| > model information | M | Information about the ML models to be used in each stage (e.g., identifiers, versions, etc.). |
| > usage information | O | Information about the planned usage of the split operation (e.g., inputs frequency/size, output frequency/size, etc.). |
| > notification target | O | Endpoint information where the result of the split operation is sent by the tail node. |
| > stage information | M | Information about the split operation stages (e.g., number, order, etc.). |
| >> head node | M | Endpoint of the head node - for providing initial the inference data. |
| >> tail node | M | Endpoint of the head node - for obtaining inference results. |
| >> node information | M | List of Information about the nodes in order of the stage (e.g., identifier, endpoint, etc.). |

* * * Next Change * * * *

8.14.3.19	Split operation pipeline update request

Table 8.14.3.19-1 shows the request sent by an AIMLE client or VAL server to the AIMLE server for split operation pipeline update request.

Table 8.14.3.19-1: Split operation pipeline update request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identity of the requestor (e.g., VAL client ID, AIMLE client ID, UE identifier or VAL server ID). |
| Security credentials | M | The security credentials of the requestor. |
| Split operation pipeline identifier | M | The identifier of the AI/ML split operation pipeline. |
| Split operation pipeline information | M | Split operation pipeline information to be updated; this may include information elements defined in the split operation profile in Table 8.14.3.3-2. |

* * * Next Change * * * *

8.14.3.21	Split operation pipeline delete request

Table 8.14.3.21-1 shows the request sent by an AIMLE client or VAL server to the AIMLE server for split operation pipeline delete request.

Table 8.14.3.21-1: Split operation pipeline delete request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identity of the requestor (e.g., VAL client ID, AIMLE client ID, UE identifier or VAL server ID). |
| Security credentials | M | The security credentials of the requestor. |
| Split operation pipeline identifier | M | The identifier of the AI/ML split operation pipeline. |

* * * End of Change * * * *