3GPP TSG-SA WG6 Meeting #65	S6-250479

Athens, Greece 17th – 21st February 2025	(revision of S6-250178)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0369 | rev | 1 | Current version: | 19.4.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add the APIs for VAL server obtaning the SCAI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_App |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Add the APIs for VAL server obtaning the SCAI based on the procedure of clause 21.2.2.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the APIs for VAL server obtaning the SCAI and modify the title for the clause 21.2.2.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | APIs for VAL server obtaning the SCAI will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New {21.2.x,21.2.x.1, 21.2.x.2, 21.2.x.2.1, 21.2.x.2.2}, 21.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

21.2.x	APIs for Satellite coverage availability information

21.2.x.1	General

Table 21.2.x.1.1-1 illustrates the APIs for satellite coverage availability information.

Table 21.2.x.1-1: List of APIs for satellite coverage availability information

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_SCAIInfoRetrieval | Obtain_SCAI_Info | VAL server | Request/Response |

21.2.x.2	SS_SCAIInfoRetrieval API

21.2.x.2.1	General

API description: This API enables the VAL server to obtain the satellite coverage availability information (SCAI) from the configuration management server.

21.2.x.2.2	Obtain_SCAI_Info operation

API operation name: Obtain_SCAI_Info

Description: Request the satellite coverage availability information.

Known Consumers: VAL server.

Inputs: Refer subclause 21.2.3.3

Outputs: Refer subclause 21.2.3.4

See subclause 21.22.2.2 for the details of usage of this API operation.

* * * Next Change * * * *

21.2.2.2	Procedures for obtaining the satellite coverage availability information (SCAI)

The procedure for VAL UE/VAL server requesting and obtaining the satellite coverage availability information from the application enabler is illustrated in figure 21.2.2.2-1.

Pre-conditions:

-	The VAL UE is a consumer and has registered to the 3GPP network via satellite access as defined in clause 5.3.2.1 of 3GPP TS 23.401 [9] or clause 4.2.2.2.2 of 3GPP TS 23.502 [11] and has not received the satellite coverage availability information from the network yet.

-	The configuration management server has obtained the satellite coverage availability information as described in clause 21.2.2.1.

Figure 21.2.2.2.1-1: Obtaning the satellite coverage availability information

1.	The consumer (Configuration management client, VAL server) sends Get satellite coverage availability information (SCAI) request to the configuration management server to require satellite coverage availability information when it is using the satellite access. The request may contain the dedicated satellite ID for the requested UE, the current UE location data, etc.

2.	The configuration management server authorizes the request. If the request is granted, it checks the requested SCAI is available or not. If it’s not available, the configuration management server obtains the SCAI as described in step 1 of clause 21.2.2.1.

NOTE:	If the dedicated satellite ID for the requested UE is included in the request, the configuration management server will only consider the satellite coverage availability information for the dedicated satellite. Otherwise, the configuration management server will consider all of satellites which can serve the requested UE.

3.	The configuration management server replies the satellite coverage availability information to the consumer (VAL UE/configuration management client, VAL server) to guide them how to minimize the service impact (e.g. indicate when to trigger the UL/DL service flow if the VAL UE is accessing the satellite). And the VAL UE may retrieve the UE unavailability period (e.g., unavailability period duration or the start time for the unavailability period) as defined in clause 5.4.1.4 of 3GPP TS 23.501 [10] based on the received satellite coverage availability information.

* * * End of Changes * * * *