3GPP TSG-SA WG6 Meeting #65	S6-250514

Athens, Greece 17th – 21st February 2025	(revision of S6-250492)

Source:	Huawei, Hisilicon

Title:	Pseudo-CR on deployment of SEAL services involving SEAL Client(s)

Spec:	3GPP TR 23.700-35 v0.2.0

Agenda item:	8.9

Document for:	Approval

Contact:	Yanmei Yang yangyanmei@huawei.com

- Introduction

This contribution is to propose to provide the solution for the Gap of Incomplete deployment model of  SEAL service.

2. Reason for Change

The existing deployment model in the specification 23.434 is not completed to cover all the SEAL layer entities. SEAL client is one of important SEAL layer entities, the deployment of SEAL client should be captured.

This contribution is proposed to add the related descirption.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-35 v0.2.0.

* * * First Change * * * *

8.X	Solution # X: SEAL deployment involving SEAL Client(s)

8.x.1	Solution description

8.x.1.1 General description

TS 23.434 only specifies the deployment alternatives of SEAL server. This solution provides addtionaly the description of potential SEAL Client(s) deployment alternatives.

SEAL server service provider may provide SEAL Client(s) as terminal SDK on top of terminal OS. Or SEAL Client(s) may be implemented by UE vendor domain (natively supported by OS or as a terminal SDK running over terminal OS).

8.x.1.2 Deployment of SEAL Client(s) in the same provider domain of SEAL server(s)

Figure 8.x.1.1-1 illustrates deployment of the SEAL client(s) in the same domain of SEAL server(s). In this case, SEAL server(s) can be deployed in any of the domain described in caluse 8.2.1 or 8.2.2, or 8.2.3 of TS 23.434.

Figure 8.x.1.2-1: SEAL client(s) deployed in the same domain of SEAL server

8.x.1.3 Deployment of SEAL Client(s) in UE vendor domain

Figure 8.x.1.3-1 and Figure 8.x.1.3-2  illustrates deployment of the SEAL client(s) in UE Vendor domain, which is different with the domain of SEAL server. In this case, SEAL server(s) can be deployed in any of the domain described in caluse 8.2.1 or 8.2.2, or 8.2.3 of TS 23.434.

Figure 8.x.1.2-1: SEAL client(s) deployed in UE vendor domain (within OS)

Figure 8.x.1.3-2: SEAL client(s) deployed UE vendor domain (outside OS)

8.x.2	Evaluation

* * * Next Change * * * *

* * * End of  Change * * * *