3GPP TSG-SA WG6 Meeting #65	S6-250545

Athens, Greece, 17th – 21st February 2025	(revision of S6-250474)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0260 | rev | 2 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of the ROF and Authorization Function responsibilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The definition of the CAPIF Authorization Function and the Resource Owner Function are incomplete and have ambiguities, unclear understanding of the scope of their responsibilities. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifications of the Authorization function responsibilities, consistency alignment. Clarification of the ROF, which is sometimes perceived as just a front end that helps the user provide consent, and sometimes is a back-end with more functionality. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete solution that does not work as specified, ambiguity of the roles and responsibilities of Authorization function and the ROF |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.31, 8.31.1, 8.31.2, 8.31.3, 8.33, 8.33.1, 8.33.2, 8.33.3, 8.34, 8.34.1, 8.34.2, 8.34.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 8.31	API invoker obtaining authorization from resource owner

### 8.31.1	General

CAPIF may authorize the API invoker to invoke the service API based on the authorization information from the resource owner given before the API invocation.

Clause 8.31.3 shows the procedure for obtaining the authorization information.

### 8.31.2	Information flows

NOTE:	The security aspects of this procedure are specified in TS 33.122 [12].

### 8.31.3	Procedure

Figure 8.31.3-1 illustrates the procedure for API invoker obtaining authorization from resource owner.

Pre-conditions:

1.	The resource owner function (ROF) can communicate with the CCF (Authorization function).

2.	Access to an API exposing function offered service API requires obtaining authorization from a resource owner (RO).

Figure 8.31.3-1: Procedure for API invoker obtaining authorization from resource owner

1.	The API invoker requests to obtain authorization information to invoke the service API exposed by the API exposing function (AEF) and access information owned by the resource owner at the AEF through the invocation by sending an obtain service API authorization request to the CCF.

2.	The CCF perfoms the authentication of the API invoker (using authentication information). Then the Authorization function determines the authorization by checking the authorization information provided by the RO via the ROF. The request contains API invoker information (its identity and optionally application service information such as application service provider and application identifier), the purpose for data processing and any information required for authentication of the API invoker. The request may include resource owner-related finer level service API access requirements (e.g., access per service API operation or access per API service resource) and resource owner-related information the API invoker intends to provide to the AEF at service invocation (i.e., service API data model information elements the API invoker seeks authorization to provide to the AEF).

NOTE 1:	The detailed procedure to obtain the RO's authorization information is specified in TS 33.122 [12].

NOTE 2:	Step 1 can occur when the API invoker receives a failure in Service API invocation response indicating that authorization information from resource owner(s) is required.

Editor's Note:	Authorization for finer level service API access and information intended to be provided by the API invoker at invocation, whilst accounting for the provided purpose for data processing, is to be detailed by SA3.

3.	Based on the RO authorization information obtained via the ROF, the CCF sends an obtain service API authorization response to the API invoker.

4.	The API invoker sends service API invocation request to the API exposing function with the authorization information received from CCF (Authorization function) in step 3.

5.	The API invoker receives the service API invocation response resulting from the service API invocation once the API exposing function has checked whether the API invoker is authorized to invoke that service API based on the authorization information received from CCF (Authorization function).

* * * Next Change * * * *

## 8.33	Obtaining collective authorization from a resource owner

### 8.33.1	General

Using the procedure described in clause 8.31 the CAPIF may authorize an API invoker to invoke a service API based on the authorization information from the resource owner given before the API invocation. This clause describes an extension to that procedure to enable the CAPIF to obtain authorization information from the resource owner relating to multiple API invokers and service APIs in a single interaction.

Clause 8.33.3 shows the procedure for obtaining collective authorization information from a resource owner via their resource owner function.

### 8.33.2	Information flows

Editor's note:	The security aspects of this procedure are to be specified in TS 33.122 [12].

### 8.33.3	Procedure

Figure 8.33.3-1 illustrates the procedure for API invokers obtaining collective authorization from resource owner.

The mechanism to obtain collective authorization from a resource owner is supported by the CAPIF core function.

Pre-conditions:

1.	The resource owner function can communicate with the CCF (Authorization function).

2.	Access to an API exposing function offered service API requires obtaining authorization from a resource owner.

3. 	The API invoker is successfully onboarded as described in clause 8.1.

4.	The CCF is configured for handling collective authorization, see table 8.33.3-1.

Figure 8.33.3-1: Procedure for API invoker obtaining collective authorization from resource owner

1.	An API invoker sends obtain service API authorization request to the CAPIF core function for obtaining permission to access a service API by including its API invoker identity information and any information required for authentication of the API invoker. The API invoker includes in the request information about the purpose and the targeted resources and operations to be performed on those resources.

2.	The CCF performs the authentication of the API invoker (using authentication information) and checks whether it is permitted to access the requested service API by checking whether a resource owner authorization information already available for that API invoker. If the authorization for accessing the requested resources for the signalled purpose is already granted, the procedure continues with step 5.

3. 	If authorization information is not available regarding whether the API invoker is permitted to access the requested service API (i.e., there is no applicable resource owner authorization information available) the collective resource owner authorization configuration (table 8.33.3-1) is assessed.

If authorization trigger criteria have been specified as part of the collective resource owner authorization configuration, interactions with the resource owner are only initiated once the trigger conditions are met.

If trigger criteria have not been set or once the trigger conditions are met, the authorization function determines the authorization by interacting with the resource owner via the ROF deployed on a UE. Based on the configured collective resource owner authorization configuration at the CCF and any cached “obtain service API authorization requests”, the scope of the request towards the ROF can be expanded to include additional API invokers and or service APIs based on the assumption that there is an association between the requesting API invoker and the API exposing functions and API invokers listed in the CCF configuration.

Editor's Note: The detailed procedure to obtain the resource owner's authorization information for multiple API invokers in one request require further investigation and is in scope of SA3. However, it is recommended that the resource owner is provided with at least the option to only grant authorization for only the API invoker initiating the authorization request and then only for the specific service being requested.

NOTE 2:	The details of the interaction between RO and ROF for capturing the resource owner authorization are considered out-of-scope of the present specification.

4.	Based on the RO authorization information obtained via the ROF, the CCF sends an API invoker specific service API authorization response to the API invoker that initially triggered the collective resource owner authorization information requested and to each API invoker for which a obtain service API authorization request was received (for the same service API) whilst the collective resource owner authorization configuration duration timer was running, i.e., the cached requests.

5.	An API invoker that receives the service API authorization response can send service API invocation requests to the API exposing function with their API invoker specific resource owner authorization information.

6.	The API exposing function will send a service API invocation response once it has successfully checked that the API invoker is authorized to invoke that service API based on the authorization information.

Table 8.33.3-1: Collective resource owner authorization configuration

| Information element | Status | Description |
| --- | --- | --- |
| List of API invoker associations (NOTE 1) | O | Lists of API invokers with their associated service APIs |
| > API invoker identity information list | M | List of API invoker identities |
| > Service API identification list | M | List of AEF service API identities. |
| List of trigger criteria (NOTE 1) | O | Criteria for triggering requesting of resource owner authorization |
| > Duration (NOTE 2) | O | Initiate request towards RO via ROF after the given duration (in seconds) after receiving a obtain service authorization request from an API invoker |
| > Timeperiod (NOTE 2) | O | Time period during which notifications may be sent towards RO via ROF. |
| > Resource owner identifier | O | Can only be included if a duration or timeslot is included. Indicates that the resource owner authorization trigger criteria is to be applied to the specific resource owner identified by their GPSI, i.e., no other resource owner authorization trigger criteria shall be applied for that resource owner. |
| NOTE 1: At least one of these attributes shall be present NOTE 2: Only one of these attributes shall be present |  |  |

* * * Next Change * * * *

## 8.34	UE-deployed API invoker accessing other UEs’ resources of a group

### 8.34.1	General

This clause specifies the procedures for API invoker on a UE to be able to access the services exposing resources of other UEs of a group. The UEs belong to the same group and the same VAL service provider (IoT case) or the same user.

### 8.34.2	Information flows

Editor’s note:	The security aspects of this procedure are to be specified by SA3.

### 8.34.3		Procedure

Figure 8.34.3-1 presents the procedure for enabling a UE-hosted API invoker accessing network-hosted resources owned by other UEs.

Pre-condition:

1.	The network hosts individual resources for a group of UEs which can be accessed via AEF provided the necessary security conditions are met.

2.	UE 1 and UE 2 belong to the same group and the same VAL service provider (IoT case) or the same user.

3.	CCF is provisioned with the information of a Group Resource Owner (GRO) which indicates UE 1 as the authorized RO (e.g. by business relationship between the group of UEs) to provide resource owner authorization for the the resources belonging to the UEs in this group.

Figure 8.34.3-1: UE-deployed API invoker accessing other UEs’ resources

1.	The API invoker (e.g., in UE 2) sends an Obtain service API authorization request to the CCF for obtaining permission to access the service API (and other UEs’ resources hosted in the network – e.g., location) by including the API invoker identity information and any information required for authentication of the API invoker. The request includes the UE identifier of UE 2, the group identifier, and optionally the list of UEs in a group whose resources are to be accessed. Additionally, the API invoker may include in the request information about the purpose and operations (scope) to be performed on the targeted resources.

2.	CCF performs authentication of the API invoker (using authentication information) as specified in 3GPP TS 33.122 [12].

3.	CCF checks if UE 2 belongs to the group and further resolves the identity of the GRO responsible for the group of UEs based on the provisioned information related to the GRO.

4.	If step 3 is successful, the CCF performs the resource owner authorization check using the GRO as the RO for the requested resources of other UE(s) belonging to the group.

Editor’s note:	The authorization details for step 4 of how CCF interacts with GRO to obtain authorization which also includes the group identification is FFS and will be specified by SA3.

5.	Based on the resource owner authorization received in step 4, CCF sends an Obtain service API authorization response to the API invoker.

6.	The service API invocation is performed by validating the authorization information received in the previous step.

Editor’s note:	How the AEF performs validation of the received authorization information is FFS and will be specified by SA3.

* * * END of Change * * * *