3GPP TSG-SA WG6 Meeting #65	S6-250596

Athens, Greece 17th – 21st February 2025	(revision of 250161,250529, 250591)

3GPP TSG-SA Meeting #105	SP-241375

Melbourne, Australia, 10th – 13th September 2024	was SP-241062, 241208

3GPP TSG-SA WG6 Meeting #62	S6-243742

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243437)

# Source:	Huawei

# Title:	Revised study on Service Enabler Architecture Layer (SEAL) Phase 4

# Document for:	Approval

# Agenda Item:	11

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Service Enabler Architecture Layer (SEAL) Phase 4

Acronym:	FS_SEAL_Ph4

Unique identifier:	1050034

Potential target Release:	Rel-20

1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  |  |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| SEAL_Ph3 | SA6 | 980131 | Service enabler architecture layer for vertical phase 3 |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 1010006 | SEAL DD (Data Delivery) Phase 2 | Rel-19 SEALDD work |
| 1040075 | Application enablement for AI/ML services | SEAL services to support AI/ML |
| 1040074 | Application enablement for XRM Services Phase 2 | Application enablement for XRM Services |
| 1040077 | Application enablement for satellite access Phase 3 | SEAL enhancement for XR |
| 1040076 | Application enablement for mobile metaverse services | SEAL enhancement for mobile metaverse service |
| 1040072 | Enhanced application layer support for location services | location services enhancement WID |

3	Justification

Network capabilities exposure is one of important topics of 3GPP. Lots of network APIs were defined by SA2, SA4, and SA5 from 4G phase and beyond. In the specifications defined by these WGs, those APIs can be categorized into two types: one is the NF APIs exposed by Network functions like PCF, AMF, and MB-SMF…., and the other is the SCEF/NEF APIs exposed by network exposure function (See figure1). The NF APIs are defined in a network centric way i.e., one API can serve many use cases to simplify the implementation and save cost of Network Function. Most of the NEF APIs follow the same pattern of NF APIs, i.e., the inputs and outputs are the same as the NF APIs).

Network Function (e.g 5GC NFs)

Trusted AF

Untrusted AF

NEF/SCEF/EGMF

NEF APIs

NF APIs

NF APIs

Figure 1 SA2 defined Network exposure architecture

SEAL server

Provided by 3rd party

Network Function（e.g NFs）

SEAL server

Provided by MNO

Trusted domain+NEF

NEF/SCEF/EGMF

Untrusted AF

SEAL service APIs

NEF APIs

NF APIs

NF APIs

Trusted AF

SEAL service APIs

5GC

Figure 2   Core Network APIs abstraction by SEAL

Since Rel16, SA6 has started the works on SEAL to ease the usage of 5G system capabilities by APP developers. As figure 2 shows, how SEAL provides service Layer services aimed at providing developer-friendly APIs for APP domain to use the services exposed by network layer entities like SCEF/NEF or by PCF, AMF, MB-SMF and other Network entities (to authorized AFs). These service Layer services provide value-add functionality including abstraction, optimization, aggregation, etc. for both MNOs and 3rd parties.

The service API exposed by SEAL can be based on MNO Network capabilities/resource (e.g., Reserve_Network_Resource API exposed by NRM use only 5G QoS capability). There are also SEAL services which combine terminal capabilities/resource and MNO network capabilities/resource (e.g Subscribe_Location_Info API exposed by LMS is the combination of Core network location capabilities and Location Management client reported Location information).

Industry promotion organizations (e.g., CAMARA/GSMA Open Gateway) start to investigate how to simplify network APIs and transform them to be more consumable by the APP developers. In addition, it is also an important topic in 5GACIA about how vertical applications can well use network layer exposed capabilities. However, considering gaps with industry requirements there are some aspects that needs to be enhanced from SA6 perspective.

a)	Lack of sufficient description explaining the relationship of different SEALs services, lack of Usage instructions for each APIs (e.g. Valued use case and value added, Parameters mapping between 3GPP parameter and APP domain‘s parameters, APP impacted).

b)	Required parameters for some APIs do not fully consider the information/parameters availability of application layer, but reusing application layer existed information/parameters as possible as it can is very important to APP development. SA6 can further enhance API these aspects. For example, Location retrieval API from OPG/OPAG specifies that the identifier of the target UE could be IPV4 address or IPV6 address or phone number or network access identifier (GPSI). Location retrieval API defined by SA6 accepts the identifier of the target UE as VAL UE ID or VAL user ID which could be GPSI. Other input formats required by the GSMA open gateway Location retrieval API is not supported, but can be added. Another example is, Quality on Demand (QoD) API from GSMA OPG/OPAG provides the customer with the ability to set quality for a flow within an access network connections and to get the notifications if the network cannot fulfil. Resource request and Resource Modification request APIs as part of Network Resource Management SEAL service offers the same service required by GSMA but it is lacking the functionality of identifying a particular application session to be guaranteed with a requested QoS attributes.

c)	Some SEAL services, may be further improved by absorbing other network services to provide a larger granularity services aligning to industry requirements.

d)	When SEAL service servers are deployed by MNO, they can invoke NF API from network functions without NEF, but this option is to be well captured for several services (e.g.,  Location management, NRM, ADAES, NSCALE etc.). This may avoid the misunderstanding of the different business deployment options available for SEAL services and the potential value-add they offer.

e)	SEAL is a layer which is expected to be deployed by either the MNO or an edge/cloud provider as a set of aPaaS capabilities in the cloud platform. In such deployments, there is a need to support cloud native applications as possible consumers of SEAL services, and this may require the enhancement or “upgrade” of existing SEAL architecture as well as nomenclature to be in-line with the new requirement. When interacting with ASPs/ IT domain, the evolution of SEAL architecture towards micro-service based architecture (MSA) or even server-less architecture is necessary in order to support cloud native deployments. For supporting such evolution, the proper definition of SEAL services as well as new challenges arising based on different service-based deployment considerations (e.g. service bus vs mesh, gRPC vs REST API design considerations, use of sidecars/proxies etc) need to be investigated thoroughly.

Below are some of the APIs defined in GSMA/CAMARA that could adopt SEAL services for the realization of service APIs:

-	Quality on Demand (QoD) (boost)

-	Geofencing

-	Location retrieval

-	Device Location verification

4	Objective

Objectives of this WID include the following：

1.	Investigate how to enhance the content of existing SEAL technical specifications based on the following aspects:

-	Perform an overall investigation on how to design easy-to-use API services and providing enhancements to the existing API service design guidelines in TS 23.222.

- 	Evaluate technical enhancements to SEAL API services from easy-to-use perspectives, if any gaps exist

NOTE:  New Release-20 SEAL capabilities and API services will not be covered by this SID.

2.	Review each SEAL services procedures and clarify the interaction with the core network considering the deployment option where SEAL service server is deployed by PLMN operator if it is missing.

3.	Review the current adoption of the 3GPP APIs by other SDOs to understand the transformation being made and make the SEAL services APIs consumable directly without the need of transformation.

4.	Investigate possible enhancements to SEAL architecture towards to support cloud native applications.

5	Expected Output and Time scale

{If this WID covers both stage 2 and stage 3, clearly indicate the different completion dates.}

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 23.700-35 | Study on Service Enabler Architecture Layer (SEAL) Phase4 | TSG#109 (Sep-25) | TSG#110 (Dec-25) | Yang, Yanmei |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

6	Work item Rapporteur(s)

Yang, Yanmei, Huawei Technologies Co. Ltd. <yangyanmei@huawei.com>

7	Work item leadership

SA6

8	Aspects that involve other WGs

CT3 for stage3 implementation.

9	Supporting Individual Members

Supporting IM name

Huawei Technologies Co Ltd

Hisilicon

Samsung

CMCC

CATT

China Telecom

Convida Wireless

Lenovo

ZTE