3GPP TSG-SA6 Meeting #66	S6-251061

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.437 | CR | 0023 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removing ENs in Spatial map subscription |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_App |  |  |  |  |  | Date: |  |  | 2025-03-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Following ENs needs to be removed Editor's Note: Objects are limited to UEs and objects other than UE is FFS. No need to limit object types to UE only for spatial mapping service. In this release, objects are not considered for localization. Editor's Note: Details about "layers" event is FFS. Editor's Note: Details on "Changed object information" is FFS. Existing text seems sufficient for current release. Additional details, if required, can be added in the next release. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing ENs in Spatial map subscription |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ENs will remain present in the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.3.6.2.1	Subscribe spatial map event

Figure 9.3.6.2.1-1 depicts the procedure for the authorized spatial map consumer (VAL server or SEAL SM client) to subscribe the spatial map event. The service is provided by SEAL SM server. Spatial map consumer can get notification information when the the SM server detects subscribed event.

Figure 9.3.6.2.1-1: Subscribe spatial map event procedure

1)	The VAL server (or SEAL SM client) sends a request message to the SEAL SM server to subscribe spatial map event. The request includes requestor ID, security credentials, a notification endpoint, a spatial map ID, event to subscribe which triggers the notification. The request message may also include a list of spatial map layers with its corresponding layer IDs to subscribe spatial map layering information related events. The requestor (VAL server or SEAL SM client) may subscribe to one or more of the following events:

a)	"changes of objects" events to indicate the SM server to notify the requestor when changes of the objects are detected. The notification triggering criteria for "changes of objects" event may include added or removed objects, position or direction changes of the spatial map objects.

b)	"layers modification" events to indicate the SM server to notify the requestor when any layer in the spatial map is added or updated or removed.

c)	"spatial map ready" events to indicate the SM server to notify the requestor when the SEAL SM server has sufficient information to map all the objects related to area of interest the spatial map and the spatial map is ready to provide service (e.g. employed for localization). This event is used when the result IE in the Create spatial map response indicates "in-progress".

2)	The SEAL SM server authorizes VAL server (or SEAL SM client), and validates the request. Then server add the requestor into the subscriber list of the event. SM server starts monitoring the events.

3)	The SEAL SM server sends response message with the result. For the successful subscribe request, a subscription ID is included in the message. Otherwise, the response message includes an indication of failure and may include a reason for failure.

* * * Next Change * * * *

9.3.6.3	Information flows

9.3.6.3.1	Subscribe spatial map request

Table 9.3.6.3.1-1 shows the subscribe spatial map event request sent by VAL server (or SEAL SM client) to SEAL SM server to subscribe for spatial map event(s).

Table 9.3.6.3.1-1: Subscribe spatial map event request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or VAL user) |
| Security credentials | M | The security credentials of the requestor. |
| Application service ID | M | ID of the requesting VAL application service |
| Expiry time | O | Proposed subscription expiry time |
| Target spatial map(s) | M | Spatial map(s) to subscribe |
| > List of spatial map IDs | O | ID(s) of the spatial map(s) to subscribe |
| > Area of interest | O | Three-dimensional area information to discover spatial map(s) to subscribe |
| List of events | M | Events to subscribe. Each element includes the information described below. |
| > Event type | O | List of events to subscribed. Possible values are: - Change of objects - Layers modification - spatial map ready |
| > specified objects | O | List of specified objects for "change of objects" event |
| > Triggering criteria | O | Specifies the condition to determine whether the event is occurred, e.g. Spatial map is employed and ready for localization service. |
| >> Changes in objects | O (NOTE) | Triggers notification if any or specified (or specified type of) objects are added, removed, etc. |
| >> Layer IDs | O (NOTE) | List of layers. Triggers notification if any or specified spatial map layer change is detected |
| > Notification intervals | O | Specifies when to notify the detected event, Possible values are: - Event based (e.g., promptly); Or - Periodically (with minimum time between consecutive notifications) - intervals (i.e. minimum time between consecutive notifications) |
| > interval time | O | Time between consecutive notifications (applicable only if notification intervals is set to intervals. |
| NOTE: At least one of the sub-elements shall be provided. |  |  |

9.3.6.3.2	Subscribe spatial map response

Table 9.3.6.3.2-1 shows the subscribe spatial map event response sent by SEAL SM server to VAL server (or SEAL SM client) as the response to the subscribe spatial map event request.

Table 9.3.6.3.2-1: Subscribe spatial map event response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result of the subscribe spatial map event request |
| Subscription Id (NOTE) | O | Identity of the subscription |
| Expire time | O | Subscription expire time |
| Failure cause | O | Indicates reason for the failure. |
| NOTE: This IE is mandatory when Result IE indicates success. |  |  |

9.3.6.3.3	Notify spatial map event

Table 9.3.6.3.3-1 shows the spatial map notification sent by SEAL SM server to VAL server (or SEAL SM client) to notify about spatial map event(s).

Table 9.3.6.3.3-1: Spatial map event notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription ID for the event |
| Notification information | M | Specifies what matches the triggering notification condition, including related information |
| > Spatial map ID | M | ID of the spatial map for which the event is detected |
| > Event type | M | Type of event detected. One of the event as specified in Table 9.3.6.3.1-1. |
| >> Changed object | O (NOTE) | Information on the object which triggered the notification |
| >>> Object ID | O | ID of the changed object |
| >>> Type of change | O | Specifies the detected type of change for the object (e.g., added, removed, or changed position or direction) |
| >>> Changed object information | O | A container to carry the changed object specific information |
| >> list of layers (NOTE 2) | O | Information about layers for the spatial map. |
| NOTE 1: This IE is applicable for "Change of objects" event type. NOTE 2: This IE is applicable for "Layer modification" event type. |  |  |

* * * End of Change * * * *