3GPP TSG-SA WG6 Meeting #66	S6-251161

Gothenburg, Sweden 7th – 11th April 2025	(revision of S6-251xxx)

Source:	Lenovo

Title:	Key Issue on monitoring ML model/analytics correctness

Spec:	3GPP TR 23.700-83 v0.0.0

Agenda item:	9.4

Document for:	Approval

Contact:	Emmanouil Pateromichelakis <epateromiche@lenovo.com>

1. Introduction

This paper proposes a new key issue on improving ML model or application layer analytics correctness.

2. Reason for Change

Correctness of predictions is usually associated to accuracy, which represents the most prominent KPI to rate ML models. However, the accuracy can be corrupted by a wrong calculation and/or unfair feedback collection. It is thus of utmost importance to ensure that the accuracy is meaningful, consistent, and comparable.

Incorrect predictions can be due to the fact that the accuracy of an ML model during inference may be lower than the accuracy of the same ML model during training. This is likely to happen if the training data set differs significantly in terms of distribution and features from the input data that the ML model is fed with during inference. Knowing how the ML model is meant to be used would help to train the ML model efficiently and effectively. In such cases, a possible drift may be due to an issue of the data source, and such issue may not be in the control of MNO to examine whether the data source itself (which can be trusted or untrusted) provides correct inference data.

In Rel-19, in AIML_App, a basic capability for performance monitoring activity has been specified (in TS 23.482). In this capability, the AIMLE server based on VAL request provides a capability for monitoring and detecting a degradation related to an ML operation / analytics operation and translating to an ML model performance degradation (expected or predicted) and performing a trigger action to alleviate this issue (new model training or re-training). Such trigger action may be either an adaptation of the AIMLE service, such as training of a new ML model for the AIMLE by the same or a different AIMLE client, or re-training of the ML model.

As continuation of this work, the following aspects need to be studied:

-	Whether and how the enabler layer can support assuring the correctness of ML-enabled operation (e.g. analytics, inference, training)?

-	How to detect and manage possible drift due to data source abnormal / unreliable behavior of application layer data sources which are used at the ML model lifecycle operations?

3. Proposal

It is proposed to agree the new key issue for 3GPP TR 23.700-82 v 0.0.0.

* * * * First Change * * * *

# 5	Key issues

## 5.x	Key issue #x: Key issue on monitoring ML model/analytics correctness

### 5.x.1	Description

Incorrect predictions can be due to the fact that the accuracy of an ML model during inference may be lower than the accuracy of the same ML model during training. This is likely to happen if the training data set differs significantly in terms of distribution and features from the input data that the ML model is fed with during inference. Knowing how the ML model is meant to be used would help to train the ML model efficiently and effectively. In such cases, a possible drift may be due to an issue of the data source, and such issue may not be in the control of MNO to examine whether the data source itself (which can be trusted or untrusted) provides correct inference data.

In Rel-19, in AIML_App, a basic capability for performance monitoring activity has been specified (in TS 23.482). In this capability, the AIMLE server based on VAL request provides a capability for monitoring and detecting a degradation related to an ML operation / analytics operation and translating to an ML model performance degradation (expected or predicted) and performing a trigger action to alleviate this issue (new model training or re-training). Such trigger action may be either an adaptation of the AIMLE service, such as training of a new ML model for the AIMLE by the same or a different AIMLE client, or re-training of the ML model.

### 5.x.2	Open Issues

The following open issues need to be studied:

-	Whether and how the enabler layer can support assuring the correctness of ML-enabled operation (e.g. analytics, inference, training)?

-	How to detect and manage possible drift due to data source abnormal / unreliable behavior of application layer data sources or the application ML model which are used at the ML model lifecycle operations?

* * * * End of Change * * * *