3GPP TSG-SA WG6 Meeting #67	S6-252255

Fukuoka, Japan, 19th – 23rd May 2025	(revision of S6-252xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0311 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to events description and AEF instantiation figure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Currently the table 8.8.6-1 is confusing where, theere are two events – “API invoker status” and “CAPIF-1/1e interaction events” – which both provides onboarded API invokders. Actually the “CAPIF-1/1e interaction events” provides the collective infomration of number of API invokers onboarded or number of open discovered APIs. Added clarification in the description to clarify the same. - In Annex H, the title of the Figure indicates “AEF Instantiation” – which is not correct. The Figure shows possible interaction between API provider functions with management functions for possible AEF instantiation. Corrected the name. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added clarification in Table 8.8.6-1 - Corrected the figure title |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The meaning of the events remains confusing and ambiguous, and the Figure title remains wrong. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.8.6, Annex H |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.8.6	List of CAPIF events

Table 8.8.6-1 provides a non-exhaustive list of CAPIF events.

Table 8.8.6-1: List of CAPIF events

| Events | Events Description |
| --- | --- |
| Availability of service APIs | Availability events of service APIs (e.g. active, inactive) |
| Service API updated | Events related to change in service API information |
| Monitoring service API invocations | Events corresponding to service API invocations |
| API invoker status | Events related to API invoker status in CAPIF (onboarded, offboarded, onboarding criteria not met status along with the criteria information that is not met.) |
| API topology hiding status | Events related to API topology hiding status in CAPIF (created, revoked) |
| System related events | Alarm events providing fault information |
| Performance related events | Events related to system load conditions |
| CAPIF-1/1e interaction related events | Events related to CAPIF-1/1e interactions for a service API (e.g. API invoker onboarding/offboarding/updating API list, number of onboarded API invokers, API invoker discovering APIs, number of open discovered APIs) |

* * * Next Change * * * *

Annex H (informative):
AEF instantiation

As illustrated in figure H-1, CAPIF architecture supports AEFs from multiple API providers (e.g. MNO, 3rd party). The instances of such AEFs are managed by the respective management systems of the API provider (e.g. PLMN management system, 3rd party management system).

The AMF monitors the events reported by the CAPIF core function. These events can be utilized to trigger AEF instantiation by the PLMN management system or 3rd party management system. The AMF can utilize the services of PLMN or 3rd party management systems (e.g. trigger instantiation of corresponding AEFs) via the APIs exposed by the PLMN or 3rd party management systems. It is upto implementation of CAPIF to utilize the services of the PLMN and/or 3rd party management systems for AEF instantiation.

Figure H-1: Interaction with management system for AEF instantiation

* * * End of Change * * * *