3GPP TSG-SA WG6 Meeting #68	S6-253067

Gothenburg, Sweden 25th – 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.437 | CR | 0050 | rev | - | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Spatial map UE to object association |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_Ph2-APP |  |  |  |  |  | Date: |  |  | 2025-08-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Objects in a spatial map can be stationary or mobile. Mobile objects can be trackable. If information can be provided to associate a UE with a mobile object, the SEAL SM server can use location services within the 5GS to assist with tracking the object. UE-object association information can be used by SEAL SM server to perform the association between UE ID and object ID. This provides the ability for more accurate tracking and localization of UEs and objects in a spatial map. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add UE-object association information for SEAL SM server to associate object ID with UE ID - Update spatial map procedures to use UE-object association information to assist with object tracking and localization. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The association of UE ID with object ID is unclear and it is unclear how object tracking will work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.3.1, 7.3.3.x, 9.3.1.2, 9.3.1.3.1, 9.3.4.2, 9.3.4.3.1, 9.3.6.2.1, 9.3.6.2.2, 9.3.6.3.1, 9.3.6.3.3, 9.4.2, 9.4.3.1, 9.4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 7.3.3.1	Spatial map information

Table 7.3.3.1-1 describes information elements for the spatial map information stored in the SM server.

Table 7.3.3.1-1: Spatial map information

| Information element | Status | Description |
| --- | --- | --- |
| Spatial map ID | M | The identifier of the spatial map |
| Spatial map coverage | M | The three-dimensional space coordinates information of the spatial map |
| Allowed entities list | O | Identity of entities (e.g. VAL client, VAL server or VAL service identity) which are permitted to discover and access the spatial map |
| Spatial map media format | O | The media format of the spatial map. |
| List of spatial map layers | M | The list of spatial map layer information. At least one or more spatial map layers should be included. Each element includes the information described below. |
| > Spatial map layer ID | M | The identifier of the corresponding spatial map layer. |
| > Spatial map layer information | O | A container to carry the corresponding spatial map layer specific information |
| >> area | O | The area covered by this layer. This area can be the same or a subset of the Spatial map coverage space. |
| >> associated spatial map | O | The details about a spatial map associated with this layer. For example, if the layer is for a particular floor of the building, the detailed spatial map of that particular floor can be associated with this layer. |
| > List of objects | O | List of object information. Each element contains the information as specified in Table 7.3.3.1-2. |

Table 7.3.3.1-2 describes object information details for the spatial map information specified in the Table 7.3.3.1-1.

Table 7.3.3.1-2: Spatial map object information

| Information element | Status | Description |
| --- | --- | --- |
| Object ID | M | The identifier of the corresponding object |
| Object type | M | The type of object |
| Associated UE ID | O | Identifier of the UE associated with the object. |
| Object mobility information | M | Classifies whether the object is stationary or trackable. |
| > Object position | O | The 3D position of the object in the layer. For stationary object, this IE shall be present. For trackable object, 3D position of it shall be provided while it is being tracked, otherwise no value. |
| > Direction (NOTE) | O | Direction of the trackable object while it’s being tracked, otherwise no value. |
| > Velocity (NOTE) | O | Velocity of the trackable object while it’s being tracked, otherwise no value. |
| Object specific information | O | Object specific attribute information |
| NOTE: These IEs may be present when object is moving or changes it’s location frequently. |  |  |

NOTE 1:	Capability of SM server to distinguish whether object is stationary or trackable is outside the scope of this specification.

NOTE 2:	For the current release, the object specific information is implementation specific and out of scope.

Editor's Note: Spatial map management procedures to request object tracking need to be provided.

* * * Next Change * * * *

#### 7.3.3.x	UE-object association information

Table 7.3.3.x-1 describes information elements for the UE-object association information.

Table 7.3.3.x-1: UE-object association information

| Information element | Status | Description |
| --- | --- | --- |
| Association ID | M | Identifier of the UE-object association. |
| UE ID | M | Identifier of the UE with an associated object. |
| Associated object information | M | Information to specify the associated object or to specify which object can be associated with the UE. |
| > Object ID | O (NOTE) | Identifier of the associated object. |
| > Object type | O (NOTE) | The type of associated object. |
| > List of association conditions | O (NOTE) | The conditions to specify when the object can be associated with a UE (e.g., the object can be associated with a UE when they are co-located for at least a certain period of time). |
| > AIML model | O (NOTE) | AIML model for object recognition to associate with the UE. |
| > AIML model ID | O (NOTE) | The AIML model identifier for object recognition to associate with a UE (e.g., if AIML model distribution is supported). |
| NOTE: At least one of the IEs shall be present. |  |  |

* * * Next Change * * * *

#### 9.3.1.2	Procedure

Figure 9.3.1.2-1 depicts the procedure for creating a spatial map. For the request from VAL server or SEAL SM client, as a spatial map consumer, SEAL SM server creates a spatial map. A spatial map can be structured in layers where each layer signifies a specific aspect of the spatial information (e.g. three dimensional space of area of interest, objects).

Figure 9.3.1.2-1: Create spatial map procedure

1)	The requestor (e.g., VAL server or SEAL SM client) sends a request message to the SEAL SM server to create a spatial map. The request includes requestor ID, security credentials, three-dimensional area of interest, information to be included in the spatial map such as allowed entities list defining which entities are permitted to discover and access the spatial map. The request may include spatial map layering information parameters and may also include augmented layer information that can be requested with the spatial map. The request may include a required media format for the spatial map. The request may include a list of UE-object association information for the SEAL SM server to identify associations between UEs and objects in a spatial map.

NOTE:	SEAL SM server can obtain augmented layer information and can provide the obtained augmented layer information to the requestor in the response message.

2)	The SEAL SM server authorizes the requestor, and validates the request. The SEAL SM server produces a requested spatial map using layering information and processed sensor data, according to the requested spatial map media format. The SEAL SM server does not create a spatial map if the requested spatial map media format is not supported and can create a spatial map based on local configuration if no spatial map media format is included in the request.

a)	If the augmented layer information indicates to include VAL UE information, the SEAL SM server fetches the list of VAL UEs in the area of interest from LM server using clause 9.3.10 of 3GPP TS 23.434 [4] and/or from NEF as specified in 3GPP TS 23.502 [5].

b)	If the augmented layer information indicates to include spatial anchors information within area of interest, the SEAL SM server fetches the list of spatial anchors in the area of interest as specified in clause 8.3.4.

NOTE:	For the current release, the processed sensor data is stored at VAL layer (e.g., in database at VAL layer) and the SEAL SM server gets access to such database to create a spatial map via application specific way which is out of scope.

3)	The SEAL SM server sends response message to the requestor with a indication of success, in-progress or fail. If the requested spatial map has been created sussesfully, the response message includes assigned spatial map ID and information which includes three-dimensional space defined by the spatial map and can include a list of spatial map layers with their corresponding layer ID, objects belong to the layer, etc.. If the response indicates "in-progress", an assigned spatial map ID is included in the message. Further, if the response indicates "in-progress" and the requestor has not provided a notification target address, then the requestor subscribes for spatial map event to receive notification when the spatial map is created. Otherwise, the response includes an indication of failure and may include a reason for the failure and indication of supported spatial map media formats if the failure is related to an unsupported spatial map media format.

4)	If the response indicates "in-progress", the SM server detects when the SEAL SM server has sufficient information to map all the objects related to area of interest and the spatial map is ready to provide service (e.g. employed for localization). If the requestor has provided a notification target address in the request or has subscribed for spatial map events, the SEAL SM server notifies the requestor with the spatial map ready notification message. The notification includes parameters as specifed in Table 9.3.6.3.3-1.

* * * Next Change * * * *

##### 9.3.1.3.1	Create spatial map request

Table 9.3.1.3.1-1 descibes the information elements from VAL server (or SEAL SM client) to SM server for create spatial map request.

Table 9.3.1.3.1-1: Create spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or VAL User or VAL UE) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Application service ID | M | ID of the requesting VAL application service |
| Area of interest | M | Three-dimensional area information to produce the spatial map |
| Notification Target Address | O | Notification target address (e.g. URL, URI, IP) where the notifications should be sent. |
| Spatial map media format | O | The media format requested for the spatial map. |
| Information to include | O | Information to be included in the spatial map |
| > Allowed entities list | O | Identity of entities (e.g. VAL client, VAL server or VAL service identity) which are permitted to discover and access the spatial map |
| > List of spatial map layers | O | List of spatial map layer information. Each element includes the information described below. |
| >> Spatial map layer information | O | Information to specify the corresponding spatial map layer specific information |
| > Augmented layer information | O | List of required augmented layer information. This IE can contain multiple values. Possible values are: - VAL_USERS; - SPATIAL_ANCHORS. |
| List of UE-object association information | O | List of UE-object association information to identify associations between UEs and objects in a spatial map. |

NOTE 1:	For the current release, the spatial map layer information is implementation specific and out of scope.

* * * Next Change * * * *

#### 9.3.4.2	Procedure

Figure 9.3.4.2-1 depicts the procedure to update the spatial map. For the request from the spatial map consumer (VAL server or SEAL SM client), SEAL SM server updates the spatial map as requested.

Figure 9.3.4.2-1: Update spatial map procedure

1)	The requestor (e.g., VAL server or SEAL SM client) sends a request message to the SEAL SM server to update a spatial map. The request includes requestor ID, security credentials, spatial map ID, information on what to update such as updated spatial map layering information and updated spatial map coverage area. As a part of update, the request may include augmented layer information (e.g., VAL users, spatial anchors). The request may include an updated list of UE-object association information for the SEAL SM server to identify associations between UEs and objects in a spatial map.

2)	The SEAL SM server authorizes the requestor and validates the request. The SEAL SM server updates the spatial map as requested.

3)	The SEAL SM server sends response message to the requestor with result of the request and updated spatial map information with the spatial map ID. SEAL SM server may include the requested augmented layer information in the response message.

* * * Next Change * * * *

##### 9.3.4.3.1	Update spatial map request

Table 9.3.4.3.1-1 descibes the information elements for update spatial map request from VAL server or SEAL SM client to SEAL SM server.

Table 9.3.4.3.1-1: Update spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or VAL User or VAL UE) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Application service ID | M | ID of the requesting VAL application service |
| Spatial map ID | M | ID of the spatial map to update |
| Spatial map update information | M | The spatial map Information to update as described in described in Table 7.3.3.1-1. Only IEs which need to be updated are included. |
| > Augmented layer information | O | List of required augmented layer information. This IE can contain multiple values. Possible values are: - VAL_UE; - SPATIAL_ANCHORS. |
| List of UE-object association information | O | List of UE-object association information to identify associations between UEs and objects in a spatial map. |
| NOTE: At least one of the information elements shall be provided. |  |  |

* * * Next Change * * * *

##### 9.3.6.2.1	Subscribe spatial map event

Figure 9.3.6.2.1-1 depicts the procedure for the authorized spatial map consumer (VAL server or SEAL SM client) to subscribe the spatial map event. The service is provided by SEAL SM server. Spatial map consumer can get notification information when the the SM server detects subscribed event.

Figure 9.3.6.2.1-1: Subscribe spatial map event procedure

1)	The VAL server (or SEAL SM client) sends a request message to the SEAL SM server to subscribe spatial map event. The request includes requestor ID, security credentials, a notification endpoint, target spatial map(s), notification intervals and duration for which events are notified, event to subscribe which triggers the notification. The request message may also include a list of spatial map layers with its corresponding layer IDs to subscribe spatial map layering information related events. The request may include a list of UE-object association information for the SEAL SM server to identify associations between UEs and objects in a spatial map. The requestor (VAL server or SEAL SM client) may subscribe to one or more of the following events:

a)	"changes of objects" events to indicate the SM server to notify the requestor when changes of the objects are detected. The notification triggering criteria for "changes of objects" event may include added or removed objects, position, velocity or direction changes of the spatial map objects, and UE association changes.

b)	"layers modification" events to indicate the SM server to notify the requestor when any layer in the spatial map is added or updated or removed.

c)	"spatial map ready" events to indicate the SM server to notify the requestor when the SEAL SM server has sufficient information to map all the objects related to area of interest the spatial map and the spatial map is ready to provide service (e.g. employed for localization). This event is used when the result IE in the Create spatial map response indicates "in-progress".

2)	The SEAL SM server authorizes VAL server (or SEAL SM client), and validates the request. If authorized, the SEAL SM server determines the target spatial map(s) using spatial map ID(s), area of interest, and/or location information provided in the request. Then the SEAL SM server adds the requestor into the subscriber list of the event. The SEAL SM server starts monitoring for events. For the "changes of objects" events, SM server can obtain the position, direction and velocity information of trackable objects from 5GS or SEAL Location Management service.

3)	The SEAL SM server sends response message with the result. For the successful subscribe request, a subscription ID is included in the message. Otherwise, the response message includes an indication of failure and may include a reason for failure.

* * * Next Change * * * *

##### 9.3.6.2.2	Notify spatial map event

Figure 9.3.6.2.2-1 depicts the notify spatial map event operation between the authorized spatial map consumer (VAL server or SEAL LM client) and SEAL SM server.

Figure 9.3.6.2.2-1: Notify spatial map event

1)	The SEAL SM server detects an event that triggers the notification to the subscribers for the corresponding event.

a)	If the subscribed event is "changes of objects", the SM server detects any changes of the spatial map objects. For the tracking objects, based on the data analytics for position, direction and velocity, the SM server can do trackable object mobility prediction and estimate the duration for which they stay in the spatial map. If UE-object association information has been provided, the SEAL SM server can use this information to determine if a detected spatial map object is associated with a UE.

NOTE:	How to do trackable object mobility prediction and estimate the duration of object tracking is implementation specific and out of scope.

b)	If the subscribed event is "Layers modification", the SM server detects any changes of the spatial map layers information.

a)	If the subscribed event is "spatial map ready", the SM server detects when the SEAL SM server has sufficient information to map all the objects related to area of interest the spatial map and the spatial map is ready to provide service (e.g. employed for localization).

SM server determines whether the detected event matchs triggering criteria and notifies subscribers accordingly.

2)	The SEAL SM server notifies the VAL servers (or SEAL SM clients) which subscribed the corresponding event. The notification includes a subscription ID and event-specific information. If the event is "changes of objects", spatial map notification message includes the information on the detected triggering event, e.g. added or removed objects, position or direction changes of the objects in the spatial map, UE association, etc. If the event is "Layer modification", the notification message includes the list of all layers for the spatial map.

* * * Next Change * * * *

##### 9.3.6.3.1	Subscribe spatial map request

Table 9.3.6.3.1-1 shows the subscribe spatial map event request sent by VAL server (or SEAL SM client) to SEAL SM server to subscribe for spatial map event(s).

Table 9.3.6.3.1-1: Subscribe spatial map event request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or VAL user) |
| Security credentials | M | The security credentials of the requestor. |
| Application service ID | M | ID of the requesting VAL application service |
| Expiry time | O | Proposed subscription expiry time |
| Notification endpoint | M | Indicates the address (e.g., IP address, FQDN, URI) where the notifications should be sent. |
| Target spatial map(s) | M | Spatial map(s) to subscribe |
| > List of spatial map IDs | O (NOTE 1) | ID(s) of the spatial map(s) to subscribe |
| > Area of interest | O (NOTE 1) | Three-dimensional area information to discover spatial map(s) to subscribe |
| > Location information | O (NOTE 1) | Location, orientation and pose information that is used to identify relevant spatial maps to subscribe. |
| List of events | M | Events to subscribe. Each element includes the information described below. |
| > Event type | O | List of events to subscribed. Possible values are: - Change of objects - Layers modification - spatial map ready |
| > specified objects | O | List of specified objects for "change of objects" event |
| > Triggering criteria | O | Specifies the condition to determine whether the event is occurred, e.g. Spatial map is employed and ready for localization service. |
| >> Changes in objects | O (NOTE 2) | Triggers notification if any or specified (or specified type of) objects are added, removed, changed in position, direction, velocity, UE association, etc. |
| >> Layer IDs | O (NOTE 2) | List of layers. Triggers notification if any or specified spatial map layer change is detected |
| > Notification intervals | O | Specifies when to notify the detected event, Possible values are: - Event based (e.g., promptly); Or - Periodically (with minimum time between consecutive notifications) - Intervals (i.e. minimum time between consecutive notifications) |
| > Interval time | O | Time between consecutive notifications (applicable only if notification intervals is set to intervals. |
| Tracking end condition | O | Specifies the condition until when events for the tracking objects are notified (e.g. as long as tracking objects stay in the spatial map, etc.) |
| NOTE 1: At least one of the sub-elements shall be provided. NOTE 2: At least one of the sub-elements shall be provided. |  |  |

* * * Next Change * * * *

##### 9.3.6.3.3	Notify spatial map event

Table 9.3.6.3.3-1 shows the spatial map notification sent by SEAL SM server to VAL server (or SEAL SM client) to notify about spatial map event(s).

Table 9.3.6.3.3-1: Spatial map event notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription ID for the event |
| Notification information | M | Specifies what matches the triggering notification condition, including related information |
| > Spatial map ID | M | ID of the spatial map for which the event is detected |
| > Event type | M | Type of event detected. One of the event as specified in Table 9.3.6.3.1-1. |
| >> Changed object | O (NOTE) | Information on the object which triggered the notification |
| >>> Object ID | O | ID of the changed object |
| >>> Type of change | O | Specifies the detected type of change for the object (e.g., added, removed, or changed position, direction, velocity, or UE association) |
| >>> Estimated duration of tracking | O | Estimated duration for which the object being tracked stays in the spatial map. Applicable for tracking objects. |
| >>> Object mobility prediction | O | Information predicting the mobility of the object being tracked. Applicable for tracking objects. |
| >>> Changed object information | O | A container to carry the changed object specific information |
| >> list of layers (NOTE 2) | O | Information about layers for the spatial map. |
| NOTE 1: This IE is applicable for "Change of objects" event type. NOTE 2: This IE is applicable for "Layer modification" event type. |  |  |

NOTE:	For the current release, the object specific information is implementation specific and out of scope.

* * * Next Change * * * *

### 9.4.2	Procedure

Figure 9.4.2-1 depicts the procedure for the authorized spatial map consumer (VAL server or SEAL SM client) to invoke spatial localization services. The service is provided by SEAL SM server.

Pre-condition:

1)	Spatial map is created and the authorized consumer is aware of the spatial map identity.

Figure 9.4.2-1: Spatial localization services

1)	The VAL server (or SEAL SM client) sends request to perform localization service in the spatial map(s) to the SEAL SM server. The request includes identity of the map(s) on which the action is to be performed. The request may include list of users (or UEs, objects) to localize. The request may include a list of UE-object association information for the SEAL SM server to identify associations between UEs and objects in a spatial map.

2)	The SEAL SM server authorizes the requestor (e.g. VAL server or SEAL SM client). If the requestor is not authorized then the SEAL SM server sends failure response. If the request is authorized: the SEAL SM server, identifies all users/UEs localization along with their three dimensional location and pose in the spatial map. If area of interest IE is present then the SEAL SM server identifies the target users/UEs within the area of interest only, otherwise, the SEAL SM server identifies the target users for whole map (according to the area associated with spatial map identifier). If UE-object association information has been provided, the SEAL SM server can use this information to determine if a spatial map object is associated with a UE. If a UE-object association is determined, the SEAL SM server can use localization information of the associated object to determine user/UE localization in the spatial map, or use localization information of the associated UE to determine object localization in the spatial map.

3)	The SEAL SM server sends response to the VAL server (or SEAL SM client). The response includes the object localization and identity.

NOTE:	In this release, the localization of UE is considered. Localization of all objects will be considered in future release.

* * * Next Change * * * *

#### 9.4.3.1	Spatial localization service request

Table 9.4.3.1-1 describes information elements for the spatial localization service request from the SM client or VAL server to the SM server.

Table 9.4.3.1-1: Spatial localization service request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL User or VAL UE, VAL server) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Spatial Map list |  | List of spatial maps to be used for spatial localization |
| > Spatial Map Id | M | Identity of the spatial map |
| Area of interest | O | Contains three dimensional area within spatial map |
| Target identities | O | List of identity of the users or objects to localize |
| List of UE-object association information | O | List of UE-object association information to identify associations between UEs and objects in a spatial map. |

#### 9.4.3.2	Spatial localization service response

Table 9.4.3.2-1 describes information elements for the spatial localization service response from the SM server to the SM client or VAL server.

Table 9.4.3.2-1: Spatial localization service response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of the action |
| Object details (NOTE) | O | A list of objects or users |
| >> UE id | O | Identity of the UE (or associated UE) |
| >> Object ID | O | Identifier of the object (or associated object) |
| >> localization | O | 3D position and pose of the VAL UE or object |
| NOTE: This IE should be added in case of successful response. |  |  |

* * * End of Changes * * * *