3GPP TSG-SA WG6 Meeting #68	S6-253075

Gothenburg, Sweden, 25th – 29th August 2025	(revision of S6-xxxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0397 | rev |  | Current version: | 18.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Temporary group formation fix |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEAL_Ph3 |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In step 3 & step 4 of clause 10.3.10.1 Temporary group formation within a VAL system, there are “temporary VAL group ID” for the information of the temporary group, but there are “VAL group ID” of Information elements in 10.3.2.39 Temporary group formation response, 10.3.2.40 Temporary group formation notify, 10.3.2.41 Temporary group formation notification, 10.3.2.42 Temporary group formation notification response, althrough the related Descriptions are “VAL group ID of the temporary group”. - And there are “VAL group ID list” Information elements 10.3.2.40 Temporary group formation notify, 10.3.2.41 Temporary group formation notification, which are “List of constituent VAL group IDs”. “VAL group ID” & “VAL group ID list” are too similar, it is not friendly API design. - In 3GPP TS 24.544 R18 - parameter VAL Group Id set to the VAL group ID of the temporary group - VAL Group Ids Represents the list of constituent VAL group IDs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace Information elements ““VAL group ID” to “Temporary VAL group ID”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not friendly API design and mismatch procedure of Temporary group formation within a VAL system. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.2.39, 10.3.2.40, 10.3.2.41, 10.3.2.42 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

10.3.2.38	Temporary group formation request

Table 10.3.2.38-1 describes the information flow for the temporary group formation request from the group management client to the group management server.

Table 10.3.2.38-1: Temporary group formation request

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID list | M | List of VAL group IDs to be combined |
| VAL service list (see NOTE) | O | A subset of the common VAL service(s) to be applied for the temporary group |
| NOTE: If this information element is not present, all the VAL service(s) that are common to the groups being combined will be applicable for the temporary group. |  |  |

10.3.2.39	Temporary group formation response

Table 10.3.2.39-1 describes the information flow for the temporary group formation response from the group management server to the group management client.

Table 10.3.2.39-1: Temporary group formation response

| Information element | Status | Description |
| --- | --- | --- |
| Temporary VAL group ID | O (see NOTE) | VAL group ID of the temporary group |
| VAL service list | O | List of VAL services whose service communications are to be enabled on this temporary group |
| Result | M | Indicates the success or failure of temporary group formation. |
| NOTE: Shall be present if the Result information element indicates that the temporary group formation operation is successful. Otherwise VAL group ID shall not be present. |  |  |

10.3.2.40	Temporary group formation notify

Table 10.3.2.40-1 describes the information flow temporary group formation notify from the group management server to the VAL server.

Table 10.3.2.40-1: Temporary group formation notify

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID list | M | List of constituent VAL group IDs |
| Temporary VAL group ID | M | VAL group ID of the temporary group |
| VAL service list | O | List of VAL services whose service communications are to be enabled on this temporary group. |

10.3.2.41	Temporary group formation notification

Table 10.3.2.41-1 describes the information flow temporary group formation notification from the group management server to the group management client.

Table 10.3.2.41-1: Temporary group formation notification

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID list | M | List of constituent VAL group IDs |
| Temporary VAL group ID | M | VAL group ID of the temporary group |

10.3.2.42	Temporary group formation notification response

Table 10.3.2.42-1 describes the information flow temporary group formation notification response from the group management client to the group management server.

Table 10.3.2.42-1: Temporary group formation notification response

| Information element | Status | Description |
| --- | --- | --- |
| Temporary VAL group ID | M | VAL group ID of the temporary group |

# * * * * End of changes * * * *