3GPP TSG-SA WG6 Meeting #68	S6-253175

Stor-Göteborg, Sweden, 25th Aug 2025 - 29th Aug 2025

Source:	Jio Platforms

Title:	Introduction of AIMLE client participation negotiations

Spec:	3GPP TR 23.700-83

Agenda item:	9.4

Document for:	Approval

Contact:	Raghvendra Bhushan <Raghvendra.Bhushan@ril.com>

1. Introduction

The Key Issue#6, on the study aspects on support enhancements related to diverse AIMLE deployments scenarios includes:

- Whether and how to Support Architecture and Functions Enhancement on diverse AIMLE deployments scenarios.

The AIMLE server gets client registration request with availability schedule of client. However, an AIMLE client may run multiple other tasks on request of its owner (like running GPU intensive games etc.). Under such scenario, it may happen that the client’s resource remains utilized even during its proposed schedule and it becomes difficult for UE/AIMLE client to predict and propose an operational schedule. This problem exacerbates in case of emergency mode. Moreover, operational schedule is an optional parameter in client’s registration request suggesting that a client can register without an operational schedule. Hence there shall be a mechanism in which an AIMLE server negotiates with AIMLE clients/UE about their current availability schedule if UE has registered without operational schedule or extensively long operational schedule (like a daylong or more).

This paper proposes a participation negotiation mechanism in which AIMLE server will negotiate with AIMLE clients about their availability. These clients will be nominated on basis of selection criteria and if no AIML task is currently running on the AIMLE client. The AIMLE server may propose multiple time slots and UE may choose one slot out of them. For synchronous task, AIMLE server shall propose same slot to all nominated UE and agreeing UEs will be selected. While in case of asynchronous task, AIMLE server may select UEs with different slots. For Asynchronous task, the slots provided by AIMLE server shall be nearby to avoid stale updates.

2. Reason for Change

This paper proposes solution to enhance client participation mechanism. Currently, an AIMLE client needs to predict and propose a time schedule when it will be available to run a ML task. However, owner of UE or emergency services may run resource consuming task on UE during the proposed schedule. Moreover, some emergency training may fail if UE/AIMLE client is not available in near future. Hence, it becomes important to design a framework where AIMLE client and server negotiate availability in real-time to schedule an AIML task in present or near future.

This pCR proposes to enhance the AIMLE client participation by introducing client participation negotiation phase and interactions between client and server to agree on task schedule.

3. Conclusions

The mechanism proposed in this paper enhances AIMLE client participation procedure by managing task schedule on clients after discussions between client and AIMLE server in paticipation phase.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v0.2.0.

* * * First Change * * * *

## 7.XX	Introduction of AIMLE Client participation negotiation

### 7.XX.1	Solution Description

#### 7.XX.1.1	General

The AIMLE client participation phase has two type of negotiations. In one, the AIMLE server sends multiple time slots to UE to choose one from them and participate in task. While in another, the UE who has registered with no time schedule suggests it nearby time shedule when it can perform AIML Task. The AIML server provides an ACK to the clients agreed schedule.

### 7.XX.2	Procedure

#### 7.XX.2.1	AIMLE client participation negotiations

Figure 7.XX.2.1-1: AIMLE Client Participation Negotiation

- The VAL server sends an AIMLE client selection request to AIMLE server with member selection criteria. The AIMLE server checks that the training shall be done on priority and search for clients who are available in near future.

The AIML server finds some UE registered without availability schedule or very long availability schedule. The AIMLE server initiates participation negotiation phase with those UEs. While it initiates participation request with the UE who have registered with a training schedule in nearby future.

In case the request contains a list of clients, the server initiates participation negotiation with those clients to participate in ML training.

The step 2 and step 3 of clause 8.9.2.1 remains unaffected and server can procced with them if required.

- The AIML server initiate “AIML Participation negotiation request” which may contain list of slots when AIML task can be scheduled.

- The AIMLE client selects some schedules and reply back to AIMLE server in “AIML participation negotiation response”. If the server request does not contain time slots the UE suggests time slots of near future as per its resource availability and current ongoing tasks.

- The server send acknowledgement to AIMLE client in “AIML participation negotiation acknowledgement” with a selected time slot among those provided by AIMLE client in response. The AIMLE server receives time slots from multiple AIMLE clients in “AIML participation negotiation response”. It selects those AIMLE clients who have proposed almost common time slots in case of synchronous AIML task. While it may choose AIMLE clients of varying schedules in near future for asynchronous AIML task.

- The AIMLE server sends client selection response to VAL server with client set identifier or list of selected clients.

#### 7.XX.3.1	AIMLE client participation negotiation request

Table 7.XX.3.1-1 shows the request sent by the AIMLE server to each AIMLE client selected for AIMLE client participation negotiation procedure.

Table 7.XX.3.1-1: AIMLE client participation negotiation request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor. |
| AIMLE client set identifier | M | An identifier for the AIMLE client set. |
| ML model ID | M | The ML model ID to use for the AI/ML operation. |
| AI/ML operations | M | A list of AI/ML operations (e.g., training) required to be performed. |
| Operation time slots | O | Time slots options for AIMLE client to choose for AI/ML operations. |
| Dataset requirement | M | Dataset requirements for the AI/ML operations. Requirements includes dataset identifier, dataset size and age, and/or dataset features. |
| Service requirement | M | Information about the required Service including its VAL service ID and its service permission level (e.g., premium resource usage standard resource usage, limited resource usage) |

#### 7.XX.3.2	AIMLE client participation negotiation response

Table 7.XX.3.2-1 shows the response sent by the AIMLE clients to the AIMLE server for the AIMLE client participation negotiation procedure.

Table 7.XX.3.2-1: AIMLE client participation negotiation response

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor. |
| Status | M | The status for the request: success or failure. |
| Preferred time slots | M | The time slots preferred by AIMLE client to perform task |
| Cause | O | The cause for the request failure. |

#### 7.XX.3.3	AIMLE client participation negotiation acknowledgement

Table 7.XX.3.3-1 shows the response sent by the AIMLE clients to the AIMLE server for the AIMLE client participation procedure.

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor. |
| Status | M | The status for the request: success or failure. |
| AIMLE client set identifier | M | An identifier for the AIMLE client set |
| Add/remove indicator | M | Indicator for adding/removing the AIMLE client to/from the AIMLE client set. |
| Operational schedule | M | Schedule for the AI/ML operations. |
| Cause | O | The cause for the request failure. |

# *** End of Changes ***