3GPP TSG-SA WG6 Meeting #68	S6-253196

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253xxx)

Source:	Nokia

Title:	Pseudo-CR on key issue on enhancements to API invoker offboarding

Spec:	3GPP TR 23.700-43

Agenda item:	9.11

Document for:	Approval

Contact:	Sapan Shah (sapan.shah@nokia.com)

1. Introduction

This pCR provides new key issues on enhancements to API invoker offboarding

2. Reason for Change

As specified in the clause 6.1 of 3GPP TS 33.122, the API invoker obtains onboarding enrolment information from the API provider domain. Without onboarding enrolment information provided by the API provider domain, the API invoker would not be able to perform onboarding to CCF. The API provider domain (for example, Gaming server) will provide the onboarding enrolment information to the API invoker (for example, Gamin client on the UE) if the user has subscribed to (or paid for) the service provided by the API provider domain (for example, Gaming service).

Currently, in clause 8.23.3 of 3GPP TS 23.222 [r23222], AEF revokes the authorization of the API invoker to access specific service API. However, there is no provision for AEF to handle scenarios where API invoker is unsubscribed or became rogue.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-43 (v0.0.0)

* * * First Change * * * *

## 4.x	Key issue #x: Enhancements to API invoker offboarding

### 4.x.1	Description

Background: As specified in the clause 6.1 of 3GPP TS 33.122 [r33122], the API invoker obtains onboarding enrolment information from the API provider domain. Without onboarding enrolment information provided by the API provider domain, the API invoker would not be able to perform onboarding to CCF. The API provider domain (for example, Gaming server) will provide the onboarding enrolment information to the API invoker (for example, Gamin client on the UE) if the user has subscribed to (or paid for) the service provided by the API provider domain (for example, Gaming service).

Use cases:

1)	In certain scenario, the user of the UE (on which the API invoker is available) could decide to not continue receiving service from the AEF, and would like to unsubscribe the service (or may not pay for the service). In such scenario, the API provider domain would need to ensure that the API invoker cannot access any service APIs exposed by the AEF of the API provider domain after unsubscription.

2)	Out of band service unsubscription: Service subscription/unsubscription is often a business process that might involve multiple "elementary" services and that is coordinated by a BSS entity. Currently, the AMF/API provider domain supports the creation of onboarding credentials which can be used by the subscriber during onboarding the client on the UE (i.e. API invoker). However, the offboarding via AMF/API provider domain is missing and, hence, needs to be addressed.

3)	Blocking rogue API invokers: API invokers can prove to be rogue (e.g. invoking APIs even after rejected or expired authorization or even after the maximum invocation rate limit is crossed), or cannot be eligible anymore for API usage (e.g. due to unpaid bills). In such case, offboarding initiated by the AMF/API invoker domain is needed.

Possible gap: Currently, in clause 8.23.3 of 3GPP TS 23.222 [r23222], AEF revokes the authorization of the API invoker to access specific service API. However, there is no provision for AEF to handle scenarios where API invoker unsubsribes or became rogue.

### 4.x.2	Open issues

The following open issues are required to study:

1)	whether and how to enable API provider domain to offboard the API invoker who has unsubscribed the services from the API provider domain?

2)	whether and how the API provider domain revokes the authorization of the API invoker for multiple service APIs?

* * * End of Change * * * *