3GPP TSG-SA WG6 Meeting #68	S6-253227

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253xyz)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0317 | rev | - | Current version: | 19.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Charging the invocation of service APIs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS 32.254 specifies charging management for exposure and northbound interfaces, being hence the TS to be defined and referenced by TS 23.222. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the Editor’s Note. Correction of the NOTE to indicate that the information flows and architecture related to charging management are specified in TS 32.254. Other clauses (6.1, 6.3.3, 6.4.6, 8.19.3, Annex A, B.0, B.1.1, B.2.1, C.1, C.2, C.3, Annex D) are corrected accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Charging the invocation of the service API remains unsolved |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.2, 6.1, 6.3.3, 6.4.6, 8.19.3, 8.20.1, 8.20.2, 8.20.3, Annex A, B.0, B.1.1, B.2.1, C.1, C.2, C.3, Annex D |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.682: "Architecture enhancements to facilitate communications
with packet data networks and applications".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[5]	3GPP TR 26.981: "MBMS Extensions for Provisioning and Content Ingestion".

[6]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles".

[7]	ETSI GS MEC 011 (V1.1.1): "Mobile Edge Computing (MEC); Mobile Edge Platform Application Enablement".

[8]	ETSI GS MEC 009 (V1.1.1): "Mobile Edge Computing (MEC); General Principles for Mobile Edge Service APIs".

[9]	OMA-ER_Autho4API-V1_0-20141209-A: "Authorization Framework for Network APIs".

[10]	OMA-TS-REST_NetAPI_Common-V1_0-20180116-A: "Common definitions for RESTful Network APIs".

[11]	OMA-TS-NGSI_Registration_and_Discovery-V1_0-20120529-A: "NGSI Registration and Discovery".

[12]	3GPP TS 33.122: "Security Aspects of Common API Framework for 3GPP Northbound APIs".

[13]	3GPP TS 23.435: "Procedures for Network Slice Capability Exposure for Application Layer Enablement Service".

[14]	IETF RFC 6749 (October 2012): "The OAuth 2.0 Authorization Framework".

[15]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[16]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[17]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[18]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[19]	3GPP TS 28.538: "Management and orchestration; Edge Computing Management".

[20]	3GPP TS 32.254: "Telecommunication management; Charging management; Exposure function northbound Application Programing Interfaces (APIs) charging".

* * * Next  Change * * * *

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GS	5G System

AEF	API Exposing Function

AF	Application Function

AMF	API Management Function

APF	API Publishing Function

API	Application Program Interface

AS	Application Server

BM-SC	Broadcast Multicast Service Centre

CAPIF	Common API Framework

CRUD	Create, Read, Update, Delete

DDoS	Distributed Denial of Service

E-UTRA	Evolved Universal Terrestrial Radio Access

EPS	Evolved Packet System

ETSI	European Telecommunications Standards Institute

GS	Group Specification

IP	Internet Protocol

MBMS	Multimedia Broadcast and Multicast Service

MEC	Multi-access Edge Computing

NEF	Network Exposure Function

NGSI	Next Generation Service Interfaces

NR	New Radio

OMA	Open Mobile Alliance

OAM	Operations, Administration and Maintenance

OWSER	OMA Web Services

PC	Protocol Converter

PLMN	Public Land Mobile Network

REST	REpresentational State Transfer

RNAA	Resource owner-aware Northbound API Access

RO	Resource Owner

ROF	Resource Owner Function

RPC	Remote Procedure Call

SCEF	Service Capability Exposure Function

SCS	Service Capability Server

SNPN	Stand-alone Non-Public Network

UDDI	Universal Description, Discovery and Integration

URI	Uniform Resource Identifier

WSDL	Web Services Description Language

* * * Next  Change * * * *

## 6.1	General

The Common API framework (CAPIF) functional architecture is described in this subclause. The CAPIF architecture is defined as service-based and interactions between the CAPIF functions are represented in two ways:

-	A service-based representation, where CAPIF functions enable other authorized CAPIF functions to access their services;

-	A reference point representation, where interactions between any two CAPIF functions (e.g. CCF, AEF) is shown by an appropriate point-to-point reference point (e.g. CAPIF-3).

The CAPIF functional architecture can be adopted by any 3GPP functionality providing 3GPP northbound service APIs.

Charging aspects of CAPIF framework are specified in 3GPP TS 32.254 [20].

NOTE 1:	The terms “functional architecture” and “functional model” mean the same and have been used interchangeably in this specification.

NOTE 2:	The functional model described in this specification applies to both PLMN(s) and to SNPN(s).

* * * Next  Change * * * *

### 6.3.3	CAPIF core function

The CAPIF core function consists of the following capabilities:

-	Authenticating the API invoker based on the identity and other information required for authentication of the API invoker;

-	Mutual authentication with the API invoker;

-	Publishing, storing, retrieving, unpublishing, updating and discovering of service APIs information;

-	Controlling the service API access based on PLMN operator configured policies;

-	Storing the logs for the service API invocations and providing the service API invocation logs to authorized entities;

-	Monitoring the service API invocations;

-	Onboarding a new API invoker and offboarding an API invoker;

-	Storing policy configurations related to CAPIF and service APIs;

-	Accessing the logs for auditing (e.g. detecting abuse);

-	Publishing, retrieving, unpublishing, updating,and discovering service APIs information with another CAPIF core function in CAPIF interconnection; and

-	Slice related API exposure in, e.g., API publish, API discovery, API invoker authorization, API access control.

* * * Next  Change * * * *

### 6.4.6	Reference point CAPIF-3 (between the API exposing function and the CAPIF core function)

The CAPIF-3 reference point, which exists between the API exposing function and the CAPIF core function, is used for exercising access and policy related control for service API communications initiated by the API invoker.

The CAPIF-3 reference point supports:

-	Authenticating the API invoker based on the identity and credentials of the API invoker;

-	Providing authorization for the API invoker prior to accessing the service API;

-	Providing authorization revocation information;

-	Authorization verification for the API invoker upon accessing the service API;

-	Authorization verification for the API invoker based on RNAA;

-	Controlling the service API access based on PLMN operator configured policies; and

-	Logging the service API invocations.

NOTE:	The security aspects of CAPIF-3 are specified in subclause 6.6 of 3GPP TS 33.122 [12].

* * * Next  Change * * * *

### 8.19.3	Procedure

Figure 8.19.3-1 illustrates the procedure for logging service API invocations at AEF.

Pre-conditions:

1.	The API invoker(s) has invoked certain service API(s).

2.	Authorization details of the AEF are available with the CAPIF core function.

Figure 8.19.3-1: Procedure for logging service API invocations

1.	Upon invocation of service API(s) from one more API invokers, the AEF triggers API invocation log request towards the CAPIF core function.

NOTE 1:	The AEF can collect the log information associated to several API invocations before triggering API invocation log request asynchronously.

2.	The CAPIF core function makes a log entry and stores the information e.g. for monitoring purposes, for access by authorized users and entities.

NOTE 2:	API invocation log is stored for a configured duration.

3.	AEF receives the API invocation log response from the CAPIF core function.

* * * Next  Change * * * *

### 8.20.1	General

The procedure in this subclause corresponds to the architectural requirements for charging the invocation of service APIs. The AEF can be within PLMN trust domain or within 3rd party trust domain. The charging of the invocation of the CAPIF core function APIs and the charging of the invocation of service APIs is specified in 3GPP TS 32.254 [20].

### 8.20.2	Information flows

NOTE:	Charging aspects of CAPIF framework, i.e., charging architecture and information flows, are specified in 3GPP TS 32.254 [20].

### 8.20.3	Procedure

NOTE:	Charging aspects of CAPIF framework, i.e., charging architecture, information flows and deployment options, are specified in 3GPP TS 32.254 [20].

* * * Next  Change * * * *

Annex A (informative):
Overview of CAPIF operations

Depicted in figure A-1 is the overview of CAPIF operations. CAPIF operations occur between different actors involving the API invoker, the CAPIF core function, the API exposing function, the API publishing function and the API management function, and optionally the resource owner function for RNAA. High level CAPIF interactions between the actors are shown in figure A-1. This figure is only provided for illustration purposes, and does not represent the order of operations.

Figure A-1: Overview of CAPIF operations

The CAPIF defines the functional entities in subclause 6.3.

The CAPIF defines the reference points between the functional entities in subclause 6.4.

The following operations require the communication between the CAPIF entities:

1.	Publishing service APIs: the API provider utilizes the API publishing function over CAPIF-4 reference point to publish the service APIs on the CAPIF core function, as specified in subclause 8.3 of this specification;

2.	Discovering service APIs: the API invoker discovers the service APIs over CAPIF-1/CAPIF-1e reference points, as specified in subclause 8.7 of this specification;

3.	API event subscription and notification: the API invoker subscribes to and receive service API event notifications over CAPIF-1/CAPIF-1e reference points, as specified in subclause 8.8 of this specification;

4.	Authenticating with CAPIF: the API invoker authenticates itself over CAPIF-1/CAPIF-1e reference points, as specified in subclause 8.10 of this specification;

5.	Authorizing with CAPIF: the API invoker obtains service API authorization over CAPIF-1/CAPIF-1e reference points, as specified in subclause 8.11 of this specification. In RNAA scenarios, API authorization is based on the authorization information obtained from the resource owner, as specified in clause 8.31;

6.	Topology hiding:	the API provider, to hide the topology, utilizes the API exposing function over CAPIF-3 reference point, as specified in subclause 8.13 of this specification;

7.	Authenticating the API invoker prior to service API invocation: the API provider, to authenticate the API invoker prior to the service API invocation, utilizes the API exposing function over CAPIF-2/CAPIF-2e and CAPIF-3, as specified in subclause 8.14 of this specification;

8.	Authenticating the API invoker upon the service API invocation: the API provider, to authenticate the API invoker upon invocation of the service APIs, utilizes the API exposing function over CAPIF-2/CAPIF-2e and CAPIF-3, as specified in subclause 8.15 of this specification;

9.	Authorizing API invoker:  the API provider, to authorize the API invoker to access the service APIs, utilizes the API exposing function over CAPIF-2/CAPIF-2e and CAPIF-3, as specified in subclause 8.16 of this specification;

10.	Access control: the API provider, to control the access of the service API by the API invoker based on policy or usage limits,

-	utilizes the API exposing function over CAPIF-2/CAPIF-2e and CAPIF-3, as specified in subclause 8.17 of this specification; or

-	in a cascaded deployment, utilizes API exposing functions over CAPIF-2/CAPIF-2e, as specified in subclause 8.18 of this specification;

11.	Logging service: the API provider, to maintain the log of the API invocations at the CAPIF core function for services such as invocation history, utilizes the API exposing function over CAPIF-3, as specified in subclause 8.19 of this specification;

12.	Service monitoring: the API provider, to facilitate monitoring such as API invoker's ID and IP address, utilizes the API management function over CAPIF-5, as specified in subclause 8.21 of this specification; and

13.	Auditing: the API provider, for auditing, utilizes the API management function over CAPIF-5, as specified in subclause 8.22 of this specification.

NOTE:	Charging aspects of CAPIF framework, i.e., charging architecture and information flows, are specified in 3GPP TS 32.254 [20].

* * * Next  Change * * * *

# B.0	CAPIF utilization by service API provider

Figure B.0-1 illustrates the service API interaction with the CAPIF for utilizing framework aspects provided by the CAPIF.

Figure B.0-1: CAPIF utilization by service API provider

The service API aspects of the 3GPP network services and capabilities such as subscriber management, mobility management, transport and other communication services can be exposed for consumption by external 3rd party applications (e.g. API invoker).

Framework aspects typically horizontal in nature caters to common functionality such as onboarding, offboarding, publishing, unpublishing, update service API, discovery, authentication, registration, authorization, logging,  monitoring, configuration, topology hiding, that are required to provide service APIs to API invokers. Service APIs can utilize the functions of the API provider domain (i.e. API exposing function, API publishing function, API management function) and interfaces CAPIF-3, CAPIF-4 and CAPIF-5 as specified in this specification.

The service API exposure function is connected to 3GPP network entity(s) via 3GPP internal interface(s). The API publishing function provides the service API information for publishing to the CAPIF core function.

For consuming service API, the API invoker interacts with the service API exposure function via service API interface and CAPIF-2/2e. While the service API interface is responsible for providing service aspects, CAPIF-2/2e supports service API by providing framework aspects such as authentication of the API invoker, authorization verification for the API invoker upon accessing the service API.

* * * Next  Change * * * *

## B.1.1	General

The table B.1.1-1 shows the relationship between CAPIF and EPS network exposure aspects. The details of SCEF and its role in exposing network capabilities of EPS to 3rd party applications are specified in 3GPP TS 23.682 [2]

Table B.1.1-1: CAPIF relationship with 3GPP EPS network exposure

| Aspects | CAPIF | EPS network exposure |
| --- | --- | --- |
| Entity providing the APIs to external or 3rd party applications | AEF | SCEF |
| Entity providing framework related services to the applications (discovery, authentication, authorization, etc) | CAPIF core function | SCEF |
| Entity representing the external or 3rd party applications | API invoker | SCS/AS |
| Entity providing framework related services to support the APIs operation and management (publish, policy enforcements) | CAPIF core function | SCEF |
| Interface/Reference point for exposing network capabilities as APIs | CAPIF-2 and CAPIF-2e (Do not include the service specific aspects) | T8 |
| Interface/Reference point for exposing framework services as APIs to the applications | CAPIF-1 and CAPIF-1e | Not specified. (May be via T8) |
| Interface/Reference point for framework services to support the APIs operation and management | CAPIF-3, CAPIF-4 and CAPIF-5 | Internal to SCEF |

* * * Next  Change * * * *

## B.2.1	General

The table B.2.1-1 shows the relationship between CAPIF and 5GS network exposure aspects. The details of NEF and its role in exposing network capabilities of 5GS to 3rd party applications are specified in 3GPP TS 23.501 [3] and the details of NEF service operations are specified in 3GPP TS 23.502 [4].

Table B.2.1-1: CAPIF relationship with 3GPP 5GS network exposure

| Aspects | CAPIF | 5GS network exposure |
| --- | --- | --- |
| Entity providing the APIs to external or 3rd partyapplications | AEF | NEF |
| Entity providing framework related services to the applications (discovery, authentication, authorization, etc) | CAPIF core function | NEF (Not specified yet) |
| Entity representing the external or 3rd party applications | API invoker | AF |
| Entity providing framework related services to support the APIs operation and management (publish, policy enforcements) | CAPIF core function | NEF (Not specified yet) |
| Interface/Reference point for exposing network capabilities as APIs | CAPIF-2 and CAPIF-2e (Do not include the service specific aspects) | Nnef |
| Interface/Reference point for exposing framework services as APIs to the applications | CAPIF-1 and CAPIF-1e | Nnef (Not specified yet) |
| Interface/Reference point for framework services to support the APIs operation and management | CAPIF-3, CAPIF-4 and CAPIF-5 | Internal to NEF |

* * * Next  Change * * * *

# C.1	General

This annex provides the information about the role of CAPIF in charging service API invocations. The common architecture for charging is illustrated in clause 4 of 3GPP TS 32.240 [6]. There are two charging mechanisms - offline charging and online charging. The role of CAPIF in both these charging mechanims is illustrated for informational purpose in this subclause. The charging management for the exposure of northbound APIs when the AEF is within the PLMN is specified in 3GPP TS 32.254 [20].

The API invocations are subjected to charging (online, offline) as described in clause 4.4 of 3GPP TS 32.254 [20].

* * * Next  Change * * * *

# C.2	CAPIF role in online charging

The API invocations are subjected to online charging as described in clasue 5.3 of 3GPP TS 32.254 [20].

* * * Next  Change * * * *

# C.3	CAPIF role in offline charging

The API invocations are subjected to offline charging as described in clasue 5.2 of 3GPP TS 32.254 [20].

* * * Next  Change * * * *

Annex D (informative):
CAPIF relationship with external API frameworks

This annex provides the relationship of CAPIF with the OMA Network APIs and the ETSI MEC API framework. The relationship of CAPIF with these external API frameworks is illustrated in the table D-1. "Yes" means that the external API framework supports the CAPIF functionality, "No" means that the API framework does not support the CAPIF functionality, and "Partial" means that it provides a mechanism that partially supports the CAPIF functionality.

Table D-1: CAPIF relationship with external API frameworks

| CAPIF functionalities | OMA Network APIs |  | ETSI MEC API framework |  |
| --- | --- | --- | --- | --- |
|  | Supported | Reference | Supported | Reference |
| Publish and discover service API information | Partial (see NOTE) | OMA-TS-NGSI_Registration_and_Discovery [11] | Yes | ETSI GS MEC 011 [7] |
| Topology hiding of the service | Yes | Individual API exposing function | Yes | Individual API exposing function |
| API invoker authentication to access service APIs | Partial | OMA-ER_Autho4API [9] | Partial | ETSI GS MEC 009 [8] |
| API invoker authorization to access service APIs | Partial | OMA-ER_Autho4API [9] | Partial | ETSI GS MEC 009 [8] |
| Lifecycle management of service APIs | No |  | No |  |
| Monitoring service API invocations | No |  | No |  |
| Logging API invoker onboarding and service API invocations | No |  | No |  |
| Auditing service API invocations | No |  | No |  |
| Onboarding API invoker to CAPIF | No |  | No |  |
| CAPIF authentication of API invokers | No |  | No |  |
| Service API access control | Partial | OMA-ER_Autho4API [9] | Partial | ETSI GS MEC 009 [8] |
| Secure API communication | Yes | OMA-ER_Autho4API [9] | Yes | ETSI GS MEC 009 [8] |
| Policy configuration | No |  | No |  |
| API protocol stack model | Partial | for REST: OMA-TS_REST_NetAPI_Common [10] | Partial | for REST: ETSI GS MEC 009 [8] |
| API security protocol | Partial | OMA-ER_Autho4API [9] | Partial | ETSI GS MEC 009 [8] |
| CAPIF support for service APIs from multiple providers | No |  | No |  |
| NOTE: OMA-TS-NGSI_Registration_and_Discovery [11] is only applicable to a specific type of web services (OWSER using UDDI and WSDL). |  |  |  |  |

* * * End of Changes * * * *