3GPP TSG-SA WG6 Meeting #68	S6-253245

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.437 | CR | 0061 | rev | - | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Spatial anchor state management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_Ph2-APP |  |  |  |  |  | Date: |  |  | 2025-08-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Currently the spatial anchor state is being managed by the SAn server based on the time period specified in the spatial anchor profile by the spatial anchor creator. SAn server updates the spatial anchor state as active or inactive/suspended based on the time period specified in the profile. But there might be scenarios where in the SAn creator or owner may want to deactivate the spatial anchor or activate the spatial anchors atleast for the spatial anchors wherein the time period is not specified. Applications would need this capability for varied reasons. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proedure to activate the spatial anchor(s) Procedure to deactivate the spatial anchor(s) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Functionality of spatial anchor management will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Next Change * * * *

### 8.3.x	Spatial anchor state management

#### 8.3.x.1	General

Spatial anchor state can be active or inactive at any point of time and this state could be auto managed by the SAn server if the spatial anchor profile contains the time period of when it needs to be in active or inactive state. This clause defines the procedures and information flows for managing the state of spatial anchors by the SAn client or VAL server.

#### 8.3.x.2	Procedures

##### 8.3.x.2.1	Spatial anchor activation

Figure 8.3.x.2.1-1 illustrates the spatial anchor activation procedure.

Pre-conditions:

1.	The SAn client or VAL server has received information (e.g. URI, IP address) related to the SAn server; and

2.	The SAn client or VAL server has received security credentials authorizing it to communicate with the SAn server.

3.	The spatial anchor(s), which are being activated, are in inactive state.

Figure 8.3.x.2.1-1: Spatial anchor activation

1.	The requestor (e.g., SAn client or VAL server) sends a spatial anchor activate request to the SAn server. The request includes a requestor identifier, security credentials, list of spatial anchors be activated as specified in Table 8.3.x.3.1-1. The requestor may include the duration which carries the time period of keeping the spatial anchor in active state. After this duration, SAn server automatically changes the state of the spatial anchor to de-activated or suspended.

2.	Upon receiving the request, the SAn server validates if the requestor is authorized for the activation of spatial anchor(s). If the requestor is authorized, the SAn server activates the list of spatial anchors specified in the request and updates the state information in the stored spatial anchor profile of the spatial anchor to “active”.

3.	The SAn server sends a spatial anchor activate response as described in Table 8.3.x.3.2-1 to the requestor. If the SAn server activated all the spatial anchors listed in the request, the response includes an indication of success. Otherwise, the response includes an indication of failure and list of spatial anchor IDs which are not activated and includes the reason for failure.

##### 8.3.x.2.2	Spatial anchor deactivation

Figure 8.3.x.2.1-1 illustrates the spatial anchor deactivation procedure.

Pre-conditions:

1.	The SAn client or VAL server has received information (e.g. URI, IP address) related to the SAn server; and

2.	The SAn client or VAL server has received security credentials authorizing it to communicate with the SAn server.

3.	The spatial anchor(s), which are being deactivated, are in active state.

Figure 8.3.x.2.2-1: Spatial anchor deactivation

1.	The requestor (e.g., SAn client or VAL server) sends a spatial anchor deactivation request to the SAn server. The request includes a requestor identifier, security credentials, list of spatial anchors IDs to be deactivated as specified in Table 8.3.x.3.3-1. The requestor may include the duration, which carries the time period of keeping the spatial anchor in deactivated or inactive state. After this duration, SAn server automatically updates the state of the spatial anchor to active.

2.	Upon receiving the request, the SAn server validates if the requestor is authorized for the de-activation of spatial anchor(s). If the requestor is authorized, the SAn server de-activates the list of spatial anchors specified in the request and updates the state information in spatial anchor profile of the spatial anchor to “de-activated” or “inactive”.

3.	The SAn server sends a spatial anchor deactivate response as specified in Table 8.3.x.3.4-1to the requestor. If the SAn server de-activated all the spatial anchors listed in the request, the response includes an indication of success. Otherwise, the response includes an indication of failure and list of spatial anchor IDs that are not deactivated and the reason for failure.

#### 8.3.x.3	Information flows

##### 8.3.x.3.1	Spatial anchor activate request

Table 8.3.x.3.1-1 describes information elements for the spatial anchor activate request from the SAn client or VAL server to the SAn server.

Table 8.3.x.3.1-1: Spatial anchor activate request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| List of spatial anchors IDs | M | List of spatial anchors IDs to be activated. |
| Time period or duration | O | Specifies the time period of how long the spatial anchor to be in active state. |

##### 8.3.x.3.2	Spatial anchor activate response

Table 8.3.x.3.2-1 describes information elements for the spatial anchor activate response from the SAn server  to SAn client or VAL server.

Table 8.3.x.3.2-1: Spatial anchor activate response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the spatial anchor activate request was successful. |
| >List of Spatial anchor IDs | O | List of spatial anchor IDs which are activated successfully. |
| Failure response | O (NOTE) | Indicates that the spatial anchor activate request has failed. |
| > Cause | O | The cause for the request failure. |
| NOTE: Only one of these IE shall be present. |  |  |

##### 8.3.x.3.3	Spatial anchor deactivate request

Table 8.3.x.3.3-1 describes information elements for the spatial anchor deactivate request from the SAn client or VAL server to the SAn server.

Table 8.3.x.3.3-1: Spatial anchor deactivate request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| List of spatial anchors IDs | M | List of spatial anchors IDs to be deactivated. |
| Time period or duration | O | Specifies the time period of how long the spatial anchor to be in inactive state. |

##### 8.3.x.3.4	Spatial anchor deactivate response

Table 8.3.x.3.4-1 describes information elements for the spatial anchor deactivate response from the SAn server to SAn client or VAL server.

Table 8.3.x.3.2-1: Spatial anchor deactivate response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the spatial anchor deactivate request was successful. |
| >List of Spatial anchor IDs | O | List of spatial anchor IDs which are deactivated successfully. |
| Failure response | O (NOTE) | Indicates that the spatial anchor deactivate request has failed. |
| > Cause | O | The cause for the request failure. |
| NOTE: Only one of these IE shall be present. |  |  |