3GPP TSG-SA WG6 Meeting #68	S6-253277

Gothenburg, Sweden 25th – 29th Aug 2025	(revision of S6-25xxxx)

Source:	CATT

Title:	New KI on management of AIoT devices

Spec:	3GPP TR 23.700-26 v0.0.0

Agenda item:	9.8

Document for:	Approval

Contact:	Liping Wu <wuliping@cictmobile.com>

1. Introduction

The contribution proposes a new KI on management of AIoT devices.

2. Reason for Change

Some application servers (such as a logistics company) may need regularly track the location of the goods in the truck and identify an appropriate route or direction for the goods depending on the location of the truck, and may adjust the route or direction for the goods in time in case they deviate the right route/direction.

SA6 should consider meeting the above application server’s requirements on top of network layer from application enabled layer perspective.

3. Proposal

It is proposed to agree the following changes for 3GPP TR 23.700-26 v0.0.0.

* * * First Change * * * *

5	Key issues

5.x	Key issue #x: Management of AIoT devices

5.x.1	Description

Some application servers (e.g. a logistics company) may need to regularly track the location of the AIoT devices (e.g. the goods in the truck) and identify the trajectory or the direction for the AIoT devices depending on their locations, and the application server may send warnings to the AIoT devices in time in case they deviate the right trajectory/direction.

SA1 in 3GPP TS 22.369 [22369] has specified “Subject to user consent, operator’s policy and 3rd party request, the 5G system shall provide information about an Ambient IoT device or a group of Ambient IoT devices (e.g. position) to the trusted 3rd party via the 5G network.” Currently, SA2 in Rel-19 (via 3GPP TS 23.369 [23369]) supports to report AIoT Device’s location using a RAN Reader ID that represents the AIoT Device’s location at Reader granularity.

However, SA2 is not able to generate the location trajectory for the AIoT devices and report to the application server in time in case the location of AIoT devices deviate the right trajectory /direction (then application server may send warning/reminder to the AIoT devices).

SA6 should consider meeting the above application server’s requirements on top of network layer from application enabled layer perspective.

5.x.2	Open Issues

Based on the above analysis, the following open issues need to be studied in the application enabled layer:

-	Whether and how to associate the AIoT devices identifiers with different services;

-	Whether and how to monitor the AIoT devices’ status (e.g. location) to minimize the service impact.

* * * End of Change * * * *