3GPP TSG-SA WG6 Meeting #68	S6-253305

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253xxx)

Source:	Huawei, Hisilicon

Title:	Pseudo-CR onupdate adoption solution#1

Spec:	3GPP 23.700-35 v1.0.0

Agenda item:	9.3

Document for:	Approval

Contact:	Cuili Ge – gecuili@huawei.com

1. Introduction

This pCR is proposed to update the adoption solution#1.

2. Reason for Change

The current adoption solution #2 only shows the SEAL and CAPIF, however, what is the relationship between SEAL and the other VAE layer entities within the holistic picture is not fully reflected.

So it is proposed to add the other VAE entities into this solution.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.700-35 v1.0.0.

* * * First Change * * * *

8.2	Solutions for SEAL services adoption gaps

8.2.1	Adoption Solution#1: Service based Architecture of 3GPP system supporting SEAL services

8.2.1.1	Solution description

This solution is to solve GAP# 5-1-2-1.

Figure 8.2.1-1 Service based Architecture of 3GPP system supporting SEAL services

In the figure 8.1.1-1, SEAL services layer represents the SEAL server(s) and the SEAL client(s) (if applicable) supporting SEAL services defined in the SEAL specification 3GPP TS 23.434 [3].

SEAL services which are part of the 3GPP system are deployed either by the MNO or third party.

SEAL server(s)  invokes service APIs exposed by 3GPP core network functions directly over the 3GPP core network SBA when deployed as trusted AF in MNO’s trusted domain. Alternatively, SEAL server(s) invokes service APIs exposed by 3GPP core network functions via NEF as a non-trusted AF when deployed by the third party.

SEAL server(s) invokes service APIs exposed by the OAM system over the OAM SBMA when deployed by MNO as specified in 3GPP TS28.533[14].

SEAL clients take two different responsibilities:

1) consumer of SEAL server API(s)

2) providing required terminal side information/resource to support the SEAL server function(s)

SEAL services APIs are provided and exposed by MNOs or third party, and they are consumed by the Application specific layer(VAL layer) vertical application enabler layer (e.g., VAE server, UAE server), Edge Enabler layer (e.g., EES, ECS), SEAL client(s) or third party’s platform.

NOTE 1:	 It is up to application developer’s implementation how to invoke the SEAL server’s APIs in their application layer software.

NOTE 2:	The API invokers as defined in 3GPP TS 23.222 [9] can either be on the network side (Application server) or on the UE side (e.g., SEAL clients), to invoke SEAL server API(s).

8.2.1.2	Functional Elements

NOTE:	No new function elements are added to the existing 3GPP system.

8.2.1.3	Reference Points

NOTE:	No new reference points added to the existing 3GPP system.

8.2.1.4	Solution evaluation

This solution provides the holistic picture providing the role and responsibility of SEAL service layer within the complete 3GPP system perspective. It clarifies the SEAL services can be implemented as part of 3GPP system and can well avoid some misleading.