3GPP TSG-SA WG6 Meeting #68	S6-253358

Gothenburg, Sweden 25th – 29th August 2025

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on updating Solution #1 for control plane

Spec:	3GPP TR 23.700-52 v0.2.0

Agenda item:	9.7

Document for:	Approval

Contact:	changlinhui@huawei.com

- Introduction

This pCR is proposed to add the control plane description to the solution identified in clause 6.1.

2. Reason for Change

The Editor's note in Solution#1 “The control plane protocol stack of SEALDD-UU needs to be considered and is FFS.” and “The control plane protocol stack of SEALDD-S needs to be considered and is FFS.”.

So, this pCR is proposed to add the control plane protocol stack.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR  23.700-52 v0.2.0.

# * * * First Change * * * *

## 6.1	Solution #1:	SEALDD-S and SEALDD-UU user plane functionality description and protocol stack

### 6.1.1	Solution description

#### 6.1.1.1	General

Currently, SEALDD layer provide several data delivery services towards the VAL layer with different user plane characteristics as analysed in table 6.1.1.1-1.

Table 6.1.1.1-1: SEALDD delivery service user plane analysis

| Services of SEALDD service | User plane characteristic of SEALDD-S | User plane characteristic of SEALDD-UU |
| --- | --- | --- |
| Regular Transmission | Address: VAL server endpoint address (e.g., IP address and/or port, URL) and SEALDD server endpoint address (e.g., IP address and/or port, URL). | Address: SEALDD server endpoint address (e.g., IP address and/or port, URL) and SEALDD client endpoint address (e.g., UE's addresses/ports). Identifier: SEALDD-UU Flow ID |
| URLLC Transmission | Address: VAL server endpoint address (e.g., IP address and/or port, URL), and SEALDD server endpoint address (e.g., IP address and/or port, URL). | Address: SEALDD server endpoint address (e.g., IP address and/or port, URL) and SEALDD client endpoint address (e.g., UE's addresses/ports). Identifier: SEALDD-UU Flow ID shared by the two paths. |
| Data Storage | Put DL data to the SEALDD server (storage server) - Push mode in a standalone user plane Fetch data from the SEALDD server - Fetch mode in a standalone user plane | Put UL data to the SEALDD server (storage server) - Push mode in a standalone user plane Fetch data from the SEALDD server - Fetch mode in a standalone user plane |
| BAT data transfer | Refers to the Data storage | Refers to the Data storage |
| XR Transmission | Address: VAL server endpoint address (e.g., IP address and/or port, URL) and SEALDD server endpoint address (e.g., IP address and/or port, URL). | Address: SEALDD server endpoint address (e.g., IP address and/or port, URL) and SEALDD client endpoint address (e.g., UE's addresses/ports). Identifier: SEALDD-UU Flow IDs and multi-modal flow ID. |
| Transmission Quality Measurement |  | SEALDD UU DL monitoring header, or dummy DL SEALDD packet generated by SEALDD server time stamp: local time T1, T2, T3, T4 |

Editor's note:	This table may be not complete and any further updates is FFS.

#### 6.1.1.2	User plane protocol stack for SEALDD-UU

This clause illustrates the protocol stack for the user plane data delivery related with SEALDD-UU.

Figure 6.1.1.2-1: SEALDD-UU user plane stack

SEALDD-UU user plane:	This user plane of the SEALDD-UU layer corresponds to the protocol data unit carried between the SEALDD client and the SEALDD server over the SEALDD-UU flow. The protocol(s) used for this SEALDD-UU may varies due to different SEALDD data delivery service.

N6 protocol stack:	This set of protocols/layers depends on the UPF.

NOTE 1:	For simplicity, only a box named intermediate protocol stacks are shown. For details, it refers the user plane protocol stacks as described in 3GPP TS 23.501 [23.501].

The user plane of SEALDD-UU-U layer shall support the functions/characteristics listed in table 6.1.1.1.

#### 6.1.1.3	Control plane protocol stack for SEALDD-UU

This clause illustrates the protocol stack for the control plane data delivery related with SEALDD-UU.

Figure 6.1.1.3-1: SEALDD-UU control plane stack

SEALDD-UU control plane:	This control plane of the SEALDD-UU layer corresponds to the protocol data unit delivered between the SEALDD client and the SEALDD server which is used for managing SEALDD-UU user plane data transmission, transmission quality measurement. The protocol(s) used for this SEALDD-UU may varies due to different SEALDD data delivery service.

N6 protocol stack:	This set of protocols/layers depends on the UPF.

NOTE 1:	For simplicity, only a box named intermediate protocol stacks are shown. For details, it refers the control plane protocol stacks as described in 3GPP TS 23.501 [23.501].

#### 6.1.1.4	User plane protocol stack for SEALDD-S

This clause illustrates the protocol stack for the user plane data delivery related with SEALDD-S.

Figure 6.1.1.4-1: SEALDD-S user plane stack

SEALDD-S user plane:	This user plane of SEALDD-S layer corresponds to the protocol data unit carried between the VAL server and the SEALDD server over the SEALDD-S flow. The protocol(s) used for this SEALDD-S user plane may varies due to different SEALDD data delivery service.

The SEALDD-S-U layer shall support the functions/characteristics listed in table 6.1.1.1.

#### 6.1.1.5	Control plane protocol stack for SEALDD-S

This clause illustrates the protocol stack for the control plane data delivery related with SEALDD-S.

Figure 6.1.1.5-1: SEALDD-S control plane stack

SEALDD-S control plane: This control plane of SEALDD-S layer corresponds to the protocol data unit delivered between the VAL server and the SEALDD server which is used for creating and managing SEALDD-S user plane data transmission.

### 6.1.2	Architecture Impacts

This solution has no architecture impacts.

### 6.1.3	Corresponding APIs

This solution has no new API.

### 6.1.4	Solution evaluation

Editor's note:	This clause describes an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.