3GPP TSG-SA WG6 Meeting #68	S6-253446

Goteborg, Sweden 25th – 29th August 2025	(revision of S6-253159)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0459 | rev | 3 | Current version: | 19.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Combining multiple ad hoc group call |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph6-MC |  |  |  |  |  | Date: |  |  | 2025-08-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The clause introduces the possibility to combine multiple ad hoc group calls. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a text to clarify how multiple ad hoc group calls can be merged. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 1 requirements are not met. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.15.2.1, 10.15.3.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 10.15.2.1	Preconfigured regroup request (MC service client – MC service server)

Table 10.15.2.1-1 describes the information flow preconfigured regroup request from the MC service client to the MC service server.

Table 10.15.2.1-1 Preconfigured regroup request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the requester |
| MC service group ID | O | MC service group ID of the regroup group |
| MC service group ID | M | MC service group ID of the MC service group from which configuration is to be taken |
| MC service group ID list | O (see NOTE 1, NOTE 3) | List of MC service groups to be regrouped into the group regroup group |
| MC service user ID list | O (see NOTE 1) | List of MC service users to be regrouped into the user regroup group |
| MC service user criteria | O (see NOTE 1, NOTE 2) | Carries the details of criteria which will be used by the MC service server for determining the participants. For example, it can be a location based criteria to determine the MC service user ID list in a particular area, or it could be tags (e.g. "fire", "medical", "police", etc.), or a combination of tags and location, which is left to implementation. |
| NOTE 1: One and only one of these shall be present. NOTE 2: This information element is used only for the user regroup procedures NOTE 3: This information element shall be used to combine multiple MC group IDs associated to ongoing ad hoc group communications. |  |  |

# * * * * Second change * * * *

##### 10.15.3.2.1	Preconfigured Group Regroup formation in a single MC system

Figure 10.15.3.2.1-1 illustrates the procedure to initiate a group regroup using a preconfigured MC service group. The procedure takes place prior to the establishment of a group call to the MC service regroup group.

Furthermore, this procedure is applicable to allow an authorised MC service user to combine multiple MC service groups associated to ongoing ad hoc group communication.

NOTE 1:	The upper limit of the number of MC service group IDs associated to ongoing ad hoc group communications to be combined is up to implementation.

Pre-conditions:

1.	MC service client 2 is an affiliated member of MC service group 1 and MC service client 3 is an affiliated member of MC service group 2.

2.	The MC service group identity and group configuration for the preconfigured MC service group have been preconfigured in MC service clients 2 and 3, and MC service clients 2 and 3 have received the relevant security related information to allow them to communicate in the MC service regroup group.

3.	MC service client 1 is authorized to initiate a preconfigured group regroup procedure.

4.	MC service client 1 is aware of a suitable preconfigured MC service group whose configuration has been preconfigured in the MC service UEs of the group members who will be regrouped.

5.	In order to be aware whether a group is currently regrouped, the MC service server is subscribed to the group configuration in the GMS.

6.	The GMS has subscribed to group dynamic data from the MC service server using the procedures defined in subclause 10.1.5.6.

NOTE 2:	MC service client 2 and MC service client 3 are replaced by MC service group ID 1 and MC service group ID 2 associated with ongoing ad hoc group communication 1 and ad hoc group communication 2, respectivly.

NOTE 3:	Preconditions 1, 5 and 6 are not valid for MC service group IDs associated to ad hoc group communications.

MC service client 1

MC service server

MC service

client 2

MC service client 3

1. Initiate preconfigured regroup

6a. Notifyuser

7a. Preconfigured regroup response

2. Preconfigured regroup request

3. Check authorization and resolve group IDs

6b. Notify

user

9. Preconfigured regroup response

Home MCPTT service provider

5a. Preconfigured regroup request

5b. Preconfigured regroup request

7b. Preconfigured regroup response

8. Affiliate MC service clients to regroup group

4a. Preconfigured regroup reject

4b. Preconfigured regroup request return

Figure 10.15.3.2.1-1: Regroup procedure using preconfigured group in a single MC system

1.	The authorized user of MC service client 1 initiates the group regroup procedure, specifying the list of MC service groups to be regrouped (MC service groups 1 and 2), the MC service group ID of the regroup group (if available) and the MC service group ID of the group from which configuration information for the regroup group is to be taken.

2.	MC service client 1 sends the preconfigured regroup request to the MC service server.

3.	The MC service server checks that MC service client 1 is authorized to initiate a preconfigured group regroup procedure, and resolves the group identities of the MC service groups requested in step 1. The MC service server also checks which group members are affiliated to MC service groups 1 and 2. The MC service server may retrieve the configuration for the regroup group from the GMS if that configuration information is not already known to the MC service server. The MC service server also checks that none of the MC service groups that are requested for regrouping are already regrouped by any mechanism. If the preconfigured regroup request is authorized, the MC service server assigns a MC service group ID for this regroup group if:

i. one is not provided in the request or

ii. the one provided in the request is not accepted.

NOTE 1:	This procedure does not require that that the authorized user of MC service client 1 is a group member of MC service groups 1 or 2, or that the authorized user of MC service client 1 is an affiliated group member of MC service groups 1 or 2.

NOTE 2:	The list of groups included in the group regroup is held in dynamic data in the MC service server, and is not used to update group configuration information in the group management server.

4a.	If the MC service server determines that any of the groups requested for regrouping, including the regroup group, have been regrouped by other group regrouping procedures, the MC service server then sends a preconfigured regroup reject back to MC service client 1 with a reject reason indicating that one of the groups has already been regrouped, and this procedure terminates.

4b.	The MC service server shall send the preconfigured regroup request return message to MC service client 1 containing the below:

i.	result of whether the preconfigured regroup is authorized or not; and

ii.	the MC service group ID if authorized

5.	If the preconfigured regroup request is not authorized, MC service client 1 shall not proceed with the rest of the steps and this procedure terminates.

5a/b.	The MC service server sends the preconfigured regroup requests to MC service clients 2 and 3.

NOTE 3:	Only group members that are affiliated to the MC service groups that are to be regrouped are sent a preconfigured regroup request.

6a/b.	MC service clients 2 and 3 notify their users of the regrouping.

7a/b.	MC service clients 2 and 3 may send the preconfigured regroup response to the MC service server to acknowledge the regrouping action. These acknowledgements are not sent in response to a multicast transmission of the preconfigured regroup request.

8.	The MC service server affiliates the regrouped MC service clients to the regroup group.

9.	The MC service server sends a preconfigured regroup response to MC service client 1.

After the group regrouping procedure, the regrouping remains in effect until explicitly cancelled by the procedure in 10.15.3.2.3.

MC service client participation in the ongoing regroup persists until the MC service client is no longer affiliated to any of the regrouped groups (group 1 or 2 in this procedure).

MC service client affiliation to the regroup group may cease when the UE's MC service ceases, e.g. when the UE is powered down, or by performing a log-off operation.

Editor's note:	Data persistence in the MC service client following a user log-off or power down needs further study.

# * * * * End of changes * * * *