3GPP TSG-SA WG6 Meeting #68	S6-253638

Gothenburg, Sweden 25th – 29th Aug 2025	(revision of S6-25xxxx)

Source:	CATT, CMCC, Samsung

Title:	New KI on exposing the value-added information of AIoT devices to the consumer

Spec:	3GPP TR 23.700-26 v0.0.0

Agenda item:	9.8

Document for:	Approval

Contact:	Liping Wu <wuliping@cictmobile.com>

1. Introduction

The contribution proposes a new KI on exposing the value-added information of AIoT devices to the consumer.

2. Reason for Change

At present, SA2 could provide the requested AIoT Devices ID(s) to the AF. However, these AIoT Devices ID(s) are so many and may not being consolidated per services before sending to the AF. But the 3rd parties (e.g. AS/AF) are more interested in obtaining the direct and rich results. E.g., the 3rd parties expect to receive the changed AIoT Devices ID(s) compared to last time, the aggregated AIoT Devices ID(s) per different services, or the AIoT Devices ID(s) combined with the e.g. location data, etc. So how to provide this value-added information to the 3rd parties should be studied in SA6.

3. Proposal

It is proposed to agree the following changes for 3GPP TR 23.700-26 v0.0.0.

* * * First Change * * * *

5	Key issues

5.x	Key issue #x: Exposing the value-added information of AIoT devices to the consumer

5.x.1	Description

At present, 5G core network in Rel-19 could provide the requested AIoT Devices ID(s) to the AF as specified in 3GPP TS 23.369[23369]. However, these AIoT Devices ID(s) are huge and may not fulfill the service requirements from the 3rd parties (e.g. AS/AF). The following are some examples to show the gaps between the service requirements and the current exposure capabilities.

For example, a 3rd-party retail App-server (i.e., VAL server) may issue periodic inventory service requests to check the volume of some specific types of AIoT devices in a certain area. When the number of the available AIoT devices of a type drops below a certain threshold, the retail store might run into service interruption of not having sufficient goods for customers. In this scenario, the corresponding retailer is better off replenishing the stock in advance by adding more devices of the type. If the retail store asks for periodic inventory service, it’s expected to be informed when and which type of devices will be increased or decreased. However, based on current exposure results from 5G core network as specified in 3GPP TS 23.369[23369], only the AIoT devices ID will be provided for the inventory request, how to identify the AIoT devices in service granularity and how to indicate when one type of device will reach the pre-defined threshold to perform the increase or decrease actions should be studied further in application enablement layer.

In another scenario, if AF sends continuously a batch of 'inventory service’ requests covering different areas (with different area-IDs), though these areas belonging to a same (large) targeted area whose identity could be a geographical civic address. Then, the AIoT application enabler may aggregate these requests and send only one combined 'inventory service’ request to the 5G core network instead of sending several 'inventory service’ requests continuously.

Besides, some application servers (e.g. a logistics company) may need to regularly track the location of the AIoT devices (e.g. the goods in the truck) and identify the trajectory or the direction for the AIoT devices depending on their locations, and then send warnings to the AIoT devices in case they deviate the right trajectory/direction. However, based on current exposure results from 5G core network, only the device ID with RAN reader ID may be exposed, but how to map the RAN ID to the location information (e.g. civic address) and track the devices’ location continually as requested should be studied further in application enablement layer.

In a word, it would be of great advantage to AIoT services consumers, if they received enriched results from the enabler layer. Hence, how to provide value-added services to the consumers based on the AIoT devices information retrieved from the core network should be studied at the application enablement layer.

5.x.2	Open Issues

Based on the above analysis, the following open issues need to be studied in the application enabled layer:

- How to enrich the processing (e.g. filtering, categorization, consolidation) of AIoT service requests and the collected Ambient IoT device data;

- How to utilize the exposure from other working group (e.g. SA2) and provide the value-added AIoT devices information to the consumer (e.g. VAL server) based on open issue #1.

* * * End of Change * * * *