3GPP TSG-SA6 Meeting #69	S6-254104

# Wuhan, China, 13th Oct 2025 - 17th Oct 2025                             (revision of S6-254xxx)

Source:	Airbus

Title:	Updated solution #3: Recording MC Data MSRP sessions

Spec:	3GPP TR 23.700-39 V0.3.0

Agenda item:	9.2

Document for:	Approval

Contact:	Jukka Vialen (jukka.vialen@airbus.com)

1. Introduction

This contribution proposes an update to solution to Solution#3.

2. Reason for Change

Solution #3 in 3GPP TR 23.700-39 V0.3.0 is not complete.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-39 V0.3.0.

* * * First Change * * * *

## 6.3	Solution 3 (for KI#6): Recording MC Data MSRP sessions

### 6.3.1	Description

#### 6.3.1.0	General

MC Data SDS and FD can use MSRP on a SIP session (media plane).

This solution adds an MSRP Switch into the MCData server and a requirement to support MSRP protocol in REC-4 reference point (MCS server – Recording server). An MSRP Switch is able to duplicate the MRSP messages and forward a copy to the recording server.

### Multi-party Messaging with MSRP Relays

The most common way to send an MSRP message to multiple destinations is by routing it through an MSRP Relay or a MSRP Switch. These are intermediary servers that act as a central point for a messaging session. The process typically works as follows:

- Session Setup: All participants in a group chat or multi-party session (like an RCS group chat) establish their individual MSRP sessions with the central MSRP Relay. This is done using SIP/SDP signaling, where each endpoint's session description points to the relay.

- Message Routing: When a user sends a message, they send it to the MSRP Relay. The relay's role is to receive the message and then duplicate it.

- Splitting the Message: The relay checks its internal routing table for the session and then forwards a copy of the message to each of the other participants' MSRP URIs. Each forwarded message is a complete and separate MSRP message, delivered over the session established between the relay and that specific user. The message is not "split" in the sense of being fragmented, but rather duplicated and re-sent.

This architecture is vital for modern messaging services as it solves several issues:

- NAT/Firewall Traversal: It allows users behind firewalls or Network Address Translation (NAT) to communicate without a direct peer-to-peer connection.

- Presence and Session Management: The relay can manage the status of all participants, handling things like join/leave events and message delivery reports for all members of the group.

- Scaling: It centralizes the messaging logic, making it more scalable and manageable for a service provider.

### The Role of Headers

The MSRP protocol itself, as defined in RFC 4975 and its extensions, supports the concept of a path. The To-Path header can include a sequence of one or more MSRP URIs. This allows for messages to be relayed through intermediate points. While this feature is primarily for routing through relays and is not for sending a message to multiple final destinations simultaneously in a true peer-to-peer fashion, it is the underlying mechanism that allows the relay to know where to send the message.

In a group chat scenario, the relay effectively takes on the role of the "sender" for each individual user, ensuring that each member of the group receives a complete and correctly formatted message.

The following text in the text box is generated by Google Gemini and is only meant for educational purposes. Its correctness is not checked. This text will not be added to the normative TSs.

Yes, an MSRP relay can support multi-party messaging, but it doesn't do so by itself. The core MSRP protocol is a one-to-one, or peer-to-peer, protocol. To enable multi-party messaging (like a group chat), the MSRP relay must be used in conjunction with a centralized conferencing server, often called a "conference focus" or "MSRP switch."

### How It Works

- Session Initiation Protocol (SIP): The multi-party session is not created or managed by MSRP itself. Instead, the Session Initiation Protocol (SIP) is used to set up and manage the conference. The SIP server, acting as the conference focus, handles the invitations, participant lists, and other signaling aspects of the group chat.

- MSRP Relay's Role: Once the SIP session is established, each participant's client opens an MSRP session to the centralized conferencing server. The MSRP relay then acts as a conduit, forwarding each participant's messages to the server. Its main job is to ensure that the messages can traverse Network Address Translation (NAT) and firewalls, which is a common challenge in peer-to-peer connections.

- The Conference Focus/MSRP Switch: This is the key component. When a message arrives at the conference focus, it is the focus's responsibility to duplicate the message and send a copy to all other participants in the group chat. It is this server, not the MSRP relay, that performs the multi-party message distribution.

In short, the MSRP relay facilitates the message flow between a client and the central server, but the server is what enables the multi-party functionality by replicating and forwarding messages to the correct recipients.

#### 6.3.1.0a	References

The following referece(s) need to be added to 3GPP TS 23.280 [2] and TS 3GPP 23.282 [5].

[xx]	IETF RFC 4975 (September 2007): "The Message Session Relay Protocol (MSRP)".

[yy]	IETF RFC 7701 (December 2015): Multi-party Chat Using the Message Session Relay Protocol (MSRP)

#### 6.3.1.1	Functional model and reference points

(See also 6.3.2).

- To enable recording of SDS when using media plane (3GPP TS 23.282 [2] clauses 7.4.2.3, 7.4.2.4 and 7.4.2.6) an MSRP client and MSRP switch are added to Figure 6.5.1-1: “Application plane functional model for SDS”. Updates are visible with blue color.

Figure 6.3.1.1-1: Updated Figure 6.5.1-1 of 3GPP TS 23.282 [5]

In the model shown in figure 6.5.1-1, the following apply:

-	MCData-SDS-1 reference point is primarily used for MCData application signalling during session establishment in support of SDS data transfer. Secondarily, MCData-SDS-1 reference point is used for uplink and downlink unicast SDS data transaction over signalling control plane by the SDS distribution function of the MCData server and SDS function of the MCData client.

-	MCData-SDS-2 reference point carries uplink and downlink unicast SDS data over media plane between the SDS distribution function of the MCData server and the SDS function of the MCData client.

-	MCData-SDS-3 reference point carries downlink multicast SDS data over media plane from the SDS distribution function of the MCData server to the SDS function of the MCData client.

-	MCData-12 reference point and MSRP Switch function in the MCData server are required to enable recording of on-network Short Data Service using media plane. The MSRP protocol is specified in IETF RFC 4975 [xx] and IETF RFC 7701 [yy] and is out of the scope of the present document.

Examples of SDS data (in the form of text, binary, application data, URL or combinations of these) are:

-	information pertaining to applications e.g. health parameters of MCData user for situational awareness application;

-	information pertaining to enhanced status service;

-	text or URL data between MCData users;

-	application data (e.g. health parameters) to the MCData user;

-	location information (independent or along with user or application provided data);

-	command instructions to invoke certain operations on the MCData UE e.g. invoking UE specific applications; and

-	application plane identities for the MCData user and MCData application.

The text below Figure 6.5.1-1 of 3GPP TS 23.282 [5] is amended as shown below:

- To enable recording of FD when using media plane (3GPP TS 23.282 [5] clauses 7.5.2.5 and 7.5.2.7) an MSRP client and MSRP switch are added to Figure 6.6.1-1: “Application plane functional model for file distribution”. Updates are visible with blue color.

Figure 6.3.1.1-2: Updated Figure 6.6.1-1 of 3GPP TS 23.282 [5]

In the model shown in figure 6.6.1-1, the following apply:

-	MCData-FD-1 reference point is primarily used for MCData application signalling for establishing a session in support of MCData file distribution. Secondarily, MCData-FD-1 reference point is also used for both uplink and downlink unicast data (e.g., URL associated to file, file download completed report).

-	MCData-FD-2 reference point carries uplink and downlink unicast file data between the FD functions of the MCData server and the MCData UE.

-	MCData-FD-3 reference point carries downlink multicast file data from the FD function of the MCData server to the FD function of the MCData UE.

-	MCData-FD-4 reference point carries uplink and downlink unicast file data between the media storage function of the MCData Content server and the media storage client of the MCData UE.

-	MCData-FD-5 reference point supports the MCData server to access the stored files in the MCData content server for certain file distribution functions, such as retrieval a file to be distributed through multicast etc. This reference points also supports any necessary operational requirements.

-	MCData-FD-7 reference point supports the upload and download of file data between MCData content server and MCData message store.

-	MCData-12 reference point is required to enable recording of File Distribution on-network using media plane. The MCData server shall implement an MSRP Switch function. The MSRP protocol is specified in IETF RFC 4975 [xx] and IETF RFC 7701 [yy] and is out of the scope of the present document.

NOTE:	The security aspects of MCData-FD-5 and MCData-FD-7 reference points are the responsibility of SA3 and thus outside the scope of the present document.

The text below Figure 6.6.1-1 of 3GPP TS 23.282 [5] is amended as shown below:

#### 6.3.1.2	Configurations

No changes. All configurations related to MSRP are out of the scope of MC stage 2 TSs.

#### 6.3.1.3	Procedures

Editor's note: Impacts on 3GPP TS 23.280 [2] clause 10.18.3 (Recording procedures) are FFS.

### 6.3.2	Impacts on existing functional entities and reference points

1)	For the existing reference points, Figure 7.3.1.2-3 in 3GPP TS 23.280 [2] (Relationships between reference points of MC service application plane and signalling control planes) needs a small update i.e. adding MSRP to the REC-4 reference point, see Figure 6.3.2-1.

Figure 6.3.2-1: Updated Figure 7.3.1.2-3 in 3GPP TS 23.280 [2]. Relationships between reference points of MC service application plane and signalling control planes

Editor's note: It is FFS if SIPREC should be added to REC-4 in the figure above?

#### 7.5.2.34	Reference point REC-4 (between recording server and MC service servers)

The REC-4 reference point, which exists between recording server and MC service servers, is used for transmitting metadata and media of the communication sessions of target users and target groups from the MC service servers to the recording server.

NOTE:	In this version of this specification the communication sessions that may be recorded are MCPTT and MCVideo group and private calls.

The REC-4 reference point shall use the SIP-2 reference point for transport and routing of signalling and communication sessions related metadata. If an MC service server and a recording server are served by different SIP cores, then the REC-4 reference point shall also use the SIP-3 reference point for transport and routing of signalling and communication sessions related metadata.

For the recording RTP media and related metadata of session-based user communications (e.g. MCPTT and MCVideo private and group calls), MC service server and recording server shall use Session Recording Protocol as specified in IETF RFC-7866 [40]. for establishing and controlling recording sessions between MC service servers and recording server for MCPTT and MCVideo private and group communications. IETF RFC-7866 [40] specifies the use of SIP, Session Description Protocol (SDP), and RTP to establish a recording session.

For the recording of MC Data MSRP sessions, the MC Data server shall implement an MSRP Switch (see IETF RFC 7701 [yy]), capable of duplicating the MSRP messages and sending a copy to the recording server. The recording server shall implement an MSRP client. MSRP protocol (IETF RFC 4975 [xx]) shall be supported in the REC-4 reference point.

2)	Clause 7.5.2.34 in 3GPP TS 23.280 [2] needs to be updated:

### 6.3.3	Solution evaluation

FFS.

* * * End of changes * * * *