3GPP TSG-SA WG6 Meeting #69	S6-254122

Wuhan, China 13th – 17th October 2025	(revision of S6-25xxxx)

Source:	Telefónica, Apple, Fogus, UMA

Title:	TR 23.947 Annex CAPIF plus EDGEAPP topology

Spec:	3GPP TR 23.947

Agenda item:	9.14

Document for:	Approval

Contact:	w_featherstone@apple.com

1. Introduction

This contribution provides the Annex CAPIF plus EDGEAPP topology text for the Guidelines for 3GPP Application Enablement usage (AppEnable_EXT) technical report.

2. Reason for Change

This contribution proposes text A.1 of 3GPP TR 23.947.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.947, version 0.1.0.

* * * First Change * * * *

## A.1	CAPIF plus EDGEAPP Topology

### A.1.1	General

This annex describes a deployment in a single PLMN where the Common API Framework (CAPIF) integrates with the Edge Application Enablement (EDGEAPP) architecture. The scenario illustrates how onboarding, API publication, discovery, authorization, and invocation are carried out with CAPIF security in place.

This sample deployment includes the following entities:

- CAPIF Core Function (CCF): Provides onboarding, API catalogue, authorization, and token issuance.

- Edge Configuration Server (ECS): Central API provider for Edge configuration.

- Two Edge Enablement Servers (EES): One per Edge location.

- Two Edge Application Servers (EAS): One per Edge location, co-located with each EES.

- Two User Terminals (UEs): Each has an Edge Enabler Client (EEC) and an Application Client (AC) connected to its nearest Edge location.

The Logical Topology is shown in Figure A.1.1-1, where the letters and numbers are independent, for instance AC-A or AC-B could connected to either EAS.

Central Zone

Edge Zone 2

Edge Zone 1

CCF

ECS

EES-1

EES-2

EAS-1

EAS-2

UE A

UE B

EEC-A

AC-A

EEC-B

AC-B

Figure A.1.1-1: CAPIF plus EDGEAPP sample topology

### A.1.2	Step 1: ECS Onboarding into CAPIF

The ECS needs first to be provisioned with the CAPIF Root CA, the CAPIF base URL, and a bootstrap JWT/credentials. It can then register with the CCF as an API Provider (clause 10.12.2 of 3GPP TS 23.222 [2]), establishing its trusted identity. The flow is depicted in Figure A.1.2-1.

Figure A.1.2-1: ECS onboarding flow

### A.1.3	Step 2: ECS Publishes APIs

Having obtained a valid certificate, the ECS publishes its service APIs (e.g., EESRegistration, configuration APIs) into CAPIF. The flow is depicted in Figure A.1.3-1.

Figure A.1.3-1: ECS Publish API flow

### A.1.4	Step 3: EES Onboarding into CAPIF

Before onboarding, each EES is provisioned with:

- CAPIF Root CA certificate(s), to validate TLS when connecting to the CCF.

- CAPIF Core Function base URL, the discovery root for onboarding, discovery, and token endpoints.

- Bootstrap JWT and/or credential material, initial credentials used to authenticate to CCF for onboarding (per operator policy).

The EES establishes a secure TLS session to the CCF using the CA root, authenticates with the bootstrap token/credentials, and performs:

- API Invoker onboarding (so it can later discover/consume ECS APIs), and

- API Provider registration (so it can later publish its own EES-facing APIs, e.g., EASRegistration/Discovery).

The EES onboarding as an API Invoker and then registering as an API Provider flows are depicted in Figure A.1.4-1.

Figure A.1.4-1: EES onboarding flow

### A.1.5	Step 4: EES Publishes Its APIs

The EES publishes APIs for EASRegistration, EASDiscovery, and UE-related assistance (EEC), as depicted in Figure A.1.5-1.

Figure A.1.5-1: EES publishing flow

### A.1.6	Step 5: EES Discovers ECS APIs

Before interacting with ECS, the EES discovers ECS’s published APIs through CAPIF, as depicted in Figure A.1.6-1.

Figure A.1.6-1: EES discovery flow

### A.1.7	Step 6: EES Registers with ECS

In order to register in ECS, EES needs to consume the discovered EESRegistration API. For that, the EES establishes a Security Context with the CCF using the client certificate issued/provisioned during API Invoker onboarding (Step 3) selecting the SecurityMethod, in this case, will be OAuth. Then, the EES requests an OAuth2 access token (Client Credentials grant) from the CCF, bound to the established Security Context, with scope for ECS APIs (EESRegistration).

The EES invokes EESRegistration on the ECS (EDGE-6) with the OAuth token in the bearer header. The ECS validates the token and completes EES registration into ECS.

These steps are depicted in Figure A.1.7-1.

Figure A.1.7-1: EES registration flow

### A.1.8	Step 7: EAS Onboarding into CAPIF

Before onboarding, each EAS is provisioned with:

- CAPIF Root CA certificate(s): to validate the CCF’s TLS server certificate.

- CAPIF Core Function base URL(s): the root(s) for CAPIF onboarding, discovery/catalogue, and token endpoints.

- Bootstrap JWT and/or credential material: initial credentials issued by the operator/CCF that authorize the EAS to onboard; may encode roles for API Invoker and API Provider. JWT SHALL be different for API Invoker and API Provider roles.

The EAS establishes a TLS session to the CCF (verifying with the CAPIF Root CA). It authenticates using the bootstrap token/credentials and performs:

- API Invoker onboarding, so it can later discover and consume ECS and EES APIs

- API Provider registration, so it can later publish its own application APIs to CAPIF

The EAS onboarding as an API Invoker and then registering as an API Provider flows are depicted in Figure A.1.8-1.

Figure A.1.8-1: EAS registration flows

### A.1.9	Step 8: EAS Discovers ECS/EES APIs

EAS queries CAPIF to locate both ECS and its serving EES APIs, as depicted in Figure A.1.9-1.

Figure A.1.9-1: EAS discovery flows

### A.1.10	Step 9: EAS Registers with EES

The EAS requests a CAPIF-issued token scoped to EASRegistration and registers into its local EES, as depicted in Figure A.1.10-1.

Figure A.1.10-1: EAS registration flow

### A.1.11	Step 10: EAS Publishes Its Application APIs

The EAS publishes its application APIs (telemetry, AR, gaming, etc.) into CAPIF, as depicted in Figure A.1.11-1.

Figure A.1.11-1: EAS Publishing flow

At this point, EDGEAPP infrastructure is setup and ready. All EDGEAPP entitles are registered in CCF and have published their API endpoints. EES and EAS have also registered in ECS for UEs to discover them using EDEAPP APIs.

### A.1.12	Step 11: AC/EEC Access EAS Services

ACs on UEs reach EAS services via EEC and EES. The EEC may leverage CAPIF-discovered info or operator-controlled EES discovery.

The EEC discovers its serving EES either through operator bootstrap configuration (pre-provisioned URL/endpoint), dynamic discovery via DNS or network signalling, or network-assisted redirection at session setup. CAPIF is not usually queried directly by the EEC for EES.

EEC sends an API Discovery request to the CCF to learn what APIs EAS has published (this aligns with Step10 where the EAS published APIs into CAPIF).

The EEC then passes EAS API information to AC. AC invokes the desired EAS API. The EAS processes the request, and responds to the AC.

These steps are depicted in Figure A.1.12-1.

Figure A.1.12-1: AC/EEC API Access flow

* * * End of Changes * * * *