3GPP TSG-SA WG6 Meeting #69	S6-254170

Wuhan, China, 13th – 17th October 2025	(revision of S6-25xxxx)

Source:	China Mobile

Title:	New Solution of introducing renewable energy in AIMLE

Spec:	3GPP 23.700-44 v0.1.0

Agenda item:	9.9

Document for:	Approval

Contact:	Liutangqing (liutangqing@chinamobile.com)

lvhuazhang (lvhuazhang@chinamobile.com)

1. Introduction

This pCR proposes new solution for FS_EnergySys_Ph2_APP.

2. Reason for Change

This paper proposes to give the solution to solve the key issue #1 and key issue #4. This paper proposes a solution to involve renewable energy during AIML procedure.

3. Conclusions

None.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-44 v0.1.0.

* * * First Change * * * *

6.0	Mapping of solutions to key issues

Table 6.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI#5 | KI#6 |
| --- | --- | --- | --- | --- | --- | --- |
| Sol #x | x |  |  | x |  |  |
| Sol #2 |  |  |  |  |  |  |

* * * Second Change, all new text * * * *

6.x	Solution #x: AIMLE Enhancement of Considering Renewable Energy

6.x.1	Solution Description

The proposed solution solve the problem listed in KI#1 and KI#4, and proposes to consider renewable energy in AIMLE.

As indicated in TS 28.310[4], the renewable energy and renewable energy factor have the following definitions:

Renewable energy: energy from renewable non-fossil sources.

NOTE 15:	This definition is taken from 3GPP TS 22.261 [31].

NOTE 16:	Examples of renewable energy sources include wind, solar, aerothermal, geothermal, hydrothermal and ocean energy, hydropower, biomass, landfill gas, sewage treatment plant gas and biogases

NOTE 17:	A renewable energy source is an energy source not depleted by extraction as it is naturally replenished at a rate faster than it is extracted ISO/IEC 30134-3:2016 [33].

NOTE 18:	Criteria to categorize an energy as renewable can differ among jurisdictions, based on local environmental or other reasons ISO/IEC 30134-3:2016 [33].

Renewable energy factor: ratio of the renewable energy to the total energy.

NOTE 19:	This definition is taken from ISO/IEC 30134-3:2016 [33].

Energy can be classified into two major types: renewable energy and non-renewable energy. Renewable energy sources include solar energy, hydropower, wind energy, biomass energy, wave energy, tidal energy, ocean thermal energy, geothermal energy, etc. They can be recycled and regenerated in nature. It is an inexhaustible and endless energy source that regenerates automatically without human intervention.

In order to save the energy consumption and reduce the carbon emission, the operators may consider using renewable energy or green energy to replace traditional energy. For example, some servers of operators can be powered by electricity converted from renewable energy to reduce the carbon emission. Considering the renewable energy can be recycled and regenerated in nature, if the UE has renewable energy in energy consumption, from operator's policy, the operator can improve the QoS to a certain extent.

So, in the current AIMLE procedure, the following enhancements can be added:

For AIMLE related procedure:

-	The FL member capabilities can include whether renewable energy support or not. It is more preferred to select the FL members that support renewable energy to perform such as AI model training. This capability can be indicated during the registration procedure.

-	Also, the capability that support renewable energy can be set as the Member selection criteria. During the ML training procedure, the member that supports the renewable energy can have the priority selection

6.x.2	Architecture impacts

In this solution, it doesn’t change the overall architecture of Application enablement architecture of AIMLE in section 5 of TS 23.482[x]. But the following enhancements are added:

-	The FL member can register itself of the new capability that support renewable energy supply.

-	The subscriber can be notified with that new FL member is added to the ML repository with the energy supply of renewable energy.

-	And during member selection procedure, the Member selection criteria can be enhanced that whether renewable energy support or not.

6.x.3	Procedure

6.x.3.1	Procedure of FL member registration procedure

Figure 6.x.3.1-1: Registration on FL member registry with renewable energy

1.	The same procedure as step 1 in section 8.4.2 of TS 23.482[x] with the following modification that in step 1, the FL member registers the capability that this FL member supports the renewable energy as energy supply.

2-3.	The same procedure as step 2-3 in section 8.4.2 of TS 23.482[x].

6.x.3.2	Procedure of FL-related event subscription of renewable energy

Figure 6.x.3.3-2: Procedure for FL-related event subscription

1.	The same procedure as step 1 in section 8.5.3 of TS 23.482[x] with the following modification that in step 1 of the subscriber sends the subscription to subscribe whether the FL member supports the capability of renewable energy consumption, or the FL member starts to consume the renewable energy.

In step 1, also a threshold value can be provided. This value can be the real value of renewable energy consumption or the Renewable energy factor. If the real value of renewable energy consumption exceeds the threshold value or the Renewable energy factor exceeds the threshold value, the ML repository can send the notification to subscriber.

2-3.	The same procedure as step 2-3 in section 8.4.2 of TS 23.482[x] with the following changes that if the newly registered FL member that support the capability of renewable energy consumption or the renewable energy consumption reaching the threshold value of renewable energy consumption.

6.x.3.3	Procedure of member selection during ML model training considering renewable energy

Figure 6.x.3.3-3: ML model training considering energy consumption

1.	The same procedure as step 1 in section 8.3.2 of TS 23.482[x] with the following modification that in step 1 that the VAL server wants to select the FL member supports the capability of renewable energy consumption, or wants to select the FL member that the value of renewable energy consumption reaching the threshold value of renewable energy consumption.

In this step 1, the Member selection criteria are enhanced that to support either the member supports the capability of renewable energy consumption or the value of renewable energy consumption of the member reaching the threshold value of renewable energy consumption.

2-4.	The same procedure as step 2-3 in section 8.3.2 of TS 23.482[x] that the AIMLE server notifies the FL members that satisfied the Member selection criteria defined in step 1.

6.x.4	Solution evaluation

Editor’s note: This part is for further update in the future.

* * * Third Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[5]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[6]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[x]	3GPP TS 23.482: " Functional architecture and information flows for AIML Enablement Service".

* * * End of Changes * * * *