3GPP TSG-SA WG6 Meeting #69	S6-254186

Wuhan, P.R.China, 13th – 17th October 2025	(revision of S6-25xxxx)

Source:	China Mobile

Title:	Pseudo-CR on Update of KI #1 and Solution #1

Spec:	3GPP TR 23.700-51

Agenda item:	9.5

Document for:	Approval

Contact:	Jiadi Fu, fujiadi@chinamobile.com

1. Introduction

This pCR brings proposal for update of KI #1 and Solution #1.

2. Reason for Change

Update of KI #1 and Solution #1.

3. Conclusions

Update of KI #1 and Solution #1.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-51.

* * * First Change * * * *

4.1	Key issue #1: Transmission enhancement in application layer

According to 3GPP TS 23.433 [6], SA6 has defined services for XR applications in the application enablement layer, including the transmission for XR application, multi-modal flow alignment and synchronization, Tethering link measurement and provisioning, UE-to-UE communication, XR application server selection enhancement, Application QoS coordination for Mobile Metaverse Services, etc. However, in Rel-19 SA6 did not have sufficient time to study mechanisms supporting multiple XR VAL clients traffic of same XR service transmission on the same SEAL-UU connection in TS 23.433 [6]. Additionally, many policies are defined in TS 23.433 [6], such as Multi-modal flows alignment policy, Synchronization policy, UE-to-UE policy are defined, but the VAL server has no way to know whether and when these policies are enforced, therefore the work on policy enforcement notification to VAL server needs to be studied. SA2 defined Expedited Transfer Indication in TS 23.501 [3] clause 5.37.10. An AS based trigger of reflective QoS is used to expedite the transfer of larger payload for XR application. SA6 could investigate enhancement(s) to the SEALDD transmission mechanism to support QoS based on Expedited Transfer Indication.

This key issue will study:

-	How could application enabling layer be enhanced to support for policy (Multi-modal flows alignment policy, Synchronization policy, UE-to-UE policy, expedited transfer indication, policy enforcement notification, etc.).

-	How could application enabling layer be enhanced to support for XR related policy (e.g. multi-modal XR policy, multi-modal SEALDD policy) configuration, management, synchronization of multiple VAL clients.

* * * Next Change * * * *

5.2	Solution #1: Policy enforcement notification

This solution provides a solution to add policy enforcement notification in the SEALDD policy configuration.

5.2.1	Architecture Impacts

None

5.2.2	Solution description

TS 23.433 [6] defines numerous policies, including Quality Optimization, Bandwidth Control, UE-to-UE, and Synchronization policies. The enforcement of some policies is driven by network measurements, providing an indication of the network’s state. Consumers may therefore require information regarding policy enforcement to understand policy usage and gain insight into the network’s performance.

5.2.2.1	Impact to existing SEALDD procedures

The policy enforcement notification service is provided based on the enhancement of SEALDD service in 3GPP TS 23.433 [6], highlighted in bold italics, as follows.

9.10.2.X	Policy enforcement notification subscription

Figure 9.10.2.x-1 illustrates the procedure for policy enforcement notification subscription from the SEALDD server to the VAL server.

Figure 9.10.2.x-1: Policy enforcement notification subscription

1.	The VAL server sends the policy enforcement notification subscription request to the SEALDD server. The request includes the identifiers of the application traffic (e.g. VAL service ID, VAL server ID), VAL UE identify, and the Configuration ID indicating the requested policy. If the policy enforcement notification is needed, Notification requirement may be provided as well. The Notification requirement may include the notification method, specific policy requires notification, policy counting statistics requirement and notification target address (e.g. URL) may also be provided.

2.	The SEALDD server performs authorization check to verify whether the VAL server can be accepted/authorized to subscribe for policy enforcement notification.

3.	Upon successful authorization, the SEALDD server replies to the VAL server with the policy enforcement notification subscription response.

9.10.2.XX	Policy enforcement notification subscription update

Figure 9.10.2.xx-1 illustrates the procedure for policy enforcement notification subscription update from the SEALDD server to the VAL server.

Figure 9.10.2.xx-1: Policy enforcement notification subscription update

1.	The VAL server sends the policy enforcement notification subscription update request to the SEALDD server. The request includes the Subscription ID indicating the Policy enforcement notification subscription. The Updated Notification requirement is provided as well.

2.	The SEALDD server performs authorization check to verify whether the VAL server can be accepted/authorized to update the subscription for policy enforcement notification.

3.	Upon successful authorization, the SEALDD server replies to the VAL server with the policy enforcement notification subscription update response.

9.10.2.XY	Policy enforcement notification subscription delete

Figure 9.10.2.xy-1 illustrates the procedure for policy enforcement notification subscription delete from the SEALDD server to the VAL server.

Figure 9.10.2.xx-1: Policy enforcement notification subscription delete

1.	The VAL server sends the policy enforcement notification subscription delete request to the SEALDD server. The request includes the Subscription ID indicating the Policy enforcement notification subscription.

2.	The SEALDD server performs authorization check to verify whether the VAL server can be accepted/authorized to delete the subscription for policy enforcement notification.

3.	Upon successful authorization, the SEALDD server replies to the VAL server with the policy enforcement notification subscription delete response.

9.10.2.Y	Policy enforcement notification

Figure 9.10.2.y-1 illustrates the procedure for policy enforcement notification from the SEALDD server to the VAL server.

Figure 9.10.2.y-1: Policy enforcement notification

1.	The SEALDD server determine whether the enforced policy needs notification, based on Notification requirement.

2.	If notification is required, the SEALDD server sends the Policy enforcement notification, according to the notification method, to VAL server of notification target address.

9.10.3.x	Policy enforcement notification subscription request

Table 9.10.3.x-1 describes the information flow from the VAL server to the SEALDD server for requesting the policy enforcement notification subscription request.

Table 9.10.3.x-1: Policy enforcement notification subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Configuration ID | M | Identifier of the SEALDD policy configuration. |
| Application traffic identifiers | O | Identify of the application traffic (e.g. VAL server ID, VAL service ID). |
| Identities | O | Identifier of the VAL UEs or VAL users, or VAL UE group ID (as defined in clause 7.5 of 3GPP TS 23.434 [4]), or VAL UE identity list for which SEALDD policy applies |
| Slice identifier | O | The slice identifier (S-NSSAI) for which SEALDD policy applies. |
| Notification requirement | M | Indicating if the policy enforcement notification is needed. |
| > Notification method | O | Indicating the notification method (e.g., periodic notification along with periodicity, immediate notification, etc.) |
| >>Periodic Time | O | Interval Time of periodic notification. |
| > Policy name | O | Indicating which policy that requires notification, (e.g., the Multi-modal flows alignment policy, Quality optimization policy, etc.) |
| > Notification target address | O | Notification Target Address where the notification is to be sent |
| > Policy counting statistics requirement | O | Indicating whether the statistics that the requested policy was triggered is needed. |

9.10.3.y	Policy enforcement notification subscription response

Table 9.10.3.y-1 describes the information flow from the SEALDD server to the VAL server for sending the policy enforcement notification subscription response.

Table 9.10.3.y-1: Policy enforcement notification subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| > Subscription ID | O (See NOTE) | Identifier of the Policy enforcement notification subscription. |
| > Expiration time | O (See NOTE) | Indicates the expiration time of the configured SEALDD policy |
| NOTE: This IEs is used for the successful case for SEALDD policy configuration update request. |  |  |

9.10.3.xx	Policy enforcement notification subscription update request

Table 9.10.3.xx-1 describes the information flow from the VAL server to the SEALDD server for requesting the policy enforcement notification subscription update request.

Table 9.10.3.xx-1: Policy enforcement notification subscription update request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Identifier of the Policy enforcement notification subscription. |
| Updated Notification requirement | M | Indicating if the policy enforcement notification update is needed. |
| > Updated Notification method | O | Indicating the notification method (e.g., periodic notification along with periodicity, immediate notification, etc.) |
| >>Updated Periodic Time | O | Interval Time of periodic notification. |
| > Updated Policy name | O | Indicating which policy that requires notification, (e.g., the Multi-modal flows alignment policy, Quality optimization policy, etc.) |
| > Updated Notification target address | O | Notification Target Address where the notification is to be sent |
| > Updated Policy counting statistics requirement | O | Indicating whether the statistics that the requested policy was triggered is needed. |

9.10.3.y	Policy enforcement notification subscription update response

Table 9.10.3.y-1 describes the information flow from the SEALDD server to the VAL server for sending the policy enforcement notification subscription response.

Table 9.10.3.y-1: Policy enforcement notification subscription update response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| Subscription ID | O (See NOTE) | Identifier of the Policy enforcement notification subscription. |
| NOTE: This IEs is used for the successful case for SEALDD Policy enforcement notification subscription update request. |  |  |

9.10.3.xy	Policy enforcement notification subscription delete request

Table 9.10.3.xy-1 describes the information flow from the VAL server to the SEALDD server for requesting the policy enforcement notification subscription delete request.

Table 9.10.3.xy-1: Policy enforcement notification subscription delete request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Identifier of the Policy enforcement notification subscription. |

9.10.3.xy	Policy enforcement notification subscription delete response

Table 9.10.3.y-1 describes the information flow from the SEALDD server to the VAL server for sending the policy enforcement notification subscription delete response.

Table 9.10.3.y-1: Policy enforcement notification subscription delete response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| Subscription ID | O (See NOTE) | Identifier of the Policy enforcement notification subscription. |
| NOTE: This IEs is used for the successful case for SEALDD Policy enforcement notification subscription delete request. |  |  |

9.10.3.z	SEALDD client policy enforcement notification

Table 9.10.3.z-1 describes the information flow from the SEALDD server to the VAL server for SEALDD client policy enforcement notification.

Table 9.10.3.z-1: SEALDD client policy enforcement notification

| Information element | Status | Description |
| --- | --- | --- |
| Notification report | M | Report of this notification |
| > Subscription ID | O | Identifier of the Policy enforcement notification subscription. |
| > Policy enforcement time | O | (list of) Time the policy was enforced |
| > Policy counting statistics | O | Statistics such as the number of times the policy was executed. For example, when the notification is periodically reported, this IE represents the number of times a requested policy is executed within a time period. When the Notification method is immediate notification, this statistic can be one. |

* * * End of Changes * * * *