3GPP TSG-SA WG6 Meeting #69	S6-254221

Wuhan, P.R.China, 13th – 17th October 2025	revision of S6-25xxxx

Source:	Ericsson

Title:	New Solution for KI#1 on Functional Architecture

Spec:	3GPP TR 23.700-26 v0.1.0

Agenda item:	9.8

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

As described in Key Issue #1 (TR 23.700-26 clause 6.1), the study aspects on enhance Application enablement layer for Ambient IoT services include:

- Study whether and how to enhance the application enablement architecture /functional model or new application enabler to support the Ambient IoT services (i.e., AIoT inventory and command services).

There are some common requirements from different AIoT applications, for example application AIoT inventory, command, discovery, categorization and consolidation, monitoring presence of AIoT device, configure/manage and expose conversion of inventory result data.

Thus, this pCR proposes to enhance the application enablement layer architecture by introducing new enabler service to support AIoT application relevant services.

2. Reason for Change

This pCR proposes a new solution for Key Issue #1, to enhance the application enablement layer architecture by introducing new enabler service to support AIoT application relevant services.

3. Conclusions

This paper proposes solution on functional architecture to support AIoT application relevant services.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-26 v0.1.0.

* * * First Change * * * *

## 7.0	Mapping of solutions to key issues

Table 6.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |
| Sol #2 |  |  |  |  |
| Sol #X | X |  |  |  |

* * * Next Change * * * *

## 7.X	Functional Architecture to Support AIoT Services

### 7.X.1	Functional Architecture

#### 7.X.1.1	General

The functional architecture enhancement for the AIOTE (Ambient IOT Enablement) is based on the generic functional model specified in clause 6.2 of 3GPP TS 23.434 [xx]. It is organized into functional entities to describe an architecture enhancement which addresses the support for AIoT aspects for vertical applications.

#### 7.X.1.2	On-Network Functional Architecture

Figure 7.X.1.2-1: Generic on-network functional model

Figure 7.X.1.1.2-1 illustrates the generic on-network functional model with AIOTE as new SEAL service. In the vertical application layer, the VAL client communicates with the VAL server over VAL-UU reference point. VAL-UU supports both unicast and multicast delivery modes. The AIOTE consists of a common set of services (e.g. AIoT inventory, command, discovery, categorization and consolidation, monitoring presence of AIoT device, configure/manage and expose conversion of inventory result data) and reference points. The SAE offers its services to the vertical application layer (VAL).

### 7.X.2	Functional Entities Description

#### 7.X.2.1	General

The AIOTE functional entities with AIoT relevant service functions are described in the following subclauses.

#### 7.X.2.2	AIOTE server

The AIOTE server functional entity provides for sensing application relevant services supported within the vertical application layer.

#### 7.X.2.3	3rd party SAE server

The 3rd party AIOT server functional entity provides for AIoT relevant services supported for the vertical application layer.

### 7.X.3	Reference Points Description

#### 7.X.3.1	General

The reference points for the functional model for AIoT relevant service are described in the following subclauses.

#### 7.X.3.2	AIOTE-S

The interactions related to sensing application relevant services functions between the VAL server(s) and the AIOTE server are supported by AIOTE-S reference point.

#### 7.X.3.3	AIOTE-3P

The reference point AIOTE-3P is used for AIoT information provisioning from a third-party AIOTE server.

NOTE:      Further definition of AIOTE-3P is out of scope of this specification.

### 7.X.2	Architecture impacts

Editor’s Note:	The architecture impacts of the solution is FFS.

### 7.X.3	Solution evaluation

Editor’s Note:	The evaluation of the solution is FFS.

# *** End of Changes ***