3GPP TSG-SA WG6 Meeting #69	S6-254228

Wuhan, P.R.China, 13th – 17th October 2025	revision of S6-25xxxx

Source:	Ericsson

Title:	New Solution for KI#3 on EAS Discovery and Selection for Support Energy Saving

Spec:	3GPP TR 23.700-44 v0.1.0

Agenda item:	9.9

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

As described in Key Issue #3 (TR 23.700-44 clause 5.3), the study aspects on EDGE application enablement enhancement for energy efficiency include:

- Whether and how the EDGE application enablement layer needs to be enhanced to support energy saving and energy efficiency?

- How EDGE application enablement service could be enhanced to support energy saving and energy efficiency, including how to support the selection of an edge application server based on energy consumption information.

According to the use cases and comment functions identified in S6-254224, proper discovery and selection of edge application server can support energy saving. This pCR proposes procedure for EAS selection and enhance the EAS discovery services to support energy saving.

2. Reason for Change

This pCR proposes a new solution for Key Issue #3, to introduce EAS selection procedure and enhance the EAS discovery services to support energy saving.

3. Conclusions

This paper proposes solution on EAS selection procedure and enhancements of the EAS discovery, and EAS selection services to support energy saving.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-44 v0.1.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[5]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[6]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[aa]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

* * * Next Change * * * *

## 6.0	Mapping of solutions to key issues

Table 6.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI#5 | KI#6 |
| --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |  |  |
| Sol #2 |  |  |  |  |  |  |
| Sol #A |  |  | X |  |  |  |

* * * Next Change * * * *

## 6.A	Solution #A: Enhance Edge Services for Support Energy Saving

### 6.A.1	Solution Description

The services procedures and information flows introduced in 3GPP TS 23.558 [aa] for edge services, can be enhanced to support the energy saving requirements.

### 6.A.1.1	Procedure for EAS selection

Figure 6.A.1.1-1: EAS selection

- A consumer (e.g., VAL server, EEC) sends EAS discovery request to EES for discover a list of EAS. The request includes information as described in clause 6.A.1.2 the Table 8.5.3.2-1.

- After getting the EAS information in step 1, the consumer requests energy saving assistance from the ESE server.

3.	Based on the assistance information, the consumer performs EAS selection.

### 6.A.1.2	Enhance existing edge services to support EAS discovery and selection

The information flow for EAS discovery as specified in clause 8.5.3.2 of 3GPP TS 23.558 [aa] is enhanced as follows (new text in bold italics):

#### 8.5.3.2 EAS discovery request Table 8.5.3.2-1 describes information elements for the EAS discovery request. Table 8.5.3.2-2 provides further detail about the EAS Discovery Filter information element. Table 8.5.3.2-1: EAS discovery request Information element; Status; Description. Requestor identifier; M; The ID of the requestor (e.g. EECID). UE Identifier; O; The identifier of the UE (i.e., GPSI). Security credentials; M; Security credentials resulting from a successful authorization for the edge computing service.. EAS discovery filters; O; Set of characteristics to determine required EASs, as detailed in Table 8.5.3.2-2.. UE location; O; The location information of the UE. The UE location is described in clause 7.3.2.. List of UE IDs; O; List of UE IDs in an Application Group, applicable for S-EAS or S-EES triggered EAS discovery request.. Serving MNO information (NOTE 2); O; The serving MNO information (e.g. MNO name, PLMN ID) which is serving the subscriber.. Target DNAI (NOTE 1); O; Target DNAI information which can be associated with potential T-EAS(s). EEC Service Continuity Support; O; Indicates if the EEC supports service continuity or not. The IE also indicates which ACR scenarios are supported by the EEC or, if this message is sent by the EEC to discover a T-EAS, which ACR scenario(s) are intended to be used for the ACR.. EES Service Continuity Support (NOTE 1); O; The IE indicates if the S-EES supports service continuity or not. The IE also indicates which ACR scenarios are supported by the S-EES or, if the EAS discovery is used for an S-EES executed ACR according to clause 8.8.2.5, which ACR scenario is to be used for the ACR.. EAS Service Continuity Support (NOTE 1); O; The IE indicates if the S-EAS supports service continuity or not. The IE also indicates which ACR scenarios are supported by the S-EAS or, if the EAS discovery is used for an S-EAS decided ACR according to clause 8.8.2.4, which ACR scenario is to be used for the ACR.. EAS Instantiation Triggering Suppress; O; Indicates to the EES that EAS instantiation triggering should not be performed for the current request, and Instantiable EAS Information (e.g. instantiated, instantiable but not be instantiated yet) is to be provided in response.. EAS selection request indicator; O; Indicates the request for EAS selection support from the EES (e.g., for constrained device).. Indication of service continuity planning; O; Indicates that this EAS discovery request is triggered for service continuity planning.. Prediction expiration time; O; The estimated time the UE may reach the Predicted/Expected UE location or EAS service area at the latest. This IE is used by EES as analytics input to get edge load analytics information from ADAES service as described in clause 8.8 of TS 23.436 [28].. Tunnel information; O; It includes service provider ID, the endpoint address (e.g. IP address) of the tunnel server associated with application(s).. NOTE 1: This IE shall not be included when the request originates from the EEC. NOTE 2: This IE shall be included if edge node sharing is used.. Table 8.5.3.2-2: EAS discovery filters Information element; Status; Description. List of AC characteristics (NOTE 1); O; Describes the ACs for which a matching EAS is needed.. > AC profile (NOTE 2); M; AC profile containing parameters used to determine matching EAS. AC profiles are further described in Table 8.2.2-1.. > Application group profile (NOTE 6); O; Application group profile associated with the AC Profile, as defined in Table 8.2.11-1.. List of EAS characteristics (NOTE 1, NOTE 3); O; Describes the characteristic of required EASs.. > EASID; O; Identifier of the required EAS.. > Application Group ID; O; Application group identifier as defined in 7.2.11.. > EAS content synchronization support; O; Indicates if the EAS content synchronization support is required or not.. > Bundle ID (NOTE 5); O; A list of EASIDs or a bundle ID as described in clause 7.2.10.. > List of EASIDs (NOTE 5); O; A list of EASIDs specific to a particular EAS bundle.. > Bundle type (NOTE 4); O; Type of the EAS bundle as described in clause 7.2.10. > EAS bundle requirements (NOTE 4); O; Requirements associated with the EAS bundle as described in clause 8.2.10.. > EAS provider identifier; O; Identifier of the required EAS provider. > EAS type; O; The category or type of required EAS (e.g. V2X, UAV, application enabler). > EAS schedule; O; Required availability schedule of the EAS (e.g. time windows). > EAS Geographical Service Area (NOTE 6); O; Location(s) (e.g. geographical area, route) where the EAS service should be available.. > EAS Topological Service Area (NOTE 6); O; Topological area (e.g. cell ID, TAI) for which the EAS service should be available. See possible formats in Table 8.2.7-1.. > Service continuity support; O; Indicates if the service continuity support is required or not.. > Service permission level; O; Required level of service permissions e.g. trial, gold-class. > Service feature(s); O; Required service features e.g. single vs. multi-player gaming service. > Energy mode support; O; Indicates what energy mode support is required, e.g. Economy mode, Performance mode, Balanced mode.. > Energy type support; O; Indicates what energy type support is required, e.g. non-renewable energy, renewable energy.. > Energy status share support; O; Indicates if energy status sharing is required or not. And the required granularities of energy status sharing, time period, service, application.. NOTE 1: Either "List of AC characteristics" or "List of EAS characteristics" shall be present. NOTE 2: "Preferred ECSP list" IE shall not be present. NOTE 3: The "List of EAS characteristics" IE must include at least one optional IE, if used as an EAS discovery filter. NOTE 4: When EAS discovery request is sent by the EEC, this IE shall not be included. NOTE 5: “Bundle ID" and "List of EASIDs" shall not both be present. NOTE 6: If application group profile IE is present, the expected group geographic service area IE present in the application group profile is preferentially used. If there is no EAS satisfied the expected group geographic service area or EAS does not provide better E2E response time, then the UE location and other service area IEs are considered..

The information flow of EAS Profile for EAS registration and discovery as specified in clause 8.2.4 of 3GPP TS 23.558 [aa] is enhanced as follows (new text in bold italics):

### 8.2.4 EAS Profile An EAS Profile includes information about an EAS used to describe services and service characteristics offered. NOTE: Information elements in the EAS Profile are provided by the ASP. Table 8.2.4-1: EAS Profile Information element; Status; Description. EASID; M; The identifier of the EAS. EAS Endpoint; M; Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. This information maybe discovered by EEC and exposed to ACs so that ACs can establish contact with the EAS.. List of EAS bundle information; O; List of EAS bundles to which the EAS belongs and related bundling requirements.. > Bundle ID (NOTE 3); O; Bundle ID as described in clause 7.2.10.. > List of EAS IDs (NOTE 2, NOTE 3); O; List of the EAS IDs of the EASs to be invoked by the EAS for an EAS driven association of EASs.. > Bundle type; M; Type of the EAS bundle as described in clause 7.2.10. > Main EASID; O; Indicate which EAS in a bundle takes the main EAS service role.. > EAS bundle requirements; O; Requirements associated with the EAS bundle as described in clause 8.2.10.. ACID(s); O; Identifies the AC(s) that can be served by the EAS. EAS Provider Identifier; O; The identifier of the ASP that provides the EAS.. Allowed MNO information (NOTE 4); O; Information of the allowed operator (e.g. MNO name, PLMN ID) from which its subscriber can consume the EAS services. EAS Type; O; The category or type of EAS (e.g. V2X, UAV, application enabler). EAS description; O; Human-readable description of the EAS. EAS Schedule; O; The availability schedule of the EAS (e.g. time windows). EAS Geographical Service Area; O; The geographical service area that the EAS serves. ACs in UEs that are located outside that area shall not be served.. EAS Topological Service Area; O; The EAS serves UEs that are connected to the Core Network from one of the cells included in this service area. ACs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1.. EAS Service KPIs; O; Service characteristics provided by EAS, detailed in Table 8.2.5-1. EAS service permission level; O; Level of service permissions e.g. trial, gold-class supported by the EAS. EAS Feature(s); O; Service features e.g. single vs. multi-player gaming service supported by the EAS. EAS content synchronization support; O; Indicates if the EAS supports content synchronization between EASs.. EAS Service continuity support; O; Indicates if the EAS supports service continuity or not. This IE indicates which ACR scenarios are supported by the EAS, also indicates the EAS ability (e.g. EAS bundle information) of handling bundled EAS coordinate ACR.. EAS Transport layer service continuity support; O; This IE indicates the EAS service continuity support for seamless transport layer (e.g. TCP/TLS/QUIC) relocation. General context holding time duration (NOTE 1); O; The time duration that the EAS holds the context before the AC connects to the EAS in case of ACR for service continuity planning. It is an indication of the time the EAS holds the application context for a UE to move to its service area after receiving an ACR notification from the EES following an ACR request from the EEC.. List of EAS DNAI(s); O; DNAI(s) associated with the EAS. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. It is a subset of the DNAI(s) associated with the EDN where the EAS resides.. List of N6 Traffic Routing requirements; O; The N6 traffic routing information and/or routing profile ID corresponding to each EAS DNAI.. EAS Availability Reporting Period; O; The availability reporting period (i.e. heartbeat period) that indicates to the EES how often it needs to check the EAS's availability after a successful registration.. EAS Status; O; The status of the EAS (e.g. enabled, disabled, overload warning etc.). List of associated devices; O; List of associated devices (e.g. haptic device, joy stick) required along with UE in order to provide service to the user. It includes device type.. Energy mode support; O; Indicates what energy mode is supported by the EAS, e.g. Economy mode, Performance mode, Balanced mode.. Energy type support; O; Indicates what energy type is supported by the EAS, e.g. non-renewable energy, renewable energy.. Energy status share support; O; Indicates if the EAS supports energy status sharing or not. The energy status may be shared with different granularities based on request, e.g. time period, service, application.. NOTE 1: Since the EASID of the EAS identifies the type of the application (e.g. SA6Video, SA6Game etc) as described in clause 7.2.4, "General context holding time duration" determined by EAS can depend on the EASID (type of the application). NOTE 2: This IE may be provided when only bundle ID is provided, and the bundle type indicates the proxy bundle. NOTE 3: At least one of the IEs shall be present if EAS bundle information is provided. NOTE 4: For edge node sharing scenario, in order to restrict the access to the subscriber of the partner operator, this IE should only include MNO information of the leading operator.. NOTE: The EAS Transport layer service continuity support can be used in EAS discovery, e.g. as described in 3GPP TS 23.433 [26] for SEALDD server acting as EAS, which can further support the EAS IP replacement function.

### 6.A.2	Architecture impacts

Editor’s Note:	The architecture impacts of the solution is FFS.

### 6.A.3	Solution evaluation

Editor’s Note:	The evaluation of the solution is FFS.

# *** End of Changes ***