3GPP TSG-SA WG6 Meeting #69	S6-254236

Wuhan, P.R.China, 13th – 17th October 2025	revision of S6-25xxxx

Source:	Ericsson

Title:	New Solution for KI#4 on Use Sensing Results to Enhance HD Map for V2X Services

Spec:	3GPP TR 23.700-15 v0.1.0

Agenda item:	9.13

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

As described in Key Issue #4 (TR 23.700-15 clause 4.4), the study aspects on utilizing sensing results for HD map in V2X include:

- How to enhance VAE layer to provide enhanced HD map including traditional vehicles information with considering sensing results.

- How to enhance VAE layer to provide enhanced HD map including environment information (e.g. pedestrian/animal objects, etc) with considering sensing results.

Sensing plays an important role in smart transportation. Most modern cars are equipped with onboard sensors – radar, lidar, and cameras just to mention a few – which help the car to get an overview of its immediate surroundings. However, onboard systems fail to provide information further away and cannot “see around the corner”. An sensing system can complement onboard sensory data with more “global” data that cannot be obtained by sensors mounted on the vehicle. Another automotive-related example is the detection of pedestrians or animals on highways.

Application enablement layer can aggregate the sensing demands of a large number of vehicles, enabling unified subscription to sensing data from different sensing entities. By tracking or predicting vehicle locations in real time, this layer transforms the geographically scoped sensing data into a vehicle-centric dynamic view, and simplifies the vehicle's data consumption process and significantly enhances processing efficiency and system responsiveness.

Thus, this pCR proposes to use sensing results from Sensing Application Enablement service (SAE, which is introduced in S6-254233) to enhance HD map for supporting V2X services.

2. Reason for Change

To address the study aspects in Key Issue #4, this pCR proposes a new solution on use sensing results to enhance HD map for supporting V2X services.

3. Conclusions

This paper proposes solution on use sensing results to enhance HD map for supporting V2X services.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-15 v0.1.0.

* * * First Change * * * *

## 6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |
| Sol #2 |  |  |  |  |
| Sol #X3 |  |  |  | X |

* * * Next Change * * * *

## 6.X3	Solution #X3: Use Sensing Results to Enhance HD Map for V2X Services

### 6.X3.1	Solution Description

Sensing plays an important role in smart transportation. Most modern cars are equipped with onboard sensors – radar, lidar, and cameras just to mention a few – which help the car to get an overview of its immediate surroundings. However, onboard systems fail to provide information further away and cannot “see around the corner”. An sensing system can complement onboard sensory data with more “global” data that cannot be obtained by sensors mounted on the vehicle. Another automotive-related example is the detection of pedestrians or animals on highways.

Application enablement layer can aggregate the sensing demands of a large number of vehicles, enabling unified subscription to sensing data from different sensing entities. By tracking or predicting vehicle locations in real time, this layer transforms the geographically scoped sensing data into a vehicle-centric dynamic view, and simplifies the vehicle's data consumption process and significantly enhances processing efficiency and system responsiveness.

The following clauses specify use sensing results to enhance HD map for supporting V2X services.

#### 6.X2.1.1	Procedure

Figure 6.X2.1.1-1: Sensing result subscription for enhance HD map

- The Consumer (e.g., VAL server/client, VAE server/client) sends sensing result subscription request to the SAE Server for device-centric sensing result. The request contains the context information, e.g. planned route of the vehicle.

- The SAE Server authenticates the request, checks the stored sensing data, and determines the next actions (e.g., to collect sensing data from sensing entities, to collect device location information/analytics, to generate device-centric sensing result).

- The SAE Server responds to the VAL server for the sensing result subscription request.

- The SAE Server interacts with the SAE Client (deployed on a non-3GPP sensor) for non-3GPP sensing data.

4a.	The SAE Server sends sensing result subscription request to the SAE Client to collect the non-3GPP sensing data.

4b.	The SAE Client responds to the SAE Server for the subscription request.

4c.	The SAE Client sends sensing result subscription notifications to the SAE Server with the non-3GPP sensing data.

- The SAE Server interacts with the 5GC NF (e.g. NEF) for receiving notifications with the sensing data from the 3GPP network.

- The SAE Server interacts with the 3rd party SAE server for receiving the sensing data from the 3rd party sensing data.

- After receiving the sensing data from the sensing entities in steps 4-6, the SAE Server interacts with the other SEAL servers for collection of location information/analytics (e.g., LMS for device location information, ADAES for device location analytics).

- The SAE Server filters the sensing data and location information/analytics, to generate device-centric sensing results.

- The SAE Server sends sensing result subscription notifications the Consumer with the generated device-centric sensing results.

#### 6.X3.1.2	Information Flows

Editor’s Note:	The information flows of the solution is FFS.

### 6.X3.2	Architecture impacts

Editor’s Note:	The architecture impacts of the solution is FFS.

### 6.X3.3	Solution evaluation

Editor’s Note:	The evaluation of the solution is FFS.

# *** End of Changes ***