3GPP TSG-SA WG6 Meeting #69	S6-254239

Wuhan, China, 13th – 17th October 2025	(revision of S6-254xxx)

Source:	Huawei, Hisilicon

Title:	Solution on managing concurrent AIoT device monitoring operations

Spec:	3GPP TR 23.700-26 v0.1.0

Agenda item:	9.8

Document for:	Approval

Contact:	Cuili Ge – gecuili@huawei.com

1. Introduction

This solution aims to address the issues identified in Key Issue 3 to manage concurrent AIoT device monitoring operations.

2. Reason for Change

In practical application scenarios, the monitoring operations for AIoT devices are not singular; there may be multiple monitoring operations established under various existence criteria. Therefore, it is necessary to schedule these operations reasonably. Otherwise, conflicting operations could lead to numerous network issues, such as system overload and response delays. Hence, it is essential to ensure network stability and business reliability through efficient concurrency control mechanisms.

3. Conclusions

To address the risk of network overload caused by concurrent monitoring operations of a massive number of AIoT devices, it is essential for the application enablement layer to establish a concurrency control mechanism. This is a core prerequisite for ensuring network scalability and reliability.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-26 v0.1.0.

* * * First Change * * * *

7.0	Mapping of solutions to key issues

Table 6.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #x |  |  | Y |  |
| Sol #2 |  |  |  |  |

* * * Next Change * * * *

7.x	Solution #x: managing concurrent AIoT device monitoring operations

7.x.1	Solution description

This solution address key issue#3 to provide methods to manage concurrent AIot monitoring operations.

When concurrent monitoring operations need to be performed on multiple AIoT devices, repeatedly issuing the same commands can significantly reduce system scheduling efficiency. Since base stations typically only support serial operations, meaning they can handle only one task at a time, the application enablement layer must coordinate and aggregate concurrent requests from multiple applications. For instance, if multiple operations target different AIoT devices within Area A, they can be consolidated into a single command and uniformly dispatched to the 5GC, thereby avoiding redundant scheduling and enhancing processing efficiency.

Figure 7.x-1 represents a schematic diagram of concurrent AIoT monitoring operations.

Figure 7.1-1 Concurrent AIoT monitoring operations

Additionally, the application enablement layer can intelligently filter based on device identifiers, for instance, selecting devices that match specific code segments. Drawing from the masking mechanism of RFID, these devices can be abstracted into a logical group and described through a unified masking rule. This involves filtering target devices according to identifier rules (such as those with similar ID segments) and then configuring them into a mask group. Subsequently, the application enablement layer only needs to issue a single monitoring command to the 5GC and specify this mask to achieve batch operations on all devices within the group.

Figure 7.x-2 represents a schematic diagram of concurrent AIoT monitoring operations with mask mechanism.

Figure 7.x-2 Concurrent AIoT monitoring operations with mask mechanism

7.x.2	Architecture Impacts

This solution does not involve any impact on the architecture.

7.x.3	Corresponding APIs

No service API are introduced.

7.x.4	Solution evaluation